3GPP TSG-RAN WG4 Meeting #116	                          R4-2509687

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on RRM performance requirements for low band carrier aggregation via switching

Source: 	OPPO

Agenda item:	7.3.5

Document for:	Approval

Introduction

Based on the timeline of low band CA work item, the performance requirements for low band carrier aggregation via switching need to be discussed from this meeting. In this contribution, we will provide our considerations on the test cases based on the requirements defined in core part.

Discussion

Based on the conclusions of core part discussion and the CR split, the following requirements are defined for low band CA via switching:

Agreement in AH: Table 1. Necessity of requirement on SDL CC Requirement; Whether or not RAN4 will have requirement; Note. interruption requirement for CC switching; No (can be revisited if not aligned with RAN1 further agreement.); Issue 1-2-1, Agreed on Tuesday online session. Activated SDL SCell L3 measurement; Yes; Issue 1-3-1, Agreed on Tuesday online session. Deactivated SDL SCell measurement [and corresponding interruption]; Yes; Issue 1-3-3, Agreed on Tuesday online session. SDL SCell activation/deactivation and corresponding interruption; Yes. [BFD/CBD]; Yes; Related with issue 1-3-10. TCI state switch delay; Yes; Related with issue 1-3-11. PL-RS switch delay; No; Related with issue 1-3-12. L1-RSRP/SINR; Yes; Related with issue 1-3-13.

To verify the requirements, the corresponding test cases need to be defined.

Proposal 1: Define the following test cases for LB CA via switching:

- Test case for PCell/activated SDL SCell/intra-frequency L3 measurement period

- Test case for deactivated SDL SCell measurement and corresponding interruption

- Test case for SDL SCell activation/deactivation delay and corresponding interruption

- Test case for RLM/BFD/CBD

- Test case for TCI state switch delay

- Test case for L1-RSRP/SINR

According to the requirements design, except interruption, the measurement requirements is based on the legacy requirements, and with the restriction that only the SMTC occasions overlapped with serving cell (PCell or SDL SCell) reception duration indicated by switching pattern are considered. So for each test case, the corresponding legacy test configurations in FR1 can be used as baseline and the switching pattern is configured.

To verify the different overlapping ratio between SMTC occasions and Cell reception duration, different switching patterns can be configured in different test cases.

Proposal 2: For each test case, the corresponding legacy test configurations can be used as baseline.

Proposal 3: Different switching patterns can be configured in different test cases to cover fully overlapping and partially overlapping cases between SMTC occasions and corresponding cell reception duration.

Conclusions

In this paper, we provided our views on the performance requirements for low band carrier aggregation via switching. And the following proposals are provided.

Proposal 1: Define the following test cases for LB CA via switching:

- Test case for PCell/activated SDL SCell/intra-frequency L3 measurement period

- Test case for deactivated SDL SCell measurement and corresponding interruption

- Test case for SDL SCell activation/deactivation delay and corresponding interruption

- Test case for RLM/BFD/CBD

- Test case for TCI state switch delay

- Test case for L1-RSRP/SINR

Proposal 2: For each test case, the corresponding legacy test configurations can be used as baseline.

Proposal 3: Different switching patterns can be configured in different test cases to cover fully overlapping and partially overlapping cases between SMTC occasions and corresponding cell reception duration.

Reference

- RP-243317 New WID on low band carrier aggregation via switching

- R4-2508259 WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2505556 Topic summary for [115][207] NR_LBCA_Sw, Apple