# 3GPP TSG-RAN WG4 Meeting #116	R4-2509695

# Bengaluru, India, August 25th – 29th, 2025

Title: 	On RRM requirements of SSB adaptation

Source: 	OPPO

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved for SSB adaptation [1]. In this contribution, we will provide discussion on RRM requirements for SSB adaptation.

Discussion

### Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell Agreement RAN4 to define L3 requirements at transition for SSB adaptation same as legacy L3 transition period requirement. - The cell identification and measurement period requirements apply based on the longer period before or after the transition. Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell Agreement Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity.

For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. For serving cell, Scell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity.

  - RAN4 needs to clarify this in our spec, e.g., whether to differentiate the requirements for Scell and intra-frequency neighbor cell in different tables.

  - Observation 1: The adapted SSB on neighbor cell is measured based on legacy SMTC, while for serving cell it is based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity.

Issue 1-3-4: Colliding with MG - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

  - The collision rule between SMTC and MG should be kept unchanged. We support option 1.

  - Proposal 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements - Proposals - Option 1: R18 Scell activation with L3 reporting - Option 2: multiple SCells activation scenario with SSB adaptation

About the applicable SCell activation requirements, RAN4 agreed to define requirements for SSB adaption, including normal SCell activation, direct SCell activation and PUCCH SCell activation.

We prefer not to include Rel-18 SCell activation with L3 reporting because the scenario in R18 is for unknown deactivated SCell, and the requirements apply for UE supporting l3-MeasUnknownSCellActivation-r18 and reporting valid L3 measurement results after receiving the SCell activation command for unknown SCell. To be highlighted, the SSBs measured for L3 report could be different from the adapted SSBs because SSB adaptation command comes no later than SCell  activation command. In addition, SCell activation requirements are already supported based on the SSB periodicity with another UE capability shortMeasInterval-r18. SSB adaptation does bring additional benefit for UE supporting R18 requirements.

Proposal 2: RAN4 not to include the case of R18 Scell activation with L3 reporting in this WI.

Conclusions

In this paper we provided our views on the SMTC impact on RRM requirements due to SSB adaptation and/or OD-SSB.

  - Observation 1: The adapted SSB on neighbor cell is measured based on legacy SMTC, while for serving cell it is based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity.

  - Proposal 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

Proposal 2: RAN4 not to include the case of R18 Scell activation with L3 reporting in this WI..

Reference

- R4-2508278, WF on RRM requirements for Netw_Energy_NR_enh_Part2