3GPP TSG-RAN WG4 Meeting #116				                                           R4-2509796

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.23.3.2

Title: 	Discussion on RRM requirements for adaptation of common signal/channel in NES

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NES was agreed and one of the objectives below was discussed in the last meeting [4].

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this meeting, we provide our further considers on the open issues [R4-2508441,WF].

- Discussion

  - Applicable scenarios

In the last RAN4 meeting, the following agreements/WF were achieved.

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements - Proposals - Option 1: R18 Scell activation with L3 reporting - Option 2: multiple SCells activation scenario with SSB adaptation

For R18 SCell activation with L3 reporting, the impacts due the SSB adaptation is the SSB periodicity updates which are highlighted in the spec below. In our view the necessary specification updates can be limited. Thus, we can propose that:

Proposal 1: R18 Scell activation with L3 reporting can be defined if WI competition timeline allowed.

8.3.17 SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation …. Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in milliseconds. - Tactivation_time is: - 10 ms + TL3,report+ THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 7 ms + TL3,report+ max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, If the following conditions are met: If the SCell being activated belongs to FR1 and if there is no active serving cell contiguous to the SCell on that FR1 band provided that the side condition Ês/Iot ≥ -2 dB is fulfilled: If the target SCell belongs to FR1 and none of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation; If the SCell being activated belongs to FR2-1 and if there is no active serving cell on that FR2-1 band provided that PCell or PSCell is in FR1 or in FR2-1: If the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA or if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management, and the target SCell is unknown to UE provided that the side condition Ês/Iot ≥ -2 dB is fulfilled. Otherwise, Tactivation_time in clause 8.3.2 is applied for unknown SCell activation. However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - a single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260 ns with respect to the to-be-activated SCell’s SSB numerology, or its reception power difference with contiguous FR1 active serving cell is larger than 6 dB; where, TL3, report is delay of acquiring the first available UL resource for L3 reporting from 7 ms +THARQ after receiving SCell activation command. - The L3 measurement reporting requirement is defined in clause 9.2.4.4 - UE is ready to report the L3 measurement result no later than 7 ms + THARQ ms from receiving the SCell activation command. UE is not required to report the L3 results after 3 ms + THARQ+ M ms from receiving the SCell activation command where - For FR1, - M= 2*TSSB + TL1-RSRP,report for UE supporting shortMeasInterval-r18 capability, - Otherwise, M = TSMTC +TSSB + TL1-RSRP,report - For FR2-1, - M= (X1+X2)*TSSB + TL1-RSRP,report for UE supporting reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and shortMeasInterval-r18 capability, - M = X1*TSMTC +X2*TSSB + TL1-RSRP,report for UE supporting reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas without supporting shortMeasInterval-r18 capability, - M = 16*TSSB + TL1-RSRP,report for UE supporting shortMeasInterval-r18 without supporting reduceForCellDetection and reduceForSSB-L1-RSRP-Meas capability, - Otherwise, M = 8*TSMTC +8*TSSB + TL1-RSRP,report Where, X1 and X2 are UE capabilities as reported in reduceForCellDetection and reduceForSSB-L1-RSRP-Meas respectively. TSSB is the same as Trs, enhanced as specified in clause 8.3.2. TSMTC is the same as TSMTC_MAX as specified in clause 8.3.2.

For multiple SCells activation scenario, there are similar discussion for OD-SSB and the following agreements was achieved.

Issue 1-4-1: OD-SSB based multiple SCells activation Agreement - RAN4 to define the requirements in the case of multiple OD-SSB SCells. - It doesn’t impact the WI’s completion. - The scope can be limited as follows: - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping. - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells. - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

And in our understanding, the multiple SCells scenarios for OD-SSB is limited to the non-overlapping one which is more like the single activation case from the requirements perspective. Thus we can consider the similar approach as OD-SSB requirements for SSB adaptation consistently.

Proposal 2: Deprioritize the multiple SCells activation scenario with SSB adaptation

  - SCell activation requirement for SSB adaptation

In the previous meeting, for the SCell activation requirement for SSB adaptation some proposals on how to define the SCell activation requirement (e.g. the start point of SCell activation command) was raised but without any requirements:

Issue 1-3-4: Colliding with MG - Proposals - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

In NR, the measurement within gap will be prioritized if colliding with the measurement on the serving cell. Thus, we thought the same principle shall be followed.

Proposal 3: If the SMTC occasion with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Proposal 1: R18 Scell activation with L3 reporting can be defined if WI competition timeline allowed.

Proposal 2: Deprioritize the multiple SCells activation scenario with SSB adaptation

Proposal 3: If the SMTC occasion with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

- References

- RP-240830

- TS38.133 v18.6.0

- TS38.331 v17.1.0

- R4-2502689

- R2-2501484