3GPP TSG-RAN WG4 Meeting #116	R4-2509854

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.6.3.2.1

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN HPUE TX RF requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of NR-NTN UEs for FR1 single carrier scenario.

In this contribution, our views about PC2 TX RF requirements for NR-NTN HPUE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

2.2		Previous Meeting Agreements

In RAN4#115 meeting, the agreed WF [2] provides the following works for specifying TX requirements for NR-NTN under HPUE operation. The partial contents are shown as follows.

Issue 1-2: A-MPR simulation Agreement: - For PC2, to adopt this scheme: - In the simulation assumption, the maximum UL allocation size of [25, 100] RBs is considered. - Other number of RB is not precluded Issue 1-3: AMPR assumption for n255 Agreement: - Agree to use 10dB as filter rejection assumption for GNSS frequencies 1559-1610 MHz for PC2/PC1.5/PC1 AMPR analysis for n255. Issue 1-4: PC2 AMPR Way forward: - To harmonize the AMPR regions in offline activates before next meeting. - Target in next meeting: Come up with numbers in square brackets, and confirmed by other companies. - Offline efforts before next meeting is expected.

2.3		PC2 HPUE TX RF requirements

Proposal 1: Regarding band n256 PC2 AMPR for NS_24, consider whether the PC3 regions (i.e., TS 38.101-1 Table 6.2.3.15-1) could be reused for PC2 regions.

Proposal 2: Regarding band n256 PC2(1TX) NS_24 AMPR, consider AMPR values in table below.

Table: A-MPR for modulation and waveform type for power class 2

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer | Outer/ Inner | Outer/ Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 12.5 | ≤ 7 | ≤ 4.5 | ≤ 8.5 | ≤ 19.6 | ≤ 11.7 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 12.5 | ≤ 7 | ≤ 4.5 | ≤ 8.5 | ≤ 19.6 | ≤ 11.7 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 13.3 | ≤ 7.1 | ≤ 5.2 | ≤ 8.5 | ≤ 21 | ≤ 13.0 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 13.3 | ≤ 7.1 | ≤ 5.2 | ≤ 8.5 | ≤ 21 | ≤ 13.5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 13.3 | ≤ 7.1 |  | ≤ 8.5 | ≤ 21 | ≤ 13.5 |  |
| CP-OFDM QPSK | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.1 | ≤ 15.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.6 | ≤ 15.9 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.6 | ≤ 15.9 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13.9 | ≤ 7.3 |  | ≤ 8.5 | ≤ 22.6 | ≤ 16.1 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) NOTE 2: Outer and inner allocations are defined in clause X.X.X |  |  |  |  |  |  |  |

Observation 1: Regarding band n252 PC2(2TX) NS_07N/NS_08N AMPR, the quick AMPR measurement results as shown below are performed using normal conditions (e.g., normal voltage, room temperature, ACLR is not degraded to 31dBc) for information only.

| Band | TX Freq (MHz) | TX CBW (MHz) | Pattern | Meas. Freq (dBm/MHz) | Target SEM (dBm/MHz) | PC2 Power Backoff (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n252 | 2002.5 | 5 | DFT-s QPSK w/ FRB | 1995 | -30 | 0.8 |
| n252 | 2002.5 | 5 | DFT-s QPSK w/ FRB | 1995 | -40 | 6.3 |
| n252 | 2005 | 10 | DFT-s QPSK w/ FRB | 1990 | -30 | 0 |
| n252 | 2005 | 10 | DFT-s QPSK w/ FRB | 1990 | -40 | 5.4 |
| Band | TX Freq (MHz) | TX CBW (MHz) | Pattern | Meas. Freq (dBm/MHz) | Target SEM (dBm/MHz) | PC2 Power Backoff (dB) |
| n252 | 2002.5 | 5 | CP QPSK w/ FRB | 1995 | -30 | 3.7 |
| n252 | 2002.5 | 5 | CP QPSK w/ FRB | 1995 | -40 | 8.3 |
| n252 | 2005 | 10 | CP QPSK w/ FRB | 1990 | -30 | 1.6 |
| n252 | 2005 | 10 | CP QPSK w/ FRB | 1990 | -40 | 7.1 |

3 Conclusion

In this paper, our views on PC2 TX RF requirements for NR-NTN HPUE are provided as follows:

Proposal 1: Regarding band n256 PC2 AMPR for NS_24, consider whether the PC3 regions (i.e., TS 38.101-1 Table 6.2.3.15-1) could be reused for PC2 regions.

Proposal 2: Regarding band n256 PC2(1TX) NS_24 AMPR, consider AMPR values in table below.

Table: A-MPR for modulation and waveform type for power class 2

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer | Outer/ Inner | Outer/ Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 12.5 | ≤ 7 | ≤ 4.5 | ≤ 8.5 | ≤ 19.6 | ≤ 11.7 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 12.5 | ≤ 7 | ≤ 4.5 | ≤ 8.5 | ≤ 19.6 | ≤ 11.7 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 13.3 | ≤ 7.1 | ≤ 5.2 | ≤ 8.5 | ≤ 21 | ≤ 13.0 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 13.3 | ≤ 7.1 | ≤ 5.2 | ≤ 8.5 | ≤ 21 | ≤ 13.5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 13.3 | ≤ 7.1 |  | ≤ 8.5 | ≤ 21 | ≤ 13.5 |  |
| CP-OFDM QPSK | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.1 | ≤ 15.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.6 | ≤ 15.9 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 13.9 | ≤ 7.3 | ≤ 5.6 | ≤ 8.5 | ≤ 22.6 | ≤ 15.9 | ≤ 5.5 |
| CP-OFDM 256 QAM | ≤ 13.9 | ≤ 7.3 |  | ≤ 8.5 | ≤ 22.6 | ≤ 16.1 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) NOTE 2: Outer and inner allocations are defined in clause X.X.X |  |  |  |  |  |  |  |

Observation 1: Regarding band n252 PC2(2TX) NS_07N/NS_08N AMPR, the quick AMPR measurement results as shown below are performed using normal conditions (e.g., normal voltage, room temperature, ACLR is not degraded to 31dBc) for information only.

| Band | TX Freq (MHz) | TX CBW (MHz) | Pattern | Meas. Freq (dBm/MHz) | Target SEM (dBm/MHz) | PC2 Power Backoff (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n252 | 2002.5 | 5 | DFT-s QPSK w/ FRB | 1995 | -30 | 0.8 |
| n252 | 2002.5 | 5 | DFT-s QPSK w/ FRB | 1995 | -40 | 6.3 |
| n252 | 2005 | 10 | DFT-s QPSK w/ FRB | 1990 | -30 | 0 |
| n252 | 2005 | 10 | DFT-s QPSK w/ FRB | 1990 | -40 | 5.4 |
| Band | TX Freq (MHz) | TX CBW (MHz) | Pattern | Meas. Freq (dBm/MHz) | Target SEM (dBm/MHz) | PC2 Power Backoff (dB) |
| n252 | 2002.5 | 5 | CP QPSK w/ FRB | 1995 | -30 | 3.7 |
| n252 | 2002.5 | 5 | CP QPSK w/ FRB | 1995 | -40 | 8.3 |
| n252 | 2005 | 10 | CP QPSK w/ FRB | 1990 | -30 | 1.6 |
| n252 | 2005 | 10 | CP QPSK w/ FRB | 1990 | -40 | 7.1 |

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2508008, “WF on [115][128] NR_IoT_NTN_HPUE_part1”, Samsung