3GPP TSG-RAN4 Meeting #116	R4-2509918

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:		7.10.3.2

Source:				Keysight Technologies UK Ltd, ETS-Lindgren

Title:					On NTN Scenario 2 OTA Testing with Partial Radiated Surfaces

Document for:		Approval

# Introduction

This contribution is a continuation of [1] regarding partial radiated surfaces while taking agreements from the last meeting [2] into account.

# Discussion

In RAN4#115, a compromise was reached regarding the partial sphere parameters and DUT positioning [2], i.e.,

Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2 Agreements: - For Scenario 2 UEs: - The DUT shall be mounted in a suitable hand phantom - The elevation tilt of the default position of the DUT can be: - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below - [±15, ±30, ±45] degrees; Other tilt angles are not precluded - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center Agreements: the partial sphere metric is using the  =0° (top pole) as a center Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere Agreements: - RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°. - Option 1: single range is selected and captured in TR - Option 2: multi-range in TR but based on UE declaration, details are FFS - Note1: the phi range should be 360 degrees - Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2.

These agreements were a compromise to support some flexibility to fine tune the location of the partial surface with respect to the DUT, e.g., the ability to integrate around a partial surface [3] centered around a declared direction

Proposal 2: For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining the metric to be the surface integral of EIRP/EIS over the specified range of angles and normalized by the isotropic radiator performance over the same angles. Proposal 3: For handheld IoT/NR NTN devices declaring support of Scenario 2, RAN4 to consider defining an OEM declaration of the initial DUT position and the range of elevation and azimuth angles relative to the initial position over which the performance metric is calculated. Angular granularity of this declaration can be further discussed.

and the desire to keep the partial integration implementations simple for system integrators, e.g., to integrate over surfaces centred around q=0° [1].

Proposal 1: For scenario 2 IoT-NTN and NR-NTN handheld UEs, define just one partial surface for TX and RX testing, centered around q=0° (top pole) and with an angular width of either ±60° or ±90°.

The integration ranges of ±30°, ±60°, ±90° are visualized in Figure 1, Figure 2, and Figure 3, respectively. In these figures, the range of partial integration is shown in yellow, the grid points within that range are shown in green and those outside the range in red. The TX-based grids are shown with a grid point spacing of Dq=Df=15° while the RX-based grids are shown with a grid point spacing of Dq=Df=30°.

Figure 1: Visualization of partial surface with ±30° angular width for classical TRP grids (top) and TRS grids (bottom).

Figure 2: Visualization of partial surface with ±60° angular width for classical TRP grids (top) and TRS grids (bottom).

Figure 3: Visualization of partial surface with ±90° angular width for classical TRP grids (top) and TRS grids (bottom).

Table 1 summarized the partial surface integration metrics for the three different angular widths; more details are provided in Annex A.

Table 1: Overview of the partial surface integration metrics for different angular widths

| Angular Width of Partial Surface | TX | RX |
| --- | --- | --- |
| ±90° | UHPRP≅12a6cutTX,6+i=05cutTX,i | UHPRS=21cutRX,0+cutRX,1+cutRX,2+b3cutRX,3 |
| ±60° | UPRP±60≅12a4cutTX,4+i=03cutTX,i | UPRS±60=21cutRX,0+cutRX,1+b2cutRX,2 |
| ±30° | UPRP±30≅12a2cutTX,2+i=01cutTX,i | UPRS±30=21cutRX,0+b1cutRX,1 |
| where cutTX,i=1Mj=0M-1EiRPθθi,ϕj+EiRPϕθi,ϕj⋅Wθi and cutRX,i=1Mj=0M-11EISθ(θi,ϕj)+1EISϕ(θi,ϕj)⋅W(θi), Wθi are the quadrature weights of either the Clenshaw-Curtis (CC) weights or the traditional sin(q) weights, and ak=1-cosk*15°-i=0k-1W(θi)W(θk), bk=1-cosk*30°-i=0k-1W(θi)W(θk) the coefficients for the partial segments. |  |  |

Proposal 1: Include the three different metrics/partial ranges from Table 1 in [5] for NR NTN testing.

The current default smartphone orientation in the data/browsing mode is shown in Figure 4, i.e., the phone is tilted around the y axis by -45° with the x and z axes coming out of the screen [5], [6], [7].

Figure 4: Visualization of default data/browsing mode orientation of smartphone DUTs [5], [6], [7]

The agreed default DUT orientation for the Scenario 2 NTN OTA test cases is visualized in Figure 5; for simplicity, palm spacer (shown in gray) is omitted going forward for simplicity.

Figure 5: Visualization of default DUT orientation for Scenario 2

It can be observed that this DUT&phantom orientation is different than the traditional data/browsing mode due to the lack of -45° device/phantom tilt, i.e., the x axis is coming out of the screen just as it has been defined for traditional FS OTA as well as FR2 OTA testing as shown below from [8], [9]

Classical OTA: Figure A.5.2-1: Reference coordinate system and reference device orientation [8] FR2 OTA: Figure N.1-2: DUT default alignment of example smartphone UE to coordinate system [9]

Observation 1: The Scenario 2 NTN DUT&phantom orientation is different than the traditional data/browsing mode due to the lack of -45° device/phantom tilt

As agreed in [2], the device orientation can be declared by the OEM so that the main lobe(s) of the NTN antenna pattern can be captured appropriately by the partial integration surface centred around q=0° (top pole). The non-default device/phantom orientations, based on a vendor declaration are visualized in Figure 6. In these plots, the main (system) coordinate system (x, y, z) is fixed while the DUT coordinate system (with the x axis pointing out of the screen and the z axis pointing out of the top of the DUT) is rotating with the DUT.

Figure 6: Visualization of non-default DUT orientation for Scenario 2 with device tilts based on vendor declaration. Device&phantom tilts (from top left to bottom right): -45°, -30°, -15°, 15°, 35°, 45°,

It is proposed to limit the device/phantom tilts, declared by the OEM, aside from the standard orientation to ±45°, ±30°, ±15° only as visualized in Figure 6.

Proposal 2: Limit the device/phantom tilts, declared by the OEM, aside from the standard orientation to ±45°, ±30°, ±15° only as visualized in Figure 6.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The Scenario 2 NTN DUT&phantom orientation is different than the traditional data/browsing mode due to the lack of -45° device/phantom tilt

Proposal 1: Include the three different metrics/partial ranges from Table 1 in [5] for NR NTN testing.

Proposal 2: Limit the device/phantom tilts, declared by the OEM, aside from the standard orientation to ±45°, ±30°, ±15° only as visualized in Figure 6.

Proposal 3: The positioning uncertainty does not need to be re-visited for partial radiated metrics when compared to TRP/TRS for typical NTN patterns

# References

- R4-2505849, On Partial Radiated Surfaces, Keysight Technologies, 3GPP TSG-RAN4 Meeting #115, May 2025

- R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3, vivo, 3GPP TSG-RAN WG4 Meeting #115, May 2025

- R4-2504578, On NTN TRP TRS test methodology, Apple, 3GPP RAN4 Meeting #114bis, April 2025

- OTA191305, Partial Surface Integrals with CC or Surface Weighting, ETS-Lindgren
This is a CTIA Certification Program Working Group Document available to CTIA Certification Program Working Group members

- TR 38.870, Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)

- TS 38.161, User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios

- CTIA 01.71, Test Plan for Wireless Device Over-the-Air Performance, Device Setup and Positioning Guidelines

- TS 37.544, Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R4-2505850, On Split TRP/TRS Grids for Devices with Limited Battery Capacity, Keysight Technologies, Nokia, 3GPP TSG-RAN4 Meeting #115, May 2025

- R4-2419771, NTN OTA Test Metrics, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112, August 2024

# Annex A: Mathematical Expression of Metrics

### TX Metrics:

The upper-hemisphere partial radiated power metric, see Figure 3, can be expressed mathematically as

UHPRP≅12a6cutTX,6+i=05cutTX,iwhere

cutTX,i=1Mj=0M-1EiRPθθi,ϕj+EiRPϕθi,ϕj⋅Wθi

with

a6=1-cos90°-i=05W(θi)W(θ6)≈0.5128,  &sin(q) quadrature0.5000,  &CC quadrature

for both Clenshaw-Curtis or sin(q) quadratures [4] whose weights are expressed using Wθi. The TX cuts, cutTX,i, are further visualized in Figure 7.

The partial radiated power metric with EIRP integrated from q=0° to q=60° in the upper hemisphere, see Figure 2, can be expressed mathematically as

UPRP±60≅12a4cutTX,4+i=03cutTX,i

with

a4=1-cos60°-i=03W(θi)W(θ4)≈0.5126,  &sin(q) quadrature0.4875,  &CC quadrature

The partial radiated power metric with EIRP integrated from q=0° to q=30° in the upper hemisphere, see Figure 1, can be expressed mathematically as

UPRP±30≅12a2cutTX,2+i=01cutTX,i

with

a2=1-cos30°-i=01W(θi)W(θ2)≈0.5059,  &sin(q) quadrature0.4632,  &CC quadrature

Figure 7: Visualization of TX cuts, cutTX,i.

### RX Metrics:

The upper-hemisphere partial radiated sensitivity metric, see Figure 3, can be expressed mathematically as

UHPRS=21cutRX,0+cutRX,1+cutRX,2+b3cutRX,3where

cutRX,i=1Mj=0M-11EISθ(θi,ϕj)+1EISϕ(θi,ϕj)⋅W(θi)

with

b3=1-cos90°-i=02W(θi)W(θ3)≈0.5438,  &sin(q) quadrature0.5000,  &CC quadrature

for both Clenshaw-Curtis or sin(q) quadratures [4] whose weights are expressed using Wθi. The RX cuts, cutRX,i, are further visualized in Figure 8.

The partial radiated sensitivity metric with EIS integrated from q=0° to q=60° in the upper hemisphere, see Figure 2, can be expressed mathematically as

UPRS±60=21cutRX,0+cutRX,1+b2cutRX,2

with

b2=1-cos60°-i=01W(θi)W(θ2)≈0.5253,  &sin(q) quadrature0.4757,  &CC quadrature

The partial radiated sensitivity metric with EIS integrated from q=0° to q=30° in the upper hemisphere, see Figure 1, can be expressed mathematically as

UPRS±30=21cutRX,0+b1cutRX,1

with

b1=1-cos60°-i=00W(θi)W(θ1)≈0.5117,  &sin(q) quadrature0.4150,  &CC quadrature

Figure 8: Visualization of RX cuts, cutRX,i.

# Annex B: Visualization of DUT with Integration Surfaces including Grid Points

The following tables visualize the DUT with the conventional TRP/TRS measurement grids points shown in red. The green grid points visualize the grid points within the ±30°, ±60°, and ±90° partial surfaces that are used the calculate the corresponding performance metrics. The tables visualize the default DUT&phantom orientation on the left without tilt and the optional device tilts of ±45°, ±30°, and ±15°.

Table 2: DUT/Phantom Orientations and Integration Surfaces for TX testing with ±30° angular width and 15° grid step size.

| Default Orientation | Declared DUT Tilt of -45° (top)/45° (bot) | Declared DUT Tilt of -30° (top)/30° (bot) | Declared DUT Tilt of -15° (top)/15° (bot) |
| --- | --- | --- | --- |

Table 3: DUT/Phantom Orientations and Integration Surfaces for TX testing with ±60° angular width and 15° grid step size.

| Default Orientation | Declared DUT Tilt of -45° (top)/45° (bot) | Declared DUT Tilt of -30° (top)/30° (bot) | Declared DUT Tilt of -15° (top)/15° (bot) |
| --- | --- | --- | --- |

Table 4: DUT/Phantom Orientations and Integration Surfaces for TX testing with ±90° angular width and 15° grid step size.

| Default Orientation | Declared DUT Tilt of -45° (top)/45° (bot) | Declared DUT Tilt of -30° (top)/30° (bot) | Declared DUT Tilt of -15° (top)/15° (bot) |
| --- | --- | --- | --- |

Table 5: DUT/Phantom Orientations and Integration Surfaces for RX testing with ±30° angular width and 30° grid step size.

| Default Orientation | Declared DUT Tilt of -45° (top)/45° (bot) | Declared DUT Tilt of -30° (top)/30° (bot) | Declared DUT Tilt of -15° (top)/15° (bot) |
| --- | --- | --- | --- |

Table 6: DUT/Phantom Orientations and Integration Surfaces for RX testing with ±60° angular width and 30° grid step size.

| Default Orientation | Declared DUT Tilt of -45° (top)/45° (bot) | Declared DUT Tilt of -30° (top)/30° (bot) | Declared DUT Tilt of -15° (top)/15° (bot) |
| --- | --- | --- | --- |

Table 7: DUT/Phantom Orientations and Integration Surfaces for RX testing with ±90° angular width and 30° grid step size.

| Default Orientation | Declared DUT Tilt of -45° (top)/45° (bot) | Declared DUT Tilt of -30° (top)/30° (bot) | Declared DUT Tilt of -15° (top)/15° (bot) |
| --- | --- | --- | --- |

# Annex C: Impact of Positioning Errors on the Partial Integrated Surface Metrics

The analyses in this annex are based on the analyses initially presented in [10] while utilizing the simulated NTN antenna patterns from [11]. These patterns were simulated in CST Studio Suite. Here, a full smartphone model including most components was investigated first with the device held in the hand and is shown in Figure 9

Figure 9: CST Studio Suite smartphone model in hand right (HR) talk condition

Two different antenna locations for NTN antennas were investigated, i.e., one at the top (with frequency ~1.6 GHz, n255) and one on the side (with frequency range ~2.1 GHz, n256) as further illustrated in Figure 10.

Figure 10: NTN antennas investigated on top (left) and on side (right)

The simulated antenna patterns are shown in Figure 11.

Figure 11: NTN antennas patterns for antenna location on top (left) and on side (right)

From the patterns shown in Figure 11, it can be observed that the simulated antennas yield fairly omnidirectional patterns even in the HR talk condition. In the remainder of the contribution, the simulated antenna patterns had to be adjusted/rotated to comply with the conventional smartphone UE, e.g., Table 6.1-1 of [5] or Figure 5.

In order to analyse more directive patterns, two additional patterns with higher directivity were synthesized, generally using the sinc(theta) pattern shape. One has a 120° HPBW and the other a 45° HPBW, both with a 15 dB front-to-back-ratio (FTBR). All 4 analysed patterns are visualised in Figure 12.

Figure 12: Simulated and synthesized NTN antennas patterns. Top left: simulated smartphone antenna on top, top right: simulated smartphone antenna on right, bottom left: directive antenna with 120° HPBW, bottom right: directive antenna with 45° HPBW

For these simulations studying the impact of small angular offset in the positioning of the DUT, a random angular error placing the device into the phantom/DUT fixture in azimuth and elevation of up to ±{1°, 2.5°} is assumed. This is visualized in Figure 13. It should be noted that these simulations assume that phone/fixtures/phantom are all rotated at the same time since the analyses are based on applying rotations of the antenna patterns. Simulating offsets and misalignments of the phone only would require separate EM simulations since each phone shift/rotation could result in a different antenna pattern; those simulations would require a significant more effort and would likely result in similar findings.

Figure 13: Visualization of positioning errors applied to single TRP session.

A total of 10,000 different random positioning errors were simulated for these three cases and for each angular error, the respective antenna pattern was rotated and the TRP was recorded for each of the 10,000 simulations. The standard deviation of the metrics considered for TX are tabulated in Table 8.

Table 8: Impact of positioning error on standard deviation of TRP and Partial Radiated Power Metrics with Δθ=Δϕ=15°

|  | Max Angular Position Error of ±1° |  |  |  | Max Angular Position Error of ±2.5° |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TRP | UHPRP | UPRP60 | UPRP30 | TRP | UHPRP | UPRP60 | UPRP30 |
| Simulated NTN antenna in HR talk condition designed on top of DUT | 0.0045 | 0.0106 | 0.0180 | 0.0002 | 0.0112 | 0.0266 | 0.0449 | 0.0010 |
| Simulated NTN antenna in HR talk condition designed on side of DUT | 0.0201 | 0.0157 | 0.0138 | 0.0023 | 0.0505 | 0.0395 | 0.0348 | 0.0058 |
| Directive antenna with 120° HPBW | 0.0519 | 0.0483 | 0.0356 | 0.0188 | 0.1329 | 0.1225 | 0.0889 | 0.0471 |
| Directive antenna with 45° HPBW | 0.1282 | 0.1423 | 0.1643 | 0.1263 | 0.3224 | 0.3572 | 0.4114 | 0.3168 |

These results show that for most antenna patterns that are not highly directive, the impact on the standard deviation is the reduced the smaller the partial surface is. The positioning uncertainty therefore does not need to be re-visited for partial radiated metrics when compared to TRP/TRS for typical NTN patterns.

Proposal 3: The positioning uncertainty does not need to be re-visited for partial radiated metrics when compared to TRP/TRS for typical NTN patterns