3GPP TSG-RAN WG4 #116	R4-2509944

Bengaluru, India, August 25-29, 2025

Source: 	APCO International, FirstNet

Title: 	Discussion on coexistence requirements of NR TDD 4940-4990 MHz band for US public safety operation.

Agenda item:	6.14.2

Document for: 	Discussion and Decision

- Introduction

In RAN 4 #108, a new work item [1] for a new NR TDD 4940-4990 MHz band for US Public Safety operation was approved. The WID [1] includes a one quarter short study phase to identify and document coexistence requirements for the new band as specified below:

- Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band.

    - Identify adjacent spectrum protection requirements

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

    - Identify other technologies operating in the 4940-4990 MHz band

      - Identify potential coexistence mechanism(s)

        - Frequency division sharing

        - Geographic separation

        - TDD synchronization

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

In this discussion paper, we provide our views on coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band. We would like to request RAN4 to consider our input and findings to progress the work and start normative work as soon as possible.

- Discussion

To facilitate initial discussion, this contribution provides inputs for the study phase based on the FCC filings and announcements [2], and data from the incumbents that has been submitted to the FCC [3].

As discussed in the FCC’s Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking [2], there are already updates to 47 CFR Part 90 [4] to include spectrum emission masks in part 1 §90.210.  Emission Mask L defines the emission requirements for low power (20dBm or less) transmitter and Emission Mask M for high power (> 20dBm) transmitter.  For the new band in this WI, 3GPP should define emission requirements which satisfy these Emission Masks L and M as defined in §90.210 of 47 CFR 90 [4]. It was also concluded in [2] that emission mask M is sufficient for high power transmitter and no tighter emission mask is needed for high power operations in 4940-4990 MHz band.

Observation 1: The existing Emission Masks defined in §90.210 of 47 CFR 90 [4] are sufficient for defining emission requirements for the new NR TDD 4940-4990 MHz band.

Incumbent licensees have provided their granular data outlining their detailed use of the 4940-4990 MHz band to the FCC, ending the FCC’s data Collection effort on July 9, 2025 [3].    Incumbent use of the 4940-4990 MHz band can be in 1,2,3,5,10,15 and 20 MHz channel bandwidths.  Incumbent data collection [3] shows 95% of the incumbent users in the band operate OFDM-type of Fixed Point to Point and Point to MultiPoint services while the remaining operate as Base/Mobile or Mobile to Mobile systems.

Observation 2: Incumbent licensees have provided their granular data outlining their detailed use of the 4940-4990 MHz band to the FCC providing sufficient technical information about the incumbent users to proceed defining the new NR TDD 4940-4990 MHz band.

The FCC has also identified there will be a Band Manager that will be responsible for protecting the incumbents.  The methodology used by the Band Manager is not being specified by the FCC, it is expected to consist of several different technics that are regularly used when protecting existing incumbents [2][5].  Sufficient technical information about the incumbent users is available now for this purpose. These methodologies could include geographic separation, frequency division sharing and TDD synchronization, to name a few.  The FCC has clearly stated that protecting the existing incumbents is a top priority for the Band Manager and it will be the Band Manager’s responsibility to determine how to best meet this requirement, on a case-by-case basis.

Observation 3: Regardless of the technologies used, the mechanism to provide coexistence protection to the incumbents is the Band Manager’s responsibility.  The Band manager will determine the best solution, and the protection needed at what level they deem necessary; this could be as granular as site-by-site analysis if needed.  The decision and guidance are outside the scope of 3GPP.  The Band Manager reports to the FCC.

Another item to consider in the study phase of this work is regarding any potential emission limits more stringent than the normal BS and UE emission masks.  In a detailed review of the FCC Seventh Report and Order [2] there have been no unique or enhanced emission limits beyond what is defined in the Part 90 [4] which may impose emission limits more stringent than the normal BS and UE emission masks. Like any of the other 3GPP bands defined, if situations change it should be addressed and managed later.

Observation 4:  There are no unique or enhanced emission limit requirements in the FCC’s Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking [2] which impose emission limits more stringent than the normal BS and UE emission masks. Therefore, no additional work or considerations are needed.

Based on the above observations, we have sufficient relevant technical information to proceed with defining the new NR TDD 4940-4990 MHz band.  The commission also sought comment on whether emission mask M is sufficient, or whether a tighter emission mask should be imposed for high power operations. No party commented on high power emission masks, and we take no further action on the emission mask M at this time [2]. Therefore, there are no additional emission limit requirements identified by FCC for the new band which impose emission limits more stringent than the normal BS and UE emission masks. The existing Spectrum Emission Masks such as Emission Mask M defined in §90.210 of 47 CFR 90 [4] can be considered by 3GPP for defining emission requirements for the NR TDD 4940-4990 MHz band. Therefore, RAN4 should proceed with standardization of the 4940-4990 MHz band by incorporating the FCC emissions limits presented in FCC’s Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking [2].

Proposal 1: We have sufficient relevant technical information to proceed with defining the new NR TDD 4940-4990 MHz band. RAN4 proceeds with standardization of the new NR TDD 4940-4990 MHz band by incorporating the FCC emissions limits presented in FCC’s Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking.

- Conclusion

This contribution provides our perspective and discussion points for coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz (4.9 GHz) band for the introduction of NR TDD 4.9GHz Band for US Operation. The following proposals and observations are made:

Proposal 1: We have sufficient relevant technical information to proceed with defining the new NR TDD 4940-4990 MHz band. RAN4 proceeds with standardization of the new NR TDD 4940-4990 MHz band by incorporating the FCC emissions limits presented in FCC’s Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking.

Observations:

Observation 1: The existing Emission Masks defined in §90.210 of 47 CFR 90 [4] are sufficient for defining emission requirements for the new NR TDD 4940-4990 MHz band.

Observation 2: Incumbent licensees have provided their granular data outlining their detailed use of the 4940-4990 MHz band to the FCC providing sufficient technical information about the incumbent users to proceed defining the new NR TDD 4940-4990 MHz band.

Observation 3: Regardless of the technologies used the mechanism to provide coexistence protection to the incumbents is the Band Manager’s responsibility.  The Band manager will determine the best solution, and the protection needed at what level they deem necessary; this could be as granular as site-by-site analysis if needed.  The decision and guidance are outside the scope of 3GPP.  The Band Manager reports to the FCC.

Observation 4:  There are no unique or enhanced emission limit requirements in the FCC’s Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking [2] which impose emission limits more stringent than the normal BS and UE emission masks. Therefore, no additional work or considerations are needed.

- References

- RP-251887 “New WID on Introduction of NR TDD 4.9GHz Band for US Operation,” 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 09-13, 2025.

- See FCC’s Seventh Report and Order and Ninth Further Notice of Proposed Rulemaking, Docket 07-100 issued January 18, 2023 paragraphs 30,33,34,39,48 FCC-23-3A1.pdf

- See FCC Universal Licensing Database, Radio Service Codes PF (Fixed Services) and PB (Base/Mobile and Temporary Fixed Operation)  License Search - Advanced License Search. Note: When using the link, search for Radio Service PF (Fixed Service using 4.9 GHz and PB (Base/Mobile-Temporary Fixed Operations)

- 47 CFR Part 90 – Private Land Mobile Radio Service, https://www.ecfr.gov/current/title-47/chapter-I/subchapter-D/part-90

- See FCC’s Eighth Report and Order, Docket No. 07-100 issued October 22, 2024; FCC-24-114A1_Rcd.pdf