3GPP TSG-RAN WG4 Meeting #116	R4-2509955

Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on RRM core requirements for (e)RedCap in NR NTN

Agenda item:	7.27.5.1

Document for:	Discussion

Introduction

RAN plenary has revised the WI for NR NTN enhancements in [1], which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_Ph3).

In this document we provide our views on the 5th objective, namely the support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems, related to RRM core requirements considering the open issues identified in the latest WF from RAN4 #115 [2] and further detailed in the Topic summary [3].

Discussion

In the present document we will discuss the open issues presented in the WF [2] and the Topic summary [3], as listed below:

- Issue 1-1-2: MDT requirements

- Issue 1-2-2: SMTC adjustment

- Issue 1-3-5: Impact on NR Handover due to HD-FDD

- Issue 1-4-1: UE capabilities for (e)RedCap UE with FR1-NTN bands

- Issue 1-4-2: UE feature list

  - General consideration on RRM requirements

RAN4 has discussed general aspects for RRM requirements for (e)RedCap UE with FR1-NTN.

MDT requirements

Following agreement was achieved for MDT requirements [2].

RAN4#114: #### Issue 1-1-3: MDT requirements Agreement: - For MDT requirements in RRC_IDLE and RRC_INACTIVE state, - Deprioritize to define Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN.

At RAN4#115, the following way forward was provided by Moderator [2].

Issue 1-1-2: MDT requirements <Way Forward>: - Proposal 1: Not to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap with NTN. - Proposal 2: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN. The legacy requirement from NTN can be reused. - Proposal 3: RAN4 to discuss whether to send LS to RAN2 / RAN3 informing them about the RAN4 agreement on specifying MDT requirements in RRC_IDLE and RRC_INACTIVE for (e)RedCap UE supporting MDT. - Proposal 4: - RAN4 shall consider the necessity of defining MDT requirements and then decide whether to define relevant MDT requirements. - For measurement requirements, the legacy measurement requirements shall be reused for 1Rx and 2Rx RedCap UEs in NTN. - For accuracy relative time stamp requirements, RAN4 shall reuse the same drift of relative time stamp for 1Rx and 2Rx RedCap UEs in NTN.

In fact, RAN4 has taken an earlier decision to deprioritize MDT requirements for (e)RedCap in FR1 NTN. Though, we understand the concern raised in [4]. Moreover, changes to TS 38.133 are fairly small, as only applicability to (e)RedCap UE has to be added in the existing clause 4.3C (RRC_IDLE) and clause 5.3C (RRC_INACTIVE) for UE’s supporting MDT. Hence RAN4 should agree to specify MDT requirements for (e)RedCap UE supporting MDT. In our view, RAN2 does not need to be informed on this change. However, support of MDT is optional for (e)RedCap UE with FR1-NTN, and hence it should be added to RAN4 UE capability list.

RAN4 to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for (e)Redcap over NTN. The legacy requirements for NTN in clauses 4.3C and 5.3C can be reused.

RAN4 to discuss the addition of optional UE capability for MDT support for (e)RedCap UE with FR1-NTN.

SMTC adjustment

At RAN4#115, the following way forward was provided by Moderator [2].

#### Issue 1-2-2: SMTC adjustment in idle/inactive <Way Forward>: - Proposal 1: SMTC adjustment in idle/inactive is kept mandatory for (e)RedCap UEs with NTN. - Proposal 2: RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. - Proposal 2a: Maintain the same principle used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode. - Proposal 3: For SMTC adjustment in idle/inactive, we recommend keeping it optional, yet we would support defining it as mandatory if companies have strong concern on this.

There is an ongoing discussion whether the SMTC adjustment can be considered a mandatory feature for NTN UEs in idle/inactive mode. In fact, the NTN support described in TS 38.306 does not require the SMTC adjustment to be mandatorily supported, and in TS 38.300 the following is specified:

NW-controlled adjustment of SMTCs can be based on UE assistance information reported in RRC_CONNECTED. A UE in RRC_IDLE/RRC_INACTIVE can adjust SMTCs based on its location and assistance information in SIB19.

The highlighted “can” on the box above is our own, but it means that the UE was not mandated to adjust the SMTC window based on its location and ephemeris information. However, the requirements are made transparent to UE implementation, i.e., whether the UE implements the shifting SMTC or not, this does not impact the applicability of the requirements. In NTN, the requirements are applicable considering that a UE is adjusting its SMTCs, but it does not preclude other UE implementations, as long as the UE complies with the idle/inactive mode mobility requirements. The same principle should be valid for RedCap over NTN. Thus, we support Proposals 2 and 2a above.

RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. The same principle is maintained as used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

Impact of HD-FDD

Impact on NR Handover

At RAN4 #115, the following way forward was provided by Moderator [2].

#### Issue 1-3-5: The impact on NR Handover due to HD-FDD <Way Forward>: - Proposal 1: For the impact on NR handover for HD-FDD, the principle in option 1 is accepted. - Proposal 1a: Regarding Issue 3-2-1 (The impact on NR Handover due to HD-FDD), adopt Option 1 with 'availability' defined as 'no collision with UL transmission at the UE'. - Proposal 2: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

Referred Option 1 is as follows [3].

#### Issue 1-3-5: The impact on NR Handover due to HD-FDD - Background The agreement in the last meeting: #### Issue 3-2-1: The impact on NR Handover due to HD-FDD Agreement: - Option 1: agree on the principle below and further discuss the wording in the CR: - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU - Option 2: No spec impact.

In our view, there should not be any difference in UE behaviour between the legacy NTN device and (e)RedCap device with FR1-NTN during NR handover, no matter whether the device operates in FDD or HD-FDD mode. In fact, when the (e)RedCap device receives the RRC reconfiguration command, it is expected to search and camp on the HO target cell as soon as possible. Thus, assuming a collision with UL traffic during HO interruption is not reasonable, also since the radio coverage to the old serving cell may rapidly degrade, especially in the NTN case and hence UL transmission would impact UE’s battery and in addition degrade spectral efficiency. Thus, we support Proposal 2.

RAN4 to reuse the RRM requirements for legacy NTN for HO, CHO and satellite switch for HD-FDD UE.

- 2.3 	UE capability

- 2.3.1	UE capabilities for (e)RedCap UE with FR1-NTN bands

Following way forward was proposed by Moderator [2].

#### Issue 1-4-1: General principle for designing the UE capabilities for (e)RedCap UE with NTN <Way Forward>: - Option 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). (CATT, CMCC, ZTE, Xiaomi, E///, Samsung - compromise, Apple, QC) - FFS: The Rel-19 requirement can be release independent from Rel-17. - Option 2: RAN4 to introduce new capability to declare support over RedCap over NTN. (vivo, Samsung) - Option 3: to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN). (Nokia, QC, MTK)

In fact, RAN1 has implicitly added this discussion on their current feature list discussion [5]. Currently the RedCap over NTN feature group is supported as follows:

| 65. NR_NTN_Ph3 | 65-2-1 | Handling collision Case 3 [and collision Case 4] for half-duplex FDD operation for (e)RedCap UE | 1. Support handling collision Case 3 [2. Support handling collision Case 4] FFS: whether to divide this FG into two separate FGs between case 3 and case 4 | FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission [Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission] Note: this FG is for FR1 FDD only Note: collision Case 3 handling is for RRC CONNECTED and INACTIVE states[, and collision Case 4 is for RRC CONNECTED state only] | Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The highlighted part in the fifth column indicates that the current working assumption to be confirmed in RAN1 is that a UE that supports the feature group for RedCap operations over NTN must support FG 26 (NR NTN) and FG 28 (RedCap).

RAN1 working assumption is that the UE that supports NR NTN and RedCap shall support RedCap over NTN.

Therefore, we propose to wait for RAN1 to confirm the current working assumption and do not discuss the issue in RAN4 unless needed.

RAN4 to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN).

- 2.3.2	UE feature list

Following issue was treated in [3].

#### Issue 1-4-2: UE feature list - Proposals - Proposal 1 (CATT): Introduce new UE capability to indicate the supported SMTC in parallel for (e)RedCap UE with NTN. Feature group; Components; Prerequisite feature groups; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. Parallel measurements on multiple SMTC-s for a single frequency carrier; Support of measurements on target cells belonging to 4 SMTC-s on a single frequency carrier; FDD only; FR1 only; RedCap UE is mandatory to support 2 if the feature is supported; Optional with capability signaling. - Recommended WF - To be discussed.

This issue related to parallel measurements on multiple SMTC-s on single carrier is connected to the UE capability discussion above. If we assume that an (e)RedCap UE with FR1-NTN also supports NTN, then there is no need for including the parallel SMTC measurement support in the feature list. However, this conclusion depends on the outcome of the UE capability discussion in RAN1.

RAN4 to wait for RAN1 progress on the UE capability discussion to decide further proceeding on the feature parallel SMTC measurement support for (e)RedCap UE with FR1-NTN.

Conclusion

This paper has analysed the open issues provided in the latest WF for this WI [2], and the following set of observations and proposals are made:

- RAN4 to define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for (e)Redcap over NTN. The legacy requirements for NTN in clauses 4.3C and 5.3C can be reused.

RAN4 to discuss the addition of optional UE capability for MDT support for (e)RedCap UE with FR1-NTN.

RAN4 will not make any modifications on the feature of SMTC adjustment in idle/inactive for RedCap UE with NTN. The same principle is maintained as used in NR NTN for RedCap devices: the requirements are transparent to whether the UE adjusts or not the SMTC window in IDLE/INACTIVE mode.

RAN4 to reuse the RRM requirements for legacy NTN for HO, CHO and satellite switch for HD-FDD UE.

- RAN1 working assumption is that the UE that supports NR NTN and RedCap shall support RedCap over NTN.

RAN4 to wait for RAN1 to confirm the working assumption (UE that supports RedCap and NTN shall support RedCap over NTN).

RAN4 to wait for RAN1 progress on the UE capability discussion to decide further proceeding on the feature parallel SMTC measurement support for (e)RedCap UE with FR1-NTN.

- References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2508287, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #115

- R4-2505576, Topic summary for [115][227] NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #115

- R4-2506431, Discussion on the RRM requirement of (e)Redcap UE for NTN phase3, CMCC

- R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121, Moderators (AT&T, NTT DOCOMO, INC.), RAN WG1 #121, St Julian’s, Malta, May 19th – 23rd, 2025