3GPP TSG RAN WG4 Meeting #116	R4-2509994

# Bengaluru, India, 25 – 29 August, 2025

Source: 	NEC

Title: 	Discussion on OTA test scope reduction

Agenda Item:		7.9.4.2.1

Document for:	approval

1	Introduction

This contribution discusses the BS OTA test scope reduction, focusing on interference frequency offset for Tx intermodulation requirements and interference frequency step size for out-of-band blocking requirements, and provides proposals.

2	Discussion

In the previous meetings, BS OTA test scope reduction was discussed and a WF [1] was agreed.

Test scope reduction for spurious emission could be finalized. However, there are still options for interference frequency offset for Tx intermodulation conformance testing for OBUE limits, and for interference step size for out-of-band requirement conformance testing. We further discuss these issues.

2.1	Interferer frequency offset for Tx intermodulation

Agreements for OBUE limit test simplification in the previous meeting [1] are blow.

- Further discuss how to reduce the test simplification for OBUE limits.

  - Option 1: 1xCBW offset

  - Option 2: 3xCBW offset

  - Option 3: 3xCBW offset if 3rd interfering signal is falling within the range of FDL_low to FDL_high of the operating bands and 1xCBW and 2xCBW if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE

We keep our position as stated in our previous contribution [2] in principle. Frequency offset to check OBUE limits should be decided so that 3rd order intermodulation products cover the frequency range beyond 10 MHz from the wanted signal channel edge where the OBUE limit is most stringent. Both 1xCBW and 3xCBW frequency offsets meet this condition. Considering the interfering signal position based on 3rd order frequency offset has higher risk that it may be outside of the OBUE range, 1xCBW offset looks more suitable than 3xCBW offset to check OBUE limits. Based on this, we think option 1 has advantage over option 2 as we could not say which offset is more stringent for OBUE performance. Option 3 newly proposed in the previous meeting proposes 3xCBW offset if 3rd interfering signal falls within OBUE range but proposes 1xCBW and 2xCBW if not. We do not think it makes sense. Again, unless we have clear evidence that shows 3xCBW offset is more stringent for OBUE performance than 1xCBW or 2xCBW offset we do not see any reason to select the interfering signal position conditionally. It makes the specification complex unnecessary. It should be noted that option 3 requests 2 interfering signal positions if the 3rd interfering signal does not fall within OBUE range. It is not preferable for testing workload. We are discussing the test scope reduction.

Based on the discussion above, we support option 1 to check OBUE limits.

Proposal 1:

To adopt 1st order frequency offset to check OBUE limits.

2.2	Interferer frequency step size for out-of-band blocking

Agreements in the previous meeting are blow [1].

  - Agreement on step size of OOBB requirement for further discussion:

  - Option 1: 1/3 of the minimum supported bandwidth

  - Option 2:  Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

  - Option 3: mixed EMC and option 1 approach, FFS on the details.

  - Option 4: To use 2/3 of the test signal configuration channel bandwidths

  - Option 5: Use 1MHz steps size below 100MHz and 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz.

  - Other options are not precluded

To decide the interferer frequency step size for out-of-band blocking, we should consider the principle design for Rel-15 FR2 BS OOBB CW step size in TR 38.817-02 [3] shown below.

The base station out-of-band blocking requirement is tested with a CW interferer and the wanted uplink signal. A spurious response occurs when the m-th harmonic of the interferer frequency mixes with the n-th harmonic of an internal signal, like a clock or a local oscillator signal, and this mixing product falls within the receive bandwidth of the wanted signal (or near the receive bandwidth, if the receiver selectivity is insufficient to suppress the spurious mixing product). The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. m can assume the values 1, 2, 3, … and n can assume the values 0, 1, 2, …. Spurious responses for which the harmonic number m of the interferer signal equals 4 or more are usually not significant. Hence, the frequency step size should be chosen such that any spurious response due to a maximum third harmonic of the interferer will be captured within the receive bandwidth of the wanted signal.

We have reviewed the options in the WF [1].

Option 1 proposes the frequency step size of 1/3 of the minimum supported bandwidth. It secures the spurious response due to a 3rd harmonic of the interferer signal captured within the receive bandwidth. Therefore, we support this option.

Option 2 proposes the frequency step size of up to 60 MHz. It may result in any spurious responses due to a 3rd harmonic of the interferer signal may not be captured within the receive bandwidth of the wanted signal when the frequency step size is large. We do not think we can assume passing the tests with this option guarantees passing the test with other interference frequencies. On the other hand, when the momentary frequency is low, interference step size could be as small as 0.3 MHz. It is smaller than the step size in the current specification. It means it needs more conformance testing time to test certain interferer frequency range. We are not happy with it. We also think using 1% of the momentary frequency as the step size makes the conformance test setting complex.

Option 3, detail is still not clear.

Option 4 proposes the frequency step size of 2/3 of test signal configuration channel bandwidths. It does not guarantee the 3rd harmonic of the interferer signal captured within the receive bandwidth. It means than conformance with option 4 does not secure the out-of-band blocking performance with other interfering signal offset conditions. We do not support this option.

Option 5, same as the above discussion for option 4, this option does not guarantee the 3rd harmonic of the interferer signal captured within the receive bandwidth. We do not support this option.

Based on the discussion above, we support option 1 for out-of-band blocking interferer signal frequency step size.

Proposal 2:

To adopt 1/3 of the minimum supported bandwidth for out-of-band blocking interferer signal frequency step size.

Conclusions

We discussed the issues of the NCR ACLR requirement for the simultaneous NCR-Fwd and NCR-MT transmission and provided the following proposal.

We discusses the BS OTA test scope reduction, focusing on interference frequency offset for Tx intermodulation requirements and interference frequency step size for out-of-band blocking requirements, and provide following proposals.

Proposal 1:

To adopt 1st order frequency offset to check OBUE limits.

Proposal 2:

To adopt 1/3 of the minimum supported bandwidth for out-of-band blocking interferer signal frequency step size.

Reference

[1] R4-2508654, “Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP”, ZTE

[2] R4-2506933, “Discussion on OTA test scope reduction”, NEC

[3] TR 38.817-02