TSG-RAN Working Group 4 (Radio) meeting #116	R4-2510227

Bangalore, India, 25th to 29th August 2025

Source:	Ericsson

Title:	On improvements of co-location requirements

Agenda item:	7.9.3.2

Document for:	Approval

# Introduction

At the last meeting (RAN4#115 in Malta) RAN4 collected outcome from coupling loss measurements and simulations from several companies. The results were collected in a way-forward contribution in [2]. It can be noticed that as expected the BS-to-BS coupling loss varies as function of frequency and it indicates that 30 dB may not be relevant as assumption for requirement derivation for bands in the upper region of FR1.

In this contribution we have collected information related to the technical background used for requirement derivation for all co-location requirements. The intention is to identify possibilities to develop a requirement concept where co-location requirements are not defined around a specified co-location reference antenna and a corresponding Co-Location Test Antenna (CLTA) used for conformance testing. Such approach would allow for testing co-location requirements in commercially available test rages using regular measurement antennas (e.g. horn antennas, dipoles, etc.) in the similar fashion as for e.g. regular OTA testing and EMC testing. This would guarantee more robust co-location requirements enabling reproducibility and availability for more flexible OTA conformance testing.

# Discussion

In Rel-19 BS RF evolution WI (NR_BS_RF_req_evo), the BS-to-BS port coupling loss for two different co-location scenarios have been evaluated through measurements and simulations.

- Vertical separation (Often used for tower sharing situations)

- Horizontal separation (side-by-side separated 0.1 m, according to TS 38.104, Clause 4.9)

At last meeting the outcome from the coupling loss investigation was summarized for different situations in a way-forward [2]. In this contribution the information has been restructured to clearer map to different situations, as listed in Table 2-1 intra band situations and Table 2-2 for inter band situations.

Table 2-1: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical separation | SA-to-SA | M: 54 (R4-2504358) | M: 53 (R4-2504358) | M: 56 (R4-2504358) |  |
|  | A-to-SA |  |  |  |  |
|  | A-to-A |  |  |  |  |
| Horizontal separation | SA-to-SA | M: 31 (R4-2504358) | M: 34 (R4-2504358) | M: 46 (R4-2504358) |  |
|  | A-to-SA |  | M: 35 (R4-2507633) S: 55±15 (R4-2506307) | M: 51 (R4-2507445) M: 69 (R4-2507511) | M: 55 (R4-2507633) S: 70±15 (R4-2506307) |
|  | A-to-A |  |  |  | S: 77±15 (R4-2506307) |

Table 2-2: Inter band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz to 3.5 GHz | 3.5 GHz to 4.9 GHz | 4.9 GHz to 7 GHz | 3.5 GHz to 7 GHz |
| Vertical separation | SA-to-SA |  |  |  |  |
|  | A-to-SA |  |  |  |  |
|  | A-to-A |  |  |  |  |
| Horizontal separation | SA-to-SA |  |  |  |  |
|  | A-to-SA |  |  |  | S: 57±10 (R4-2506307) |
|  | A-to-A |  |  |  | S: 54±10 (R4-2506307) |

The result consists of measurement results (M) and simulation results (S) for different coupling types:

- Sub-Array-to-Sub-Array (SA-to-SA) coupling loss

- Array-to-Sub-Array (A-to-SA) coupling loss

- Array-to-Array (A-to-A) coupling loss

For the table entries left blank no information have been provided to RAN4, which would indicate that the work is not complete.

Even though the coupling loss evaluation is not complete for all relevant cases, it’s clear that the outcome indicates that the assumption of a fixed BS-to-BS port coupling loss assumed to 30 dB for FR1 is not relevant anymore, especially not for AAS BS operating in the upper region of FR1. From the combined result from measurements and simulations it can be concluded that the coupling loss varies between 31 to 92 dB considering all scenarios, bands and coupling types. The general trend indicates that coupling loss increases for bands in the upper region of FR1.

Observation 1: It can be noticed that the coupling loss tables are not complete. RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements.

  - Requirement background

For MSR AAS BS and NR, co-location requirements are requirements which are based on a given BS-to-BS co-location scenario with a given assumption of port-to-port coupling loss of 30 dB. The intention with these requirements is to ensure that BSs serving different bands can be co-sited at the same site and/or tower. A set of co-location requirements have been defined for AAS BS in TS 37.105 and for NR BS type 1-C, BS type 1-H and BS type 1-O in TS 38.104. In Table 2.1-1, all the BS co-location requirements are listed together with references to individual requirements in TS 38.104.

Table 2.1-1: Overview of NR co-location requirements

| Requirement | Reference to clauses in TS 38.104 | Description |
| --- | --- | --- |
| Transmitter spurious emission | BS type 1-C and BS type 1-H in Clause 6.6.5.2.4. BS type 1-O in Clause 9.7.5.2.5. | This requirement is defined to protect BS operating in other 3GPP band(s) co-located at the same site. |
| Receiver out-of-band blocking | BS type 1-C and BS type 1-H in Clause 7.5.3. BS type 1-O in Clause 10.6.2.2. | This requirement is defined to guarantee UL operation when a BS is co-located to another BS operating at another 3GPP band(s). |
| Transmitter OFF power | BS type 1-C in Clause 6.4.1.2. BS type 1-H in Clause 6.4.1.3. BS type 1-O is Clause 9.5.2.2. | This requirement is defined to protect other BS(s) in the same network in neighbouring cells. |
| Transmitter intermodulation | BS type 1-C in Clause 6.7.2. BS type 1-H in Clause 6.7.3. BS type 1-O in Clause 9.8.2. | This requirement guarantees transmitter robustness with respect to an external interferer. |

Originally all requirements were derived for non-AAS BS (BS type 1-C). In Rel-13, hybrid AAS BS was introduced (In NR specifications referred to as BS type 1-H) and in Rel-15 all OTA AAS BS was introduce (In NR specifications referred to as BS type 1-O). For BS type 1-C and BS type 1-H, all requirements are defined at the transceiver ports (also known as the Transceiver Array Boundary (TAB) connector(s)). For BS type 1-O all co-location requirements are defined based on a concept of a co-location reference antenna. In conformance testing this antenna is referred to Co-Location Reference Antenna (CLTA).

Looking at the details related to the technical background for each induvial requirement it can be noticed that the BS-to-BS coupling loss assumed is of different type depending on requirement. In Tabel 2.1-2 some further details for each requirement in presented.

Table 2.1-2: Requirement derivation assumptions

| Requirement | Coupling type | Frequency aspect | Description |
| --- | --- | --- | --- |
| Transmitter spurious emission | A-to-A | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. The approach used to derive the requirement level is defined to consider A-to-A coupling. |
| Receiver out-of-band blocking | A-to-SA | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. Since the receiver blocking is related to induvial receiver branches, A-to-SA is considered when the requirement level is derived. |
| Transmitter OFF power | A-to-A | Intra band | The requirement is defined for TDD operation to protect the neighbouring BS in the same network. The approach used to derive the requirement level is defined to consider A-to-A coupling within the same band. The A-to-A coupling for this requirement should be based on a neighbouring BS in the same network and not side-by-side at the same site. |
| Transmitter intermodulation | A-to-SA | Intra band | This requirement is defined to guarantee than unwanted emission requirements is met in the case of an interferer signal. The interferer signal is injected within the band. Since the requirement is related to induvial transmitter branches, A-to-SA is considered when the requirement level is derived. |

It is worth to notice that the background for BS-to-BS coupling is different for all requirements listed in Table 2.1-2. A complete technical background for all requirements would require a coupling evaluation including following cases:

- Intra band A-to-A for relevant band combinations

- Inter band A-to-SA for relevant band combinations

- Intra band A-to-A for relevant bands considering coupling to neighbouring BS

- Intra band A-to-SA for relevant bands

Analysing the results presented in Table 2-1 and Table 2-2 would indicate that assuming BS-to-BS port coupling loss equal to a value larger than 30 dB would be reasonable for bands in the upper region of FR1. To be able to decide on a relevant coupling value for all cases, RAN4 need to adopt a common simulation methodology.

Observation 2: To establish relevant coupling assumptions for all requirements RAN4 need to establish a common simulation methodology and conduct a simulation campaign. It’s not possible to establish coupling vales based on measurements results, due to the large scope and different combination of bands and antenna configuration.

In Section 2.3 further details related to how Near-Field coupling loss can be simulated is captured. In a companion contribution [6], we also present information on how near-field measurements can be related to coupling loss. This information can be used to understand how regular antennas can be used to inject an interference signal and measure emission instead of using a specified CLTA.

  - Requirement derivation

In this section the technical background part of the requirement derivation for classical requirements is captured as reference. In Rel-15, OTA requirement was defined for AAS BS by mapping conducted requirements to the OTA domain.

Based on this assumption the BS-to-BS port coupling loss equal to 30 dB was originally derived. The detailed description can be found in TR 25.942, Clause 10.1. A single assumption on isolation was adopted for UTRA and applied in requirement derivation. At the time very few bands were considered in the range 900 MHz to 2000 MHz. The scenario considered is very different for the scenario introduced for AAS BS in Rel-15.

For any given co-location scenario, it would be reasonable to assume that the isolation between base stations will depend on frequency. With NR FR1 the frequency range spans from 410 MHz to 7125 MHz. It would be reasonable to assume that the BS-to-BS port coupling loss would increase for situations where the aggressor base station and victim base station is operating in different frequency bands, due to array antenna detuning effects and increased out-of-band transmission losses in Radio Distribution Network (RDN). In addition, the BS coupling loss will also depend on the antenna sizes. If two co-located base stations are operating a band close to each other the size is expected to be similar, while two base stations operating at 900 MHz and 3500 MHz, the size will be very different (as showed in companion contribution [7]) which would affect the coupling loss.

When requirements for AAS BS was established in Rel-15 for NR AAS BS type 1-O the legacy base station requirements were translated to the OTA domain. Unfortunately, RAN4 at the time didn’t reach the goal to define co-location requirements based on regular OTA parameters to be verified in regular OTA test environments. Instead, a concept based on a specified co-location reference antenna for a specific co-location situation was developed. The new co-location requirements are anchored at the RF connector of the co-location reference antenna.

For the Rel-15 requirements derived in TR 37.843 a co-location reference antenna based on a single column passive antenna is specified. The concept focused on early versions on AAS BS with antenna apertures with similar sizes are passive base station antennas operating below 2500 MHz. Now the with larger antennas operating at frequencies above 2500 MHz some of the fundamental assumption used for the Rel-15 co-location concept need further evolution to support bands in the upper region of FR1.

In the following section the technical background for each requirement is presented tighter with some ideas on how to proceed the requirement evolution considering BS-to-BS coupling loss to be around 50 dB. Larger coupling loss typically facilitates practical issues with respect to OTA testing (e.g. makes emission detectable in OTA test ranges using regular OTA parameters).

    - Transmitter spurious emission

The requirement background comes from the background that multiple networks operating within different 3GPP bands can be co-located in the same geographical area. This means that the aggressor base station transmitter must protect the victim base station receiver to guarantee operation with negligible performance degradation. For this purpose, 3GPP RAN4 defined a requirement for transmitter spurious emission within other 3GPP bands.

The transmitter spurious emission requirement level was originally derived from calculating emission caused by the aggressor base station within the co-located victim base station in the same geographical area receiver to maintain a maximum allowed co-location receiver sensitivity degradation. Originally, a scenario with two equal base station sites close to each other was considered. The isolation was derived based on two fixed beams from passive BS antennas pointing towards each other. The main beam for each base station was directed towards the ground to achieve coverage.

Based on the BS-to-BS coupling loss and few other assumptions the requirement level of conducted power at the aggressor base station can be established as shown in Figure 2.2.1-1.

Figure 2.2.1-1: Illustration of transmitter spurious emission requirement level derivation

The requirement level for maximum allowed spurious emission for a co-located base station have been calculated in logarithmical scale as:

Pem=kT+B+F-M+CL1		(Equation 2.2.1-1)

where kT is the thermal noise equal to -174 dBm/Hz, B is the considered receiver bandwidth in dBHz, F is the noise figure in dB, M is the noise margin corresponding to the allowed victim receiver sensitivity degradation and CL1 is the BS-to-BS port coupling loss in dB.

For co-located networks RAN4 have decided that the victim receiver noise floor can increase 0.5 dB due to a co-located network operating in another band in the same geographical area. The impact to the victim receiver total noise of 0.5 dB corresponds to an emission level M equal to 9 dB below the victim receiver noise floor. For FR1, RAN4 often assumes a BS noise figure F equal to 7 dB and BS-to-BS port coupling loss CL1 equal to 30 dB. Considering 100 kHz measurement bandwidth the maximum allowed transmitter spurious emission power spectral density level Pem can be calculated to -96 dBm/100 kHz.

This power level is used as the requirement level for NR BS type 1-C and BS type 1-H. It’s also used as background for the requirement level derived for NR BS type 1-O.

In Rel-15 when the BS type 1-O requirement was developed it was not practical to establish an OTA level due to the expected low emission levels to be measured. Instead, a requirement concept based on a CLTA was developed.  Now when we know that for bands in the upper region of FR1, the coupling is larger this decision needs to be revisited. Different reference points for spurious emission are considered:

- Total conducted spurious emission level at the aggressor, Pem

- Total conducted spurious emission level at the CLTA, Pem,CLTA

In Figure 2.2.1-2, the aggressor BS and victim BS are visualised with all different reference points. With the Rel-15 concept, the co-location reference antenna is used to mimic the victim BS. In the figure below a scenario considering co-location between AAS BS with AAS BS is considered since that is more relevant for higher frequencies within FR1.

# Figure 2.2.1-2: CLTA used to measure transmitter spurious emission

In Rel-15, the requirement level at the co-location reference antenna in conformance test specification TS 38.141-2 referred to as the CLTA was determined as:

Pem, CLTA=Pem-CL2+X		(Equation 2.2.1-2)

where CL2 is the isolation between the AAS BS and the passive BS antenna (used as CLTA), X is a scaling factor equal to 9 dB added to account additional noise due to 8-port MIMO systems already deployed before Rel-15.

Observation 3: For transmitter spurious emission requirement, it can be noticed that the 9 dB additional noise would change the meaning of the original thought behind the requirement. With 9 dB additional noise, the impact on the victim receiver will be significantly larger than 0.5 dB degradation of the sensitivity.

For the evolution of BS RF requirements, it is proposed to set the scaling factor to 0 dB to guarantee the intention with the requirement. This would also follow guidance from several administrations for general spurious emissions.

In Rel-15 RAN4 decided to assume that CL1 was set equal toCL2 which was set equal to 30 dB. By combining Equation 2.2.1-1 and Equation 2.2.1-2 the coupling loss cancels out. This would mean that the co-location concept based on the CLTA will embed the isolation required making the requirement derivation independent of the BS-to-BS port coupling loss. Hence, the requirement level is completely defined by the victim receiver properties. This would indicate a simple solution for how co-location spurious emission requirement can be derived. However, this concept is not practical and relevant for scenarios where the size of the aggressor BS antenna and the victim BS antenna is different, e.g. as for the case considering a low band passive antenna and high band AAS BS.

Observation 4: With the Rel-15 co-location requirement concept the impact of the BS-to-BS port coupling loss cancel out in the transmitter spurious emission requirement level derivation. This would indicate that the transmitter spurious emission requirement level is not dependent on frequency and antenna size, which is not in line with simulation results and measurements.

Observation 5: To evolve BS co-location transmitter spurious emission requirement to properly support bands in the upper region of FR1 it is now time for RAN4 to develop an enhanced concept for transmitter spurious emission requirement to properly capture aspects related to frequency offset between aggressor and victim, asymmetry between aggressor array antenna size and victim array antenna size and aggressor and victim beamforming characteristics. The relevance of the current requirement concept is highly questionable, and testability is questionable.

For the case where the coupling loss is larger than 30 dB it would be reasonable to consider defining the transmitter spurious emission level as a regular TRP emission requirement level. As an example, if 50 dB coupling loss is assumed the spurious emission level Pem,TRP can be calculated to -66 dBm/MHz which is in the similar range as regular spurious emission requirement levels for co-existence requirements.

Alternatively, define transmitter spurious emission requirement based on OTA parameter such as a maximum field strength level at a distance from the side of the aggressor base station. This approach would require RAN4 to establish an approach for how to translate the maximum allowed emission level at the victim to a field strength level.

Both approaches would create a requirement without dependence to the CLTA.

Proposal 1: In respect to transmitter spurious emission for BS type 1-O and bands in the upper region of FR1, consider one of following approaches:

- Define co-location transmitter spurious emission requirement based on TRP level considering relevant BS-to-BS coupling loss value.

- Define co-location transmitter spurious emission requirement based on field strength level considering co-location scenario.

    - Receiver out-of-band blocking

The requirement background comes from the background that multiple networks operating within different 3GPP bands can be co-located in the same geographical area. This means that the aggressor base station transmitter will generate an out-of-band interferer for the victim base station. For this purpose, 3GPP RAN4 defined a requirement for receiver out-of-band blocking requirement.

The receiver out-of-band blocking requirement interferer level was originally derived from calculating the interferer level at the victim receiver input assuming a certain BS-to-BS isolation value. Originally, a scenario with two equal base station sites close to each other was considered. The isolation was derived based on two fixed beams from passive BS antennas pointing towards each other. The main beam for each base station was directed towards the ground to achieve coverage.

Based on the BS-to-BS port coupling loss and few other assumptions the requirement interferer level at the victim base station can be established as shown in Figure 2.2.2-1.

Figure 2.2.2-1: Illustration of receiver out-of-band blocking requirement interferer level derivation

The blocking requirement is based on sensitivity, while an interferer signal is injected. For receiver out-of-band blocking the sensitivity shall not be degraded more than 6 dB when the out-of-band interferer signal is injected.

The requirement interferer level can be calculated in logarithmical scale as:

Pi=Ptx-CL1,		(Equation 2.2.2-1)

where Ptx is the aggressor base station transmit power and CL1 is the BS-to-BS port coupling loss in dB.

Observation 6: The requirement level for receiver out-of-band blocking interferer level currently defined in TS 38.104 does not depend on frequency and antenna aperture. The same requirement level is used for all operation bands within FR1. Hence, the current approach for deriving requirement levels does not allow for differentiation of requirement levels as function of frequency and antenna size.

Different reference points for out-of-band receiver blocking were considered:

- Receiver out-of-band blocking interferer level at the victim receiver input port, Pi .

- Receiver out-of-band blocking interferer level at the CLTA, Pi,CLTA.

In Figure 2.2.2-2, the aggressor BS and victim BS is visualised with different reference points. With the Rel-15 concept, the co-location reference antenna is used to mimic the aggressor BS. In the figure below a scenario considering co-location between AAS BS with AAS BS is considered since that is more relevant for higher frequencies within FR1.

# Figure 2.2.2-2: CLTA used to inject interferer signal

In Rel-15, the requirement level derived in TR 37.843 at the co-location reference antenna in conformance test specification TS 38.141-2 referred to as the CLTA was determined as:

Pi, CLTA=Pi+CL2		(Equation 2.2.2-2)

where CL2 is the isolation between the AAS BS and the passive BS antenna (used as CLTA).

In Rel-15 RAN4 decided to assume CL1 set equal to CL2 which was set equal to 30 dB. By combining Equation 2.2.2-1 and Equation 2.2.2-2 it can be noticed that the requirement level at the CLTA is independent of the BS-to-BS port coupling loss. Hence, the requirement level is completely defined by the aggressor BS output power level and the BS-to-BS port coupling loss is completely embedded in the considered co-location scenario.

Observation 7: With the Rel-15 co-location requirement concept the impact of the BS-to-BS port coupling loss cancel out in the out-of-band receiver blocking requirement level derivation. This would indicate that the out-of-band receiver blocking requirement level is not dependent on frequency and antenna size, which is not in line with simulation results and measurements.

Observation 8: To evolve BS co-location out-of-band receiver blocking requirement to properly support bands in the upper region of FR1 it is now time for RAN4 to develop an enhanced concept for out-of-band receiver blocking requirement to properly capture aspects related to frequency offset between aggressor and victim, asymmetry between aggressor array antenna size and victim array antenna size and aggressor and victim beamforming characteristics. The relevance of the current requirement concept is highly questionable, and testability is questionable.

If RAN4 consider changing the BS-to-BS port coupling loss assumption to be larger than 30 dB, RAN4 should also reconsider the requirement level derivation. As an example, if 50 dB coupling loss is assumed the interferer signal level can be recalculated to -4 dBm at the TAB connector. In addition, with knowledge of the victim BS array gain towards the towards the aggressor BS, the interferer EIS level can be calculated as:

EISi=Pi-Gv		(Equation 2.2.2-3)

where Gv is the victim array gain towards the aggressor base station (horizontal angle 90 degrees). The array antenna can be established from statistical evaluation of victim BS antenna gain.

With a requirement based on EIS, the requirement can be tested in regular fashion (e.g. in a CATR OTA test environment) as for reference sensitivity and in-band receiver blocking without need for the CLTA.

Proposal 2: In respect to receiver out-of-band blocking for BS type 1-O and bands in the upper region of FR1, consider defining the interferer signal based on EIS.

    - Transmitter OFF power

The requirement derivation for the transmitter OFF power requirement is similar to transmitter spurious emission, with few exceptions. Fundamentally, the transmitter OFF power requirement is not a requirement to guarantee co-location of two BS at the same site. The requirement was originally defined to guarantee that the aggressor BS don’t interfere with the next adjacent victim BS in the same network operating within the same band. This means that the coupling scenario is completely different compared to the one assumed for other co-location requirements.

For non-AAS BS, the difference was assumed to be low. Hence the same coupling loss was assumed as for all other co-location requirements. However, for AAS BS, it is expected that spatial selectivity due to beamforming would increase the coupling loss significantly.

Observation 9: For AAS the BS-to-BS port coupling loss assumed for the transmitter OFF power requirement level derivation should be established based on proper assumptions. The coupling loss should be established based on statistical analysis capturing spatial selectivity, while neighbouring BSs provide coverage to UEs dropped within each sector.

Based on the relevant coupling loss, the requirement level can be derived in the similar manner as for transmitter spurious emission from Equation 2.2.1-1.

As an example, if 50 dB coupling loss is assumed the spurious emission level Pem, can be calculated to -66 dBm/MHz which is in the same range as regular spurious emission requirement levels for co-existence requirements.

Since the transit OFF power requirement is tested together with transmitter transient period requirement it is not appropriate to define a TRP level for the transmitter OFF power level requirement. Instead, the requirement level needs to be expressed as an EIRP level. For this purpose, information about the aggressor BS antenna is needed.

The transmitter OFF power EIRP level can be calculated as:

EIRPem=Pem+Ga		(Equation 2.2.3-1)

The challenge is to determine if sub-array gain considering the OFF-leakage power is uncorrelated or full array gain considering the OFF leakage is correlated needs further discussion in RAN4.

Proposal 3: In respect to transmitter OFF power for BS type 1-O and bands in the upper region of FR1, consider defining the requirement based on EIRP.

    - Transmitter intermodulation

The transmitter intermodulation requirement was defined check that the transmitter fulfil the unwanted emissions requirements in the case where an interferer signal close to the carrier is present. The conformance test will stress the robustness of the transmitter design.

For non-AAS BS, the strongest interferer seen by a BS transmitter is coming from another co-located BS. Since the introduction of the requirement the coupling loss has been assumed to be 30 dB.

Now, considering OTA testing and AAS BS, it is not obvious whether the strongest interferer signal is coming from another BS or from intra leakage within the AAS BS itself.

For an AAS BS each transmitter will see an interferer signal from neighbouring element/sub-array due to intra-system mutual coupling effects. Typically, the coupling between elements/sub-arrays is in the range 10 to 20 dB for an AAS BS.

This would mean that if the BS can meet the unwanted emission levels without an external interferer signal, it would also meet the unwanted emission levels if the external interferer signal is significantly lower. Therefore, RAN4 should study the details behind the need for transmitter intermodulation requirement before deciding on how to proceed.

Proposal 4: In respect to transmitter intermodulation requirement for BS type 1-O and bands in the upper region of FR1, carefully investigate the technical background and relevance of the requirement and study test aspects with the intention to simplify OTA testing.

It is also worth to point out that transmitter intermodulation is regulatory requirement in some countries and regions.

  - Near-Field simulation methodology

Performing a co-location coupling analysis of two side-by-side large arrays is a formidable task that easily leads to memory overflow and long simulation times, especially if a complete coupling model is used in an Electromagnetic (EM) simulation tool. To overcome this, simplifications have been used such as using a simplified EM model of the antenna element and analytical circuit theory models of subarray networks. The overall idea is to choose these simplifications in a conservative way, i.e., to minimize the coupling loss. For example, there are no isolating structures on the sides of the arrays and no losses in the subarray models. Hence, in a real case the coupling loss should be larger compared to these simulations.

Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 2.3.4. Finally, array ports are obtained by applying beamforming weights to the subarray ports, see Section 2.3.6.

For the Near-Field simulation methodology presented in this contribution, we have split the simulations in two parts:

- In the first stage, the Element-to-Element coupling is represented by a scattering matrix simulated in an EM simulator, see Section 2.3.1. The victim and aggressor array antennas are placed according to decided co-location scenario side-by-side separated 0.1 m, as illustrated in Figure 2.3.2-1. The scattering matrix (S-matrix) includes all elements (both polarizations) of both arrays and contains element-to-element coupling loss and associated phase information. If needed, the S-matrix is calculated for several frequencies within the frequency range that is considered.

- In the second stage sub-arrays and array ports are created. Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 2.3.4, and array ports are obtained by applying beamforming weights to the subarray ports (and positions), see Section 2.3.6. For this stage a general-purpose linear algebra computation tool, e.g., MathWorks MATLAB or Python can be used.

A block diagram of the simulation methodology is shown in Figure 2.3-1.

Figure 2.3-1: Overview of the simulation methodology

    - S-matrix simulation

The antenna element scattering matrix is simulated using a commercial 3D EM field simulation software, in this case CST Studio Suite. The S-matrix contains data for all elements and polarizations in both arrays.

As an example, in the case of 3.5 GHz arrays, each of the arrays has 192 ports (12 rows x 8 columns x 2 polarization). This results in a 384x384 scattering matrix for each frequency point in the defined frequency range. For more details on geometry and element model, see below.

    - Geometry

For simulating the coupling between two arrays, two antenna arrays are placed next to each other (see Figure 2.3.2-1).

Figure 2.3.2-1: Two antenna arrays in an edge-to-edge co-location scenario

Each array comprises 12x8 dual-polarized radiating elements and a metallic reflector. The vertical element separation is 0.7λ and the horizontal element separation 0.5λ at specific design frequency (e.g., 3.5 GHz).

The distance between the two antennas (edge-to-edge) has been increased from 10 cm to 16 cm to take account of the mechanical support structures, which are present in a real product, and of space for the enclosure, which is also not included in the simulation.

    - Antenna element model

A simplified antenna element was used to get reasonable simulation time for the Element-to-Element S-matrix. Mechanical details, not relevant for the EM simulation was removed to speed up the simulation stage. These removed details include, e.g., as corners, mechanical support structures, logotypes, etc. As example of such the antenna element model used for simulation is shown in Figure 2.3.3-1. It consists of a thin crossed half-wavelength dipole which is fed in the centre by a coaxial port for each polarization (+/- 45°) and a metallic reflector at quarter-wave distance below the dipoles. There is no connection between the reflector and the dipole.

Figure 2.3.3-1: Perspective, top and side view of a simplified antenna element model

    - Sub-array modelling

The antenna elements are grouped in 3-element single column sub-arrays, as shown in Figure 2.3.4-1. A sub-array RDN, having 3 element ports and one feed port, is modelled with lossless ideal transmission lines and T-splitters, see Figure 2.3.5-1. To achieve equal power on the three elements, one of the T-splitters, S1, has an uneven power split ratio of 2:1, and the other, S2, has an even power split. Impedances of the quarter wave transformers on the output ports are chosen so that the desired power split ratio and impedance matching is achieved. The lengths of the lines L12, L13, L22 and L23 are chosen to achieve the desired fixed subarray tilt and long enough for the physical element spacing. The propagation constant of the transmission lines is chosen to equal that of free space for simplicity.

# Figure 2.3.4-1: Array and subarray layout

    - Sub-array ports S-matrix (cascading)

The S-matrices of the components in the antenna RF model are combined to an S-matrix describing the entire antenna including RDNs using well known circuit simulation theory, described, e.g., in [5], as visualised in Figure 2.3.5-1.

Z0, L13

Z0, L12

Z0, L23

Z0, L22

S1

S2

Figure 2.1.5-1: A 1:3 RDN is simulated by standard RF circuit techniques using S-matrix cascading

    - Deployment and beamforming weights

In the simulation it is assumed that two AAS base stations are co-located at the same site according to the enclosure edge-to-edge scenario defined in Rel-15 for BS type 1-O co-location requirements in TS 38.104, Clause 4.9. In this scenario it is assumed that both the aggressor AAS base station and the victim AAS base station are mounted side-by-side separated 0.1 m. The aggressor network and the victim network are configured in a hexagonal layout with 0% grid shift as visualised in Figure 2.3.6-1.

# Figure 2.3.6-1: Network layout

The victim and aggressor networks are co-sited. Narrow beams (similar to layer 1 codebook generated beams) are formed from the aggressor AAS to the UEs of the aggressor network and the corresponding power is received on the subarray ports of the co-sited AAS of the victim network. Only one cell is considered in the analysis.

Details related to deployment and BS assumptions is listed in Table 2.3.6-1, and Table 2.3.6-2.

Table 2.3.6-1: Network assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | Hexagon |
| ISD | 500 m |
| Network grid shift | 0 % |
| BS height | 25 m |
| UE height | 1.5 m |
| Number of UEs per sector | 1500 |
| Minimum 2D distance between BS and UE | 35 m |

Table 2.3.6-2: BS assumptions

| Parameter | 3500 MHz | 7000 MHz |
| --- | --- | --- |
| Sub-array configuration | 3x1 | 3x1 |
| Array configuration | 4x8 | 8x16 |
| Pre-set sub-array down-tilt | 3 degrees | 3 degrees |
| Mechanical down-tilt | 3 degrees | 3 degrees |
| Vertical element separation | 0.7l | 0.7l |
| Horizontal element separation | 0.5l | 0.5l |

For the evaluation in this contribution the physical size of the AAS BS antennas considered for 3500 MHz and 7000 MHz is equal. Considering other band combinations, such as lower bands, it is expected that the physical size will differ between aggressor and victim.

In the case of using randomly positioned UEs and A-to-A coupling loss, 1500 randomly distributed UEs per system are used. Then, at each time instance a pair of users are chosen, which are used to generate one sample of coupling loss, i.e., there will be a total of 1500 coupling loss samples. In Figure 2.3.6-2, an example of UE locations is visualised for the aggressor network and victim network.

In the case of A-to-SA coupling, only one system uses the above beamforming, and coupling is calculated between the array and the sub-array ports.

Figure 2.3.6-2: UE distribution

For an AAS BS array antenna operating outside the design frequency it is expected that the directivity will drop due to decorrelation effects due to out-of-band RF filter characteristics, transmission effects in the feeder network part of RDN and unwanted emission signal correlation.

For out-of-band beam weights a correlation model is used, corresponding to adding white noise to the antenna element weights. Here, 100% correlation (r equal to 1) corresponds to clean beam weights with no noise and 0% correlation (r equal to 0) corresponds to fully noisy beam weights. The effect of the beam pattern is that the directivity will decrease when the correlation decreases. In turn this leads to relatively more power being radiated sideways, which seems to minimize the coupling loss. Hence, 0% correlation could be used to derive conservative co-location requirements. The actual value of the correlation is unknown and is dependent on the specific implementation. Specifically, the digital predistortion will influence the out-of-band beam weights significantly.

Assuming a polynomial model, the beam weights for even harmonic frequencies equals the square of the beam weights of the carrier frequency, wc, while for odd harmonics the weights are equal to the beam weights of the carrier.

wnom=wcfor odd harmonicswc2/wc for even harmonics

where wnom, are the nominal beam weights.

All out-of-band beam weights, including the harmonics, are modelled as partly correlated with the correlation as a parameter. Partly correlated beam weights are modelled as:

w=ρ wnom+1-ρ wuncorr

where wuncorr are random weights:

wuncorr=x+jy2N

Here, x and y are normally distributed random variables with expected value 0, and N is the number of subarray ports.

# Conclusion

From the work done in Rel-19 we can conclude that the coupling loss assumed for requirement derivation need to be reconsidered for band in the upper region of FR1. It is essential to document and capture the outcome from Rel-19 to be able to continue the work in Rel-20 and include normative work with the intention to evolve the co-location requirements to better guaranteed robustness and repeatability. To be able to establish coupling loss assumptions for all co-location requirements, RAN4 need to establish a common simulation methodology. In this contribution we have a potential simulation methodology that could be used to evaluate coupling loss between two co-location base stations.

Considering relevant coupling losses established during Rel-19 from measurements and simulations, substantial impact on all co-location requirements have been observed. Instead of relying on a specified CLTA, regular OTA parameters can be adopted as indicated in Table 3-1.

Table 3-1: Overview of requirement evolution

| Requirement | Current requirement approach | Potential parameter for evolved requirement approach |
| --- | --- | --- |
| Transmitter spurious emission | CLTA | TRP, or alternatively field strength |
| Receiver out-of-band blocking | CLTA | EIS |
| Transmitter OFF power | CLTA | EIRP |
| Transmitter intermodulation | CLTA | FFS |

Defining requirement based on regular OTA parameters and assuming relevant coupling loss values would guarantee robust tower sharing and opens up for flexibility during conformance testing.

In this contribution we have observed the following:

Observation 1: It can be noticed that the coupling tables are not complete. RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements.

Observation 2: To establish relevant coupling assumptions for all requirements RAN4 need to establish a common simulation methodology and conduct a simulation campaign. It’s not possible to establish coupling vales based on measurements results, due to the large scope and different combination of bands and antenna configuration.

Observation 3: For transmitter spurious emission requirement, it can be noticed that the 9 dB additional noise would change the meaning of the original thought behind the requirement. With 9 dB additional noise, the impact on the victim receiver will be significantly larger than 0.5 dB degradation of the sensitivity.

Observation 4: With the Rel-15 co-location requirement concept the impact of the BS-to-BS port coupling loss cancel out in the transmitter spurious emission requirement level derivation. This would indicate that the transmitter spurious emission requirement level is not dependent on frequency and antenna size, which is not in line with simulation results and measurements.

Observation 5: To evolve BS co-location transmitter spurious emission requirement to properly support bands in the upper region of FR1 it is now time for RAN4 to develop an enhanced concept for transmitter spurious emission requirement to properly capture aspects related to frequency offset between aggressor and victim, asymmetry between aggressor array antenna size and victim array antenna size and aggressor and victim beamforming characteristics. The relevance of the current requirement concept is highly questionable, and testability is questionable.

Observation 6: The requirement level for receiver out-of-band blocking interferer level currently defined in TS 38.104 does not depend on frequency and antenna aperture. The same requirement level is used for all operation bands within FR1. Hence, the current approach for deriving requirement levels does not allow for differentiation of requirement levels as function of frequency and antenna size.

Observation 7: With the Rel-15 co-location requirement concept the impact of the BS-to-BS port coupling loss cancel out in the out-of-band receiver blocking requirement level derivation. This would indicate that the out-of-band receiver blocking requirement level is not dependent on frequency and antenna size, which is not in line with simulation results and measurements.

Observation 8: To evolve BS co-location out-of-band receiver blocking requirement to properly support bands in the upper region of FR1 it is now time for RAN4 to develop an enhanced concept for out-of-band receiver blocking requirement to properly capture aspects related to frequency offset between aggressor and victim, asymmetry between aggressor array antenna size and victim array antenna size and aggressor and victim beamforming characteristics. The relevance of the current requirement concept is highly questionable, and testability is questionable.

Observation 9: For AAS the BS-to-BS port coupling loss assumed for the transmitter OFF power requirement level derivation should be established based on proper assumptions. The coupling loss should be established based on statistical analysis capturing spatial selectivity, while neighbouring BS(s) provide coverage to UE(s) dropped within each sector.

Based on the observations we propose following:

Proposal 1: In respect to transmitter spurious emission for BS type 1-O and bands in the upper region of FR1, consider one of following approaches:

- Define co-location transmitter spurious emission requirement based on TRP level considering relevant BS-to-BS coupling loss value.

- Define co-location transmitter spurious emission requirement based on field strength level considering co-location scenario.

Proposal 2: In respect to receiver out-of-band blocking for BS type 1-O and bands in the upper region of FR1, consider defining the interferer signal based on EIS.

Proposal 3: In respect to transmitter OFF power for BS type 1-O and bands in the upper region of FR1, consider defining the requirement based on EIRP.

Proposal 4: In respect to transmitter intermodulation requirement for BS type 1-O and bands in the upper region of FR1, carefully investigate the technical background and relevance of the requirement and study test aspects with the intention to simplify OTA testing.

In a companion contribution [8], a draft CR to TR 37.941 with background information from the Rel-19 work have been created.

# References

[1]	R4-2415971, “On the topic of aspects related to BS-to-BS co-location”, Ericsson

[2] 	R4-2508657, “Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: CLTA”, Huawei

[3]	R4-2408086, “Evolution of BS co-location requirements”, Ericsson

[4]	R4-2412912, “On the topic of BS RF evolution related to co-location requirements”, Ericsson

[5]	I. A. Sadovskyy, “Reduction of the scattering matrix array”, Physics-Uspekhi, vol. 58, pp.872-876, 2015, DOI: 10.3367/UFNe.0185.201509c.0941

[6]	R4-2510233, “On alternatives to current CLTA concept”, Ericsson

[7]	R4-2510229, “Discussion on BS RF evolution general aspects”, Ericsson

[8]	R4-2510228, “Draft CR to TR 37.941: Addition of technical background for enhancement of co-location requirements in clause 6.4”, Ericsson