3GPP TSG-RAN Working Group 4 (Radio) Meeting #116	R4-2510228

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.941: Addition of technical background for enhancement of co-location requirements in clause 6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This draft CR is created based on structure agreed last meeting in R4-2508785 and findings from Rel-19 work. Technical background and frindings from the coupling loss evalaution (based on measurements and simulations) is collected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of technical background in clause 6.4.4.1. - Addition of measurement and simulation results in clause 6.4.4.2. - Addition of technical background for individual co-location requirements in clause 6.4.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the outcome of Rel-19 work will not be captured in technical report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Clause 6.4:

## 6.4	Co-location measurements

### 6.4.1	General

Co-location measurements are required for the OTA co-location requirements which are based on assuming the BS is co-located with another BS of the same base station class. They ensure that both co-located systems can operate with minimal degradation to each other.

Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the input or output of a co-location test antenna (CLTA).

CLTA is a practical passive antenna that is used for conformance testing of the co-location requirements and is based on the definition of the co-location reference antenna. Basic principle of the CLTA are outlines in clause 6.4.2. For more details on the co-location test antenna characteristics, refer to TS 37.145-2 [4] clause 4.15.2.2, or TS 38.141-2 [6] clause 4.12.2.2. The CLTA is suitable for testing BS implemented with a planar antenna array.

The alignment between the BS under test and the co-location test antenna is depicted in figure 6.4.1-1. For more details on the co-location test antenna alignment and test arrangement, refer to TS 37.145-2 [4] clause 4.15.2.3, or TS 38.141-2 [6] clause 4.12.2.3. The same physical alignment applies to in-band and out-of-band co-location requirements.

Co-location requirements are not applicable to FR2.

Figure 6.4.1-1: Alignment of BS and CLTA

### 6.4.2	Co-location test antenna

The co-location test antenna is a practical passive antenna based on the definition of the co-location reference antenna, and it is used for conformance testing of the co-location requirements. Thus the CLTA should be within a certain tolerance of the co-location reference antenna definition, so that commonly available passive BS antennas can be used to test the co-location requirements without having to use a large number of test antennas and without having to design specialized co-location test antennas.

For example, where the co-location reference antenna definition specifies that the co-location reference antenna, should be a single column passive antenna which has the same vertical radiating dimension (h) as the composite antenna of BS, this would be very restrictive when selecting a test antenna. Hence the CLTA should be within a certain tolerance for the vertical radiating dimension (h) such that a suitable practical BS antenna can be found for each BS.

The CLTA is hence defined as follows:

-	The CLTA is a single column passive antenna which has a vertical radiating dimension (h) which is equal to the vertical radiating dimension of the BS composite antenna ±30 %.

-	For in-band requirements, the CLTA supports the same frequency range and polarizations as the BS composite antenna.

-	For out-of-band co-location requirements, the half power vertical beamwidth of the CLTA equals the narrowest declared vertical beamwidth ±3 °.

-	The polarization should match the in-band CLTA polarization.

-	The CLTA has a horizontal half-power beamwidth of 65 ° ± 10 °.

-	The front faces of the BS and the CLTA need to be aligned in a common plane perpendicular to the mechanical bore-sight direction.

-	The centre of the vertical radiating regions of the CLTA and the BS composite antenna need to be aligned.

### 6.4.3	Standard test antenna

Co-location requirements may also be tested with a standardised test antenna such as a dipole. This has a number of advantages:

-	Standard test antennas are easily available and easier to specify.

-	Repeatability between different conformance testing runs will be greater.

-	Using standard test antennas, fewer antennas will be required for conformance testing.

-	A standard test antenna can have higher gain in the direction of the BS than a CLTA, so that signal levels can be higher relative to the noise floor in emission measurements and lower test levels can be used in interference measurements.

In order to test with a standard test antenna, the translation between measured power levels in the standard test antenna and the co-location reference antenna must be shown. The method for translating the power levels in the co-location reference antenna to the standard test antenna and vice versa is not covered by the present release of this technical report.

### Co-location measurements for higher frequency bands

#### Background on CLTA alternatives discussion

In Rel-19 BS RF evolution WI (NR_BS_RF_req_evo), the BS-to-BS port coupling loss for two different co-location scenarios have been evaluated through measurements and simulations.

- Vertical separation (Often used for tower sharing situations)

- Horizontal separation (side-by-side separated 0.1 m, according to TS 38.104, Clause 4.9)

#### BS-to-BS isolation analysis

The coupling loss between two located base station have been evaluated from measurement results and simulation results. The coupling loss have been evaluated for different situations, intra-band and inter-band considering the coupling loss as sub-Array-to-Sub-Array (SA-to-SA) coupling loss, Array-to-Sub-Array (A-to-SA) coupling loss and Array-to-Array (A-to-A) coupling loss. Coupling loss for different scenarios and types is lited in Table 6.4.4.2-1 and Table 6.4.4.2-2.

Table 6.4.4.2-1: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical separation | SA-to-SA | M: 54 | M: 53 | M: 56 |  |
|  | A-to-SA |  |  |  |  |
|  | A-to-A |  |  |  |  |
| Horizontal separation | SA-to-SA | M: 31 | M: 34 | M: 46 |  |
|  | A-to-SA |  | M: 35 S: 55±15 | M: 51 M: 69 | M: 55 S: 70±15 |
|  | A-to-A |  |  |  | S: 77±15 |

Table 6.4.4.2-2: Inter band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz to 3.5 GHz | 3.5 GHz to 4.9 GHz | 4.9 GHz to 7 GHz | 3.5 GHz to 7 GHz |
| Vertical separation | SA-to-SA |  |  |  |  |
|  | A-to-SA |  |  |  |  |
|  | A-to-A |  |  |  |  |
| Horizontal separation | SA-to-SA |  |  |  |  |
|  | A-to-SA |  |  |  | S: 57±10 |
|  | A-to-A |  |  |  | S: 54±10 |

The result in Table 6.4.4.2-1 and Table 6.4.4.2-2 consists of measurement results (M) and simulation results (S).

Even though the coupling loss evaluation is not complete for all relevant cases, the outcome clearly indicates that the assumption of a fixed BS-to-BS port coupling loss assumed to 30 dB for FR1 is not relevant anymore, especially not for AAS BS operating in the upper region of FR1. From the combined result from measurements and simulations it can be concluded that the coupling loss varies between 31 to 92 dB considering all scenarios, bands and coupling types. The general trend indicates that the coupling loss increases for bands in the upper region of FR1.

#### Impact on the co-location requirements

For MSR AAS BS and NR, co-location requirements are requirements which are based on a given BS-to-BS co-location scenario with a given assumption of port-to-port coupling loss of 30 dB. The intention with these requirements is to ensure that BSs serving different bands can be co-sited at the same site and/or tower. A set of co-location requirements have been defined for AAS BS in TS 37.105 and for NR BS type 1-C, BS type 1-H and BS type 1-O in TS 38.104. In Table 6.4.4.3-1, all the BS co-location requirements are listed together with references to individual requirements in TS 38.104.

Table 6.4.4.3-1: Overview of NR co-location requirements

| Requirement | Reference to clauses in TS 38.104 | Description |
| --- | --- | --- |
| Transmitter spurious emission | BS type 1-C and BS type 1-H in Clause 6.6.5.2.4. BS type 1-O in Clause 9.7.5.2.5. | This requirement is defined to protect BS operating in other 3GPP band(s) co-located at the same site. |
| Receiver out-of-band blocking | BS type 1-C and BS type 1-H in Clause 7.5.3. BS type 1-O in Clause 10.6.2.2. | This requirement is defined to guarantee UL operation when a BS is co-located to another BS operating at another 3GPP band(s). |
| Transmitter OFF power | BS type 1-C in Clause 6.4.1.2. BS type 1-H in Clause 6.4.1.3. BS type 1-O is Clause 9.5.2.2. | This requirement is defined to protect other BSs in the same network in neighbouring cells. |
| Transmitter intermodulation | BS type 1-C in Clause 6.7.2. BS type 1-H in Clause 6.7.3. BS type 1-O in Clause 9.8.2. | This requirement guarantees transmitter robustness with respect to an external interferer. |

Originally all requirements were derived for non-AAS BS (BS type 1-C). In Rel-13, hybrid AAS BS was introduced (In NR specifications referred to as BS type 1-H) and in Rel-15 all OTA AAS BS was introduced (In NR specifications referred to as BS type 1-O). For BS type 1-C and BS type 1-H, all requirements are defined at the transceiver ports (also known as the Transceiver Array Boundary (TAB) connector(s)). For BS type 1-O all co-location requirements are defined based on a concept of a co-location reference antenna. In conformance testing this antenna is referred to Co-Location Test Antenna (CLTA).

Looking at the details related to the technical background for each induvial requirement it can be noticed that the BS-to-BS coupling loss assumed is of different type depending on requirement. In Table 6.4.4.3-2 some further details for each requirement are presented.

Table 6.4.4.3-2: Requirement derivation assumptions

| Requirement | Coupling type | Frequency aspect | Description |
| --- | --- | --- | --- |
| Transmitter spurious emission | A-to-A | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. The approach used to derive the requirement level is defined to consider A-to-A coupling. |
| Receiver out-of-band blocking | A-to-SA | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. Since the receiver blocking related to induvial receiver branches, A-to-SA is considered when the requirement level is derived. |
| Transmitter OFF power | A-to-A | Intra band | The requirement is defined for TDD operation to protect the neighbouring BS in the same network. The approach used to derive the requirement level is defined to consider A-to-A coupling within the same band. The A-to-A coupling for this requirement should be based on to neighbouring BS and not side-by-side. |
| Transmitter intermodulation | A-to-SA | Intra band | This requirement is defined to guarantee than unwanted emission requirements is met in the case of an interferer signal. The interferer signal is injected within the band. Since the requirement is related to induvial transmitter branches, A-to-SA is considered when the requirement level is derived. |

It is worth to notice that the background for BS-to-BS coupling is different for all requirements listed in Table 6.4.4.3-2. A complete technical background for all requirements would require a coupling evaluation including the following cases:

- Intra band A-to-A for relevant band combinations

- Inter band A-to-SA for relevant band combinations

- Intra band A-to-A for relevant bands considering coupling to neighbouring BS

- Intra band A-to-SA for relevant bands

Analysing the results presented in Table 6.4.4.2-1 and Table 6.4.4.2-2 would indicate that assuming BS-to-BS port coupling loss equal to a value larger than 30 dB would be reasonable for bands in the upper region of FR1. To be able to decide on a relevant coupling value for all cases, RAN4 need to adopt a common simulation methodology.

#### CLTA alternatives

{Editors note: Information to be added}