3GPP TSG-RAN WG4 Meeting #116	R4-2510305

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1 on Blocking characteristics for LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction blocking requirements for LP-WUR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add blocking requirements, including IBB, OBB and NBB for LP-WUR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-WUR blocking requirements are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This draft CR is on top of the previously endorsed big draft CR R4-2506882 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of change>>

## 7.6M	Blocking characteristics for LP-WUR

### 7.6M.1	General

All blocking requirements specified in section 7.6.2, 7.6.3 and 7.6.4 shall apply for the UEs having the LP-WUR capabilities. Unless otherwise specified, the transmission bandwidth configuration in the Rx parameters for the blocking requirements mentioned in section 7.6.2, 7.6.3, and 7.6.4 shall refer to the transmission bandwidth configuration of UEs with LP-WUR capabilities as mentioned in section 5.3M.2. Furthermore, REFSENS shall refer to the REFSENS power levels for UEs with LP-WUR capabilities as mentioned in section 7.3M.

The MDR criterion as specified in clause 7.1M shall be met with the side conditions in clause 7.1M as the maimum throughput criterion does not apply for verification of LP-WUR requirements.

## <<End of change>>