3GPP TSG-RAN WG4 Meeting #116	R4-2510427

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on UE_BS demod performance requirements for 7 MHz CBW

Agenda item:	7.4.6

Document for:	Discussion

Introduction

In the last RAN4 meeting there were good progress. The remaining issues are to agree on the SNR values for PDSCH type A CA requirements with 7MHz as well as discussion related to release independence.

In this contribution we will give our view on the remaining issues and provide observations and proposals where relevant.

Discussion

PDSCH Requirements

Whether to define PDSCH CA requirements for 7MHz bandwidth

In RAN4#115 it was agreed to introduce CA requirements for PCSCH type A. In addition, it was agreed to not consider HST scenarios.

Whether to define PDSCH CA requirments for 7MHz bandwidth Agreements: - Define normal PDSCH type A CA requirements for 7MHz - Not to consider HST-SFN and HST-DPS scenario. - Interested companies are welcome to bring results for 7MHz. - FFS how to define requirements: - Option 1: Define new requirements for 7MHz CC based on the simulations - Option 2: Reuse existing 5MHz/10MHz requirements

It was left open how to define the actual requirements with the two options given above. Looking at the existing requirements in 38.101-4 [2] we see that there is no difference between requirements for 5MHz and 10MHz CBW, hence it is to be expected that the introduced 7MHz CBW requirements will be in the same SNR values.

Existing CA requirements for 5MHz and 10Mhz in TS38.101-4 Table 5.2A.2.1-1 and Table 5.2A.3.1-1 have the same SNR values. It can be expected that introducing 7MHz would not significantly differ from 5MHz/10MHz.

To evaluate if above observation is justified, we have simulation with the below agreed assumptions given in the WF from RAN4#115 [1].

Simulation assumptions for 7MHz CC for CA requirements Agreements: - Duplex mode: Only FDD - Bandwidth/SCS: 7MHz/15kHz - Number of RB for CORESET:30 - Number of RB: 35PRBs - MCS: 13 - Rank: Rank2 - MCS: 16QAM,0.48 - Propagation conditions: TDLA30-10 - Antenna configuration: 2T2R, 2T4R ULA Low - Other parameters: Reuse Table 5.2.1 in TS 38.101-4

Our simulations provide the following results:

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 7 | Given by simulation assumptions | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.2 |
| 7 | Given by simulation assumptions | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 5.1 |

Assuming addition of impairment and margin the difference between our simulation results and the existing 5/10MHz requirements will likely be less than 0.5dB

Our simulation results show that using simulated value for the new requirements will likely mean the 7MHz requirements will not be more than 0.5dB in difference to existing 5/10MHz requirements.

Since our 7MHz simulation results are so close to the existing requirements for 5/10MHz we see both options from the WF to be reasonable, however we would prefer to use new simulation results, if possible, for requirement definition. This of course will depend on the number of companies providing results in RAN4#116.

If alignment can be achieved with the simulations provided from companies in RAN4#115 then define the new 7MHz requirements based on the simulation results. In case alignment cannot be achieved, the existing requirements from 5/10MHz can be reused.

Release independence

In RAN4#115 the question of release independence was brought up with below proposal [1]:

Release independence for 7MHz bandwidth requirements - Proposals - Option 1: RAN4 to follow the discussion on release independent for support of 7MHz in RF session, once it is agreed, it should also apply to demodulation and CSI requirements.

In LS R4-2508088 [4] the following is stated:

RAN4 would like to clarify that we would like 7 MHz to be release independent from Rel-15 (up to RAN2 as to which version of the spec it appears with early implementation allowed), but it is acceptable if the signalling for 7 MHz as a minimum or maximum channel bandwidth for a band in a band combination is introduced in Rel-18 or Rel-19.

Based on this, we see it reasonable to also have 7MHz CBW performance requirements release independent from Rel.15.

In addition, looking in TS38.307 Table 5.1-1[3] it can be seen that when 3MHz was introduced it was release independent from Rel.15.

RAN4 has send LS to RAN2 requesting 7MHz CBW to be release independent from Rel.15.

Make the new requirements for 7MHz CBW release independent from Rel.15.

Conclusion

The following Observations and Proposals were made:

Observation 1: Existing CA requirements for 5MHz and 10Mhz in TS38.101-4 Table 5.2A.2.1-1 and Table 5.2A.3.1-1 have the same SNR values. It can be expected that introducing 7MHz would not significantly differ from 5MHz/10MHz.

Observation 2: Our simulation results show that using simulated value for the new requirements will likely mean the 7MHz requirements will not be more than 0.5dB in difference to existing 5/10MHz requirements.

Proposal 1: If alignment can be achieved with the simulations provided from companies in RAN4#115 then define the new 7MHz requirements based on the simulation results. In case alignment cannot be achieved, the existing requirements from 5/10MHz can be reused.

Observation 3: RAN4 has send LS to RAN2 requesting 7MHz CBW to be release independent from Rel.15.

Proposal 2: Make the new requirements for 7MHz CBW release independent from Rel.15.

- References

- R4-2508757 - WF on demodulation and CSI requirements for 7MHz

- 38.101-4 - 5G; NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (3GPP TS 38.101-4 version 18.8.0 Release 18)

- TS38.307 - 5G; NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band (3GPP TS 38.307 version 18.4.0 Release 18)

- R4-2508088 - Reply LS to RAN2 on Signalling for 7 MHz Channel Bandwidth