3GPP TSG-RAN WG4 Meeting #116	R4-2510678

# Bangalore, India, 25 Aug - 29 Aug 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of Air-to-ground network for NR

Agenda Item:	7.8.5

Document for:	Discussion

- Introduction

During last RAN4 meeting, good progress were achieved around ATG RRM, the following agreements around core part were approved and summarized in [1].

In this paper, we provide our further analysis on the remaining RRM aspects of R19 ATG.

- Discussion

Until last meeting, all RRM aspects have been fully discussed and almost all core part requirements have reach consensus. In this paper, we want to further discuss some details of the sharing factor between L1 measurement and RRM measurement, including both SSB based RRM measurement and CSI-RS based RRM measurement.

Sharing factor between L1 measurement and RRM measurement

In which case, Klayer1_measurement = 1 applies for, technically there are two cases:

- Case 1: there is not overlapping between L1 RS and {L3 RS+margin symbol}

- Case 2: partial overlapping between L1 RS and {L3 RS+margin symbol}

- On the contrary, Klayer1_measurement = 1.5 applies for the following case:

- Case 3: fully overlapping between L1 RS and {L3 RS+margin symbol}

- For both SSB based RRM measurement and CSI-RS based RRM measurement, the above analysis are applicable. One troublesome point is how to specify the “margin symbol”. In legacy SSB based RRM measurement in FR2, the margin symbol is decided as 1 symbol before and after the SSB symbols. This dues to the margin should cover MRTD(for CA/DC) and time error between the target cell of L1 measurement and the target cell(s) of RRM measurement. Normally the target cell of L1 measurement includes serving cell, and the target cell(s) pf RRM measurement includes both serving cell and neighbour cell. In legacy TN, for FR2, the time error between serving cell and neighbour cell could be deemed as cell phase error, so the length of margin roughly equals to MRTD(8us)+cell phase error(3us) = 11us, 1 symbol  is enough to cover the 11us margin, so the “margin symbol” in legacy is 1 symbol.

- While for ATG, how to calculate the “margin symbol”, we still start from the factors of MRTD(for CA/DC) and time error between serving cell and neighbour cell. One obvious difference between ATG and TN is ATG could be deployed in an extremely large ISD, i.e. 200km, 300km, so beside the MRTD and cell phase error, the propagation delay offset between serving cell to UE and neighbour cell to UE is another essential factor and it could be as much as 1ms, which implies 2 slots at 30kHz SCS. So the “margin symbol” should cover MRTD, cell phase error and propagation delay offset for ATG.

- Observation 1: Different from TN, the “margin symbol” should cover MRTD, cell phase error and propagation delay offset for ATG. The range of the “margin symbol” is quite wide to accommodate small ISD and large ISD deployments.

- So how to capture such a quite wide range of “margin symbol” into the spec, instead of saying L3 SSB symbols plus margin symbols, the SMTC occasion is used as a probable simplification. Which has been captured in the relevant chapter of the endorsed CR in last meeting.

- While for the CSI-RS based L3 measurement, after overview the legacy TN, actually no Klayer1_measurement used in FR2 CSI-RS based RRM measurement requirements, this is largely different from SSB based L3 measurement. The reason behind such difference lies in it was assumed that the L3 CSI-RS in MO could be configured as non-overlapping with L1 SSB/CSI-RS, so RAN4 decided no to introduce Klayer1_measurement for CSI-RS based L3 intra-frequency measurement in [2].

Observation 2: In legacy TN, no Klayer1_measurement is used in FR2 CSI-RS based RRM measurement requirements.

- Regarding to the CSI-RS based intra-frequency L3 measurement in ATG, one solution is to keep alignment with legacy TN, i.e. assume the L3 CSI-RS in MO could be configured as non-overlapping with L1 SSB/CSI-RS, so no Klayer1_measurement is needed for CSI-RS based L3 intra-frequency measurement. Another solution is still introduce the Klayer1_measurement and apply similar simplification as L3 SSB measurement, i.e. use the concept of CSI-RS window to define the overlapping/non-overlapping between L3 CSI-RS and L1 SSB/CSI-RS. Actually the CSI-RS window is clarified in Clause 9.10.2.2 of TS38.133 as “All CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms”. Between the two solutions, we prefer to the 2nd one since we concern NW could not guarantee non-overlapping between L3 CSI-RS and L1 SSB/CSI-RS afterall NW is unsure about the precise propagation delay offset.

- Regarding to the CSI-RS based inter-frequency L3 measurement in ATG, not need to consider Klayer1_measurement since the inter-frequency CSI-RS measurement is only performed within gap.

Proposal 1: For CSI-RS based intra-frequency L3 measurement in ATG, two solutions are listed:

- - Solution 1: Keep alignment with legacy TN, i.e. not to introduce Klayer1_measurement in CSI-RS based RRM measurement requirement for ATG(both single cell and CA).

- - Solution 2: Introduce Klayer1_measurement and use the concept of CSI-RS window(clarified in Clause 9.10.2.2 of TS 38.133) to distinguish the overlapping/non-overlapping between L3 CSI-RS and L1 SSB/CSI-RS.

- Solution 2 is preferred by us.

- Proposal 2: For CSI-RS based inter-frequency L3 measurement in ATG, no need to consider Klayer1_measurement since the inter-frequency CSI-RS measurement is only performed within gap.

- We noticed that the Klayer1_measurement was introduced in R18 ATG for single cell case, we provide one CR in in [3] to align with our proposals.

- For the relevant content in R19 ATG, if any consensus achieved, it could be updated to the draftCR.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

- Observation 1: Different from TN, the “margin symbol” should cover MRTD, cell phase error and propagation delay offset for ATG. The range of the “margin symbol” is quite wide to accommodate small ISD and large ISD deployments.

Observation 2: In legacy TN, no Klayer1_measurement is used in FR2 CSI-RS based RRM measurement requirements.

Proposal 1: For CSI-RS based intra-frequency L3 measurement in ATG, two solutions are listed:

- - Solution 1: Keep alignment with legacy TN, i.e. not to introduce Klayer1_measurement in CSI-RS based RRM measurement requirement for ATG(both single cell and CA).

- - Solution 2: Introduce Klayer1_measurement and use the concept of CSI-RS window(clarified in Clause 9.10.2.2 of TS 38.133) to distinguish the overlapping/non-overlapping between L3 CSI-RS and L1 SSB/CSI-RS.

- Solution 2 is preferred by us.

- Proposal 2: For CSI-RS based inter-frequency L3 measurement in ATG, no need to consider Klayer1_measurement since the inter-frequency CSI-RS measurement is only performed within gap.

- Reference

- R4-2508263, “WF on RRM requirements for NR_ATG_enh”, CMCC, 3GPP TSG-RAN WG4 Meeting #115.

- R4-2017223, “WF on remaining issues on CSI-RS based L3 measurement requirements (core part)”, Apple, 3GPP TSG-RAN WG4 Meeting #97e.

- R4-2510745 “(NR_ATG-Core) CR on CSI-RS based L3 measurements of ATG_R18”, ZTE Corporation, Sanechips, 3GPP TSG-RAN WG4 Meeting #116.