3GPP TSG-RAN WG4 Meeting #116	R4-2510865

# Bengaluru, IN, 25 - 29 Aug, 2025

Title: 	Discussion on NTN testing for NGSO demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.6.5.3

Document for:	Discussion

Background

During RAN4#115 meeting, Way Forward [1] for NTN_testing_NGSO_channel_model_and_demod was approved. In this contribution, we share our view on NTN testing for NGSO demodulation performance requirements.

Discussion

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

- agreement - Launch simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Considering the residual frequency error and timing error to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. - Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - residual frequency offset and time delay - option 1: Fix value - option 1a: frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s - option 1b: frequency error and timing error as 2Hz/s and 0.05us/s - option 2: time-varying propagation delay and Doppler shift. - FFS - Companies are encouraged to provide the simulation results during RAN4#116 meeting

Here we provider our simulation results considering 20% of the worst case (0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s) for frequency error and timing error as Following Table 2.1-1.

Table 2.1-1 simulation results considering 20% of the worst case for frequency error and timing error

| Case Number | SCS (kHz) / CBW (MHz) | Propagation condition | MCS | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | +Error |
| 1 | 15 / 10 | NTN-TDLA100-200 | 4 | 1x2 | -2.80 | -2.65 |
| 2 | 15 / 10 | NTN-TDLC5-200 | 13 | 1x2 | 4.86 | 5.24 |

Based on the simulation results, we propose to add 0.5 dB margin for NR NTN PDSCH requirements.

Add 0.5 dB margin for NR NTN PDSCH requirements.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Add 0.5 dB margin for NR NTN PDSCH requirements.

Reference

R4-2508625, Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod, RAN4#115, Samsung