3GPP TSG-RAN WG4 Meeting #116                                        R4-2511085

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:              Discussion on RRM requirements for LB CA switching

Agenda item:			7.3.4

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some initial discussions on RRM requirements for LB CA switching. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreement

#121 Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier Agreement Draft LS R1-2504868 is endorsed in principle. Agreement Final LS R1-2504869 is endorsed. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

3 Discussion

3.1 RRM requirement applicability for R19 LB CA via switching & Requirement design of activated SDL SCell L3 measurement

Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching - Proposal 1 (vivo): For defining RRM requirements for SDL SCell operation, it is assumed at least one SSB/TRS is available either during one switching pattern period or one SSB/TRS period depending on the periodicity of switching pattern and SSB/TRS. - Proposal 2 (Ericsson): RAN4 to further investigate the RRM requirements impact with at least one of the following methods. - Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures. - Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching. - Proposal 3 (ZTE): No need to consider the extension solution at least for L3 measurement, it can not work under current switching pattern periodicity P = 40 approved by RAN1. If one SMTC occasion is missed, then all SMTC occasions would be missed since typical SMTC configuration contains one or multiple complete switching pattern periodicity. - Proposal 4 (Nokia): For an activated SDL SCell with LBCA-sw, for a serving cell which can be either FDD PCell or SDL SCell: - a. If all the SSB occasions of the serving are contained within the LBCA-sw pattern, no delay extension is allowed - b. If a fraction the SSB occasions of the serving are contained within the LBCA-sw pattern, the delay is extended in accordance to the fraction of available SSB occasions - c. If no SSB occasion of the serving cell is contained within the LBCA-sw pattern, the requirements do no apply - Proposal 5 (MTK): - Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the other procedures/measurements.

Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

Considering the flexibility of NW scheduling, at least the cases maybe occur in conjunction with switching pattern design, below:

Fig. 1 Illustration on the possibility of multiple SMTCs across different carriers vs switching pattern with 0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2,

based on the following assumption

- The semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms

- The restriction of maximum X switch(es) within Y slot(s), support Y = 40 and X = 8

- UL transmit timing requirements are applicable provided that at least one SSB is available at the UE during the last 160 ms

As the above figure says, roughly speaking, the relationship between patterns (LB CA via switching pattern and SMTC pattern) and the corresponding level of availability of the measurement resources/occasions for either FDD PCell or activated SDL SCell can be categorized into:

- Case 1：All measurement occasions on PCell and SCell are available within the pattern

- Case 2：None of the measurement occasions on PCell/SCell are available within the pattern

  - Case 2-1：None of measurement occasions on SCell are available within the pattern

- Case 3：A fraction of the measurement occasions on PCell/SCell are not available within the pattern

  - Case 3-1：Only a fraction of the measurement occasions on SCell are not available within the patter

Particularly, from our understanding, the per-frequency carrier configured SMTC (offset and periodicity) would be set appropriately according to SS burst set transmission duration and periodicity for each frequency carrier so that UE measurement on each frequency carrier can be efficient. In this sense, under the consideration of accommodating the measurements on all cells (serving cells and neighbor cells), Case 3 occurs with a certain probability. And put restrictions on scheduling and limitations on SMTC are not expected from NW.

On top of such observations, from our understanding, the collision situations can be categorized into:

- Fully overlapping (Case 1), i.e., all the measurement occasions are available based on the LBCA switching pattern

- No delay extension is needed

- Non-overlapping (Case 2, Case 2-1), i.e., SMTC occasions in the given switching pattern are not candidates to measure

- The applicability of the requirement is not met

- Partially overlapping (Case 3, Case 3-1), i.e., a fraction of SMTC occasions are not available based on the LB-CA switching pattern

- Necessary scaling factor (per carrier) is needed to extend the delay requirement proportionately

Proposal 1:

For gap-less based measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

- If all the measurement occasions are available based on the LB-CA switching pattern, no delay extension is needed

- If a fraction of measurement occasions are not available based on the LB-CA switching pattern, the delay is scaled

- If no measurement occasions in the given LB-CA switching pattern are available to measure, the RRM requirement applicability for R19 LB CA via switching is not met,

Wherein, the measurement occasions can be SMTC occasions for L3 measurements or RS occasions for L1 measurement

There are two solutions can be considered to define the gap-less requirement, the two solutions are the same in principle, just from different perspectives

Assume: Pattern utilization repetition period (PURP)= P. After the PURP, the possible usage of each measurement occasion can to repeat.

- Solution 1: Using available measurement occasion based. Window= LCM (P, SMTC period, xRP_max) principle, here LCM is equivalent to max (∙),

the new scaling factor KLB definition is

KLB = Ntotal_LB / Navailable_LB,

Where,

Ntotal_LB: (..the existing limitation is omitted) and those are overlapped with PCell duration of the switching pattern within the window

Navailable_LB (..the existing limitation is omitted) and the number of measurement occasions are not overlapped with PCell duration of the switching pattern

Requirements do not apply due to fully non-overlapping between measurement occasions and SCell duration of the switching pattern occasions within the window W

Cons: If TSMTC>P (i.e., case 3-1 in our figure happens on SCell), KLB is irrelevant to switching pattern, the pattern cannot be represented in the computation

- Solution 2: Using non-available measurement occasion rate

To be more specific, KLB is to account for the delay when some measurement occasions be supposed to but cannot be used for measurements on SDL SCell

- Measurement behaviour assumption:

  - For our n5/n29 LB CA switching case: The number of carriers can be measured during each PCell/SDL SCell duration of the switching pattern is 1, aligning to the RAN1 agreement and WID

  - During each SDL SCell duration of the switching pattern, if measurement resources exist, the available measurement occasions should be sufficient to guarantee that UE shall attempt to make use of the measurement occasions to perform robust L3 measurement/L1 measurement/AGC

- Simple Computation

If TSMTC for the SDL SCell =< LLB: scaling factor KLB=P-LLBTSMTC×TSMTCTSMTC

If TSMTC for the SDL SCell > LLB: scaling factor KLB=P-LLBTSMTC+1×TSMTCTSMTC, where LLB: is the size of the bitmap b1, where b1: the size of value '1' in the bitmap configured, following RAN1 agreement: ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2

Note: this scaling factor cannot perfectly match the case that part of the measurement resources (i.e., part of SSBs in the SMTC window) are punctured by pattern switching

Proposal 2:

(SMTC based measurement) For gap-less based measurement of a serving cell which can be either FDD PCell or activated SDL SCell, new scaling factor can be used for defining requirement, two alternative solutions can be considered:

- Alt. 1 Using available measurement occasion based. Window= max (P, SMTC period, xRP_max) principle，KLB = Ntotal_LB / Navailable_LB

- Alt. 2 Using non-available measurement occasion rate. KLB is to account for the delay when some measurement occasions be supposed to but cannot be used for measurements on SDL SCell

Proposal 3: For KLB = Ntotal_LB / Navailable_LB solution

- Ntotal_LB: (..the existing limitation is omitted) and those are overlapped with PCell duration of the switching pattern within the window

- Navailable_LB (..the existing limitation is omitted) and the number of measurement occasions are not overlapped with PCell duration of the switching pattern

- Cons for the solution: If TSMTC>P, KLB is irrelevant to switching pattern, the pattern cannot be represented in the computation

Proposal 4: For using non-available measurement occasion rate solution, the new scaling factor KLB can be simply defined as the following:

- If TSMTC for the SDL SCell =< LLB: scaling factor KLB=P-LLBTSMTC×TSMTCTSMTC

- If TSMTC for the SDL SCell > LLB: scaling factor KLB=P-LLBTSMTC+1×TSMTCTSMTC,

  - where LLB is the size of the bitmap b1, where b1: the size of value '1' in the bitmap configured, TSMTC is the SMTC period

3.2 Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered

The RF agreement achieved in RAN4 # 114 meeting explicitly indicated: -	The combination of those two low bands with other band can be further discussed in the future release. Besides, according to the feedback from the operator and the WID objective confirmation, for the serving carrier BC in this WI in Rel-19, only 2 CCs are considered. In this sense, we can agree that

Proposal 5: For the necessity of interruption requirement in RRM when more carriers are considered, RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

3.3 Requirement design of deactivated SDL SCell measurement and corresponding interruption

Issue 1-3-3: Necessity of deactivated SDL SCell measurement and corresponding interruption requirement Online agreement on Tuesday: - RAN4 to define the requirement for deactivated SDL SCell L3 measurement. Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

Interruption rate:

The interruption on the PCC is unavoidable when deactivated SDL SCell is measured based on the consideration of single Rx chain implementation. UE cannot receive or transmit during the interruption, which will result in packet loss, the packet loss refers to DL data lose, UL grants and ACK/NACK for UL transmission loss, etc.. The amount of data loss depends on interruption rate and interruption length. Besides, in the legacy, the corresponding single loss requirement: up to 0.5% probability of missed ACK/NACK is defined for configured measurement cycles (measCycleSCell) equal to or more than 640 ms.

However, such requirement is not applicable to LB CA switching, another X%(s) should be defined to evaluate packet loss probability under different SCC measurement cycle (ms) ranging from 160 ms. Another reason to define different X% is that, the preferred solution in the legacy: to ensure 0.5% average packet drop rate by performing less frequent of retuning, doesn’t work anymore

Proposal 6: For deactivated SDL SCell measurement, RAN4 to define X% interruption rate, X can be different for different SDL SCC measurement cycle (i.e., configured measCycleSCell)

Based on RF discussion, RF tuning/retuning is needed for LB CA switching, otherwise, e.g., potential image issue may arise, which will incur DL performance degradation.

In this sense, the interruption rate defined as the following is more reasonable:

X=2×(RF tuning+RF retuning+Measurement window duration)Measurement cycle=2×(1+TSMTC_duration)measCycleSCell, assuming RF retuning time is 0.5 ms

Besides, from our understanding, PDCCH should be protected, otherwise, the self-scheduling and/or cross-carrier scheduling cannot be supported. And based on the NW implementation and proper configuration, the search space can to be placed appropriately to avoid the collision

Proposal 7: For deactivated SDL SCell measurement, the X% probability of missed ACK/NACK can be defined as: X=YmeasCycleSCell, where Y=2×1+TSMTC_duration, assuming RF retuning time is 0.5 ms, because both scheduled PDSCH and ACK/NACK transmission is interrupted

3.4 Issue 1-4-1: Impact on timing requirement for PCell

Regarding impact on FDD PCell, from our understanding, PCell should be always active, measurement results of PCell should always be available, and the loss of some measurement resources is typically not fatal to the PCell connection

Observation 1: Regarding impact on FDD PCell, PCell should be always active, and the loss of some measurement resources is typically not fatal to the PCell connection

L1-RSRP

For L1-RSRP measurement, the evaluation period is based on the measurements of M samples depending on timeRestrictionForChannelMeasurement configuration. NW then can use the reported measurement result containing channel quality information to perform beam management, in this sense, it is important for UE to provide accurate measurement. If scaling factor based mechanism can be accepted in activated SDL SCell measurement issue, from our understanding, it can be deemed a generic solution leveraged on the case that available samples are unavoidably “missing” on PCell due to LB CA switching. Based on such, UE is able to 1) have desirable resources to measure; 2) monitor sufficient number of SSB and/or CSI-RS resource. The satisfied L1-RSRP accuracy requirements can be guaranteed by the means of extending the measurement period based on the scaling factor eventually.

Specifically, the KLB in our definition is related to {b0}, if it is used to perform PCell measurement

Proposal 8:To perform L1-RSRP measurement of FDD PCell in LB CA via switching, the measurement period of PCell is to extend in the case of a fraction of the SSB/CSI-RS measurement occasions on PCell are not available within the pattern

- The handling rules of scaling factor defined for SDL SCell can be applied to PCell

3.5 Scheduling restriction during SDL SCell activation/deactivation

Distinct from the interruption on deactivated SCell, for which the measurement occasion is not explicitly specified and up to UE implementation, and the interruption is allowed before and after every SMTC on deactivated SCells, but the total amount of interruptions is restricted with missing ACK/NACK rate, for SCell activation/deactivation, after MAC-CE SCell activation command reception and processing, the first possible/complete SSB burst in time domain can be determined. In this sense, scheduling instead of interruption is acceptable during SDL SCell activation/deactivation. Regarding the scheduling restriction length, inspiration from switching gap location (ends at the end of the last slot) of pattern switching on activated SCell, we prefer to inherit the same timeline, i.e.,

The length of scheduling restriction= 1 slot + Tmeasurement_duration+ 1 slot, otherwise additional work load to discuss how many symbols to be used for switching from FDD-SDL and SDL-FDD will be introduced.

Proposal 9: RAN4 to define the scheduling restriction during SDL SCell activation procedure

- The length of scheduling restriction is defined as 1 slot + TSMTC/SSB_duration+ 1 slot

3.6 Reporting related issue

- RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. (Apple)

From our understanding, in theory, the collision should not be expected. Since in LB CA switching case, UL resources are only available on PCell, if the CQI/L1-RSRP reporting occasions are collided with the switching pattern, it is highly possible that the critical and valid CSI information reflecting the channel characteristics cannot be known at NW side timely and successfully. From this, unexpected DL performance degradation or even link failure event would be occurred. At the same time, since the reporting parameters can be also configured by RRC, a clear NW can arrange the two configurations in an appropriate way.

Proposal 10: Prefer not to consider the case that CQI/L1-RSRP reporting is collided with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation in LB CA switching

4 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for LB CA switching. The following observations and proposals are obtained:

Observation 1: Regarding impact on FDD PCell, PCell should be always active, and the loss of some measurement resources is typically not fatal to the PCell connection

Proposal 1:

For gap-less based measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

- If all the measurement occasions are available based on the LB-CA switching pattern, no delay extension is needed

- If a fraction of measurement occasions are not available based on the LB-CA switching pattern, the delay is scaled

- If no measurement occasions in the given LB-CA switching pattern are available to measure, the RRM requirement applicability for R19 LB CA via switching is not met,

- Wherein, the measurement occasions can be SMTC occasions for L3 measurements or RS occasions for L1 measurement

Proposal 2:

(SMTC based measurement) For gap-less based measurement of a serving cell which can be either FDD PCell or activated SDL SCell, new scaling factor can be used for defining requirement, two alternative solutions can be considered:

- Alt. 1 Using available measurement occasion based. Window= max (P, SMTC period, xRP_max) principle，KLB = Ntotal_LB / Navailable_LB

- Alt. 2 Using non-available measurement occasion rate. KLB is to account for the delay when some measurement occasions be supposed to but cannot be used for measurements on SDL SCell

Proposal 3: For KLB = Ntotal_LB / Navailable_LB solution

- Ntotal_LB: (..the existing limitation is omitted) and those are overlapped with PCell duration of the switching pattern within the window

- Navailable_LB (..the existing limitation is omitted) and the number of measurement occasions are not overlapped with PCell duration of the switching pattern

- Cons for the solution: If TSMTC>P, KLB is irrelevant to switching pattern, the pattern cannot be represented in the computation

Proposal 4: For using non-available measurement occasion rate solution, the new scaling factor KLB can be simply defined as the following:

- If TSMTC for the SDL SCell =< LLB: scaling factor KLB=P-LLBTSMTC×TSMTCTSMTC

- If TSMTC for the SDL SCell > LLB: scaling factor KLB=P-LLBTSMTC+1×TSMTCTSMTC,

where LLB is the size of the bitmap b1, where b1: the size of value '1' in the bitmap configured, TSMTC is the SMTC period

Proposal 5: For the necessity of interruption requirement in RRM when more carriers are considered, RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

Proposal 6: For deactivated SDL SCell measurement, RAN4 to define X% interruption rate, X can be different for different SDL SCC measurement cycle (i.e., configured measCycleSCell)

Proposal 7: For deactivated SDL SCell measurement, the X% probability of missed ACK/NACK can be defined as: X=YmeasCycleSCell, where Y=2×1+TSMTC_duration, assuming RF retuning time is 0.5 ms, because both scheduled PDSCH and ACK/NACK transmission is interrupted

Proposal 8:To perform L1-RSRP measurement of FDD PCell in LB CA via switching, the measurement period of PCell is to extend in the case of a fraction of the SSB/CSI-RS measurement occasions on PCell are not available within the pattern

- The handling rules of scaling factor defined for SDL SCell can be applied to PCell

Proposal 9: RAN4 to define the scheduling restriction during SDL SCell activation procedure

- The length of scheduling restriction is defined as 1 slot + TSMTC/SSB_duration+ 1 slot

Proposal 10: Prefer not to consider the case that CQI/L1-RSRP reporting is collided with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation in LB CA switching

5 References

[1] R4-2508259, “WF on RRM requirements for NR_LBCA_Sw”, Apple