3GPP TSG-RAN WG4 Meeting #116 	R4-2511153

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.26.4

Source: 	Ericsson

Title:	On RRM performance for XR

Document for:	Approval

# Introduction

The WI on WID on XR (eXtended Reality) for NR Phase 3 includes both core and performance parts [1]. While the core part is being finalized, the performance issues have not been yet discussed much, which is the scope of this contribution.

# Discussion

## Measurement accuracy requirements for UE under XR operation

In RAN4#113, during the core requirements discussions for XR, RAN4 has made an agreement [6] related also to measurement performance requirements for UE under XR operation:

- XR skipping requirements impact

  - Reuse existing measurement accuracy requirements.

  - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping

    - For the delay requirements, FFS whether and how to extend the delay

Since later discussions did not impact the above agreement on the accuracy requirements, it is proposed that RAN4 uses this agreement also in its work in the performance part of the WI.

- Proposal 1:

  - RAN4 confirms the earlier agreement:

    - Reuse existing measurement accuracy requirements for UE under XR operation.

## RRM test cases for UE under XR operation

According to the RAN4 agreements and the agreed work split [9], the following requirements are being defined for XR in Rel-19:

| 9.2.6 | Intra-freq measurements with gaps |
| --- | --- |
| 9.3.4 | Inter-freq measurements with gaps |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements |
| 9.15.5 | Inter-frequency L1-RSRP with MG |

It is also worth noting that the new requirements are defined for both FR1 and FR2-1, with and without DRX.

Furthermore, RAN4 agreed that the new requirements are specified for Type 1 and Type 2 MGs, per-UE gaps and per-FR gaps.

Defining test cases for all combinations of the above would result in a big set of test cases.

- Proposal 2: A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation.

- Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both.

- Proposal 4: Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG.

- Proposal 5: RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19.

- Proposal 6: Test cases are specified only for per-UE MGs.

Even if there will be the same accuracy requirements with XR as in legacy, there is still a need to verify whether accuracy can be met in the new XR scenarios since the measurement period is defined differently for XR and the time between samples can become very non-uniform.

- Proposal 7: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

# Summary

- Proposal 1:

  - RAN4 confirms the earlier agreement:

    - Reuse existing measurement accuracy requirements for UE under XR operation.

- Proposal 2: A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation.

- Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both.

- Proposal 4: Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG.

- Proposal 5: RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19.

- Proposal 6: Test cases are specified only for per-UE MGs.

- Proposal 7: Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, Aug./May 2024.

[3] R4-2414910/ R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1 LS, Oct./Aug. 2024.

[4] R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, Aug. 2024.

[5] R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, Oct. 2024.

[6] R4-2420103, WF on RRM requirements for XR_Ph3, Qualcomm, Nov. 2024.

[7] R4-2502700, WF on RRM requirements for XR_Ph3, Nokia, Feb. 2025.

[8] R4-2503921, draftCR to 38.133, Inter-frequency measurement requirements with XR, Ericsson, Apr. 2025.

[9] R4-2504911, WF on RRM requirements for XR_Ph3, Nokia, Apr. 2025.

[10] R4-2508285, WF on RRM requirements for NR_XR_Ph3, Nokia, May 2025.