3GPP TSG-RAN WG4 Meeting #116	R4-2511232

Bangalore, India, 25th – 29th August 2025

Agenda Item:	7.1.4

Source:	Ericsson

Title:	On UE demodulation and CSI reporting requirements for 6Rx

Document for:	Discussion

# 1	Introduction

Following the agreements made in the RF session below, the following UE demodulation and CSI reporting requirements will be defined but applied to handheld UE and FWA devices separately.

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

After the discussion in RAN4 #115 meeting in Malta, companies have reached several agreements regarding the test scenarios and some parameter assumptions.

In this paper, we will present more views on the aspects of UE demodulation and CSI reporting requirements in order to facilitate the parameter assumptions.

# 2	Discussion

During the previous meeting, the following three general issues were raised by multiple companies, indicating that these topics are of broad concern and merit careful consideration and discussion going forward:

- Issue 1-1: Test applicability rule for 6Rx tests [1]

- Issue 1-2: Test applicability rule for different numbers of Rx antenna ports [1]

- Issue 1-3: Specification structure [1]

Although our position remains that introducing complex test applicability rules at this stage may be premature, we recognize the relevance of these topics. In particular, we believe that Issue 1-1 and Issue 1-2, both of which relate to the design of test applicability rules, should be put on hold until the relevant test cases are better defined and agreed upon. This approach will help avoid unnecessary revisions and ensure consistency between the applicability framework and the finalized test scenarios.

Regarding Issue 1-3, which concerns the structure of the specification, we consider this to be an editorial matter. As such, we believe that any necessary changes to the structure can be addressed directly during the Change Request (CR) process at a later stage, without requiring further discussion at this point.

In summary, we propose:

- To defer further technical discussion on Issue 1-1 and Issue 1-2 until test case definitions are sufficiently mature;

- To handle Issue 1-3 through the regular CR process, where the structural proposals can be reviewed and discussed in context.

We remain open to continued dialogue on these matters in future meetings.

Proposal 1: Postpone the discussion of test applicability rule until the relevant test cases are better defined

Proposal 2: Postpone the discussion on the specification structure

## 2.1	PDSCH demodulation requirements

RAN4 has made the following agreements for PDSCH requirements so far:

Issue 2-1-1: Test scenarios for PDSCH performance requirements - Agreement - Test scenarios without interference: PDSCH Mapping Type A in Rel-19 Issue 2-2-2: Propagation channel for 4 MIMO layers - Agreement - TDLA30-10 for both FWA and Handheld UE Issue 2-2-4: Antenna configuration for 4 MIMO layers case - Agreement - 4Tx6Rx Issue 2-2-6: MCS for 4 MIMO layer case for Handheld UE with ULA medium (If agreed) - Agreement - MCS 13 (Table 1) Issue 2-3-2: Propagation channel and MIMO correlation for 6 MIMO layers - Agreement - TDLA 30-10 ULA Low Issue 2-3-4: Whether to consider 256QAM for 6 MIMO layers case - Agreement - Not consider 256QAM Issue 2-3-5: MCS for 6 MIMO layers case - Agreement - MCS17 (Table 1)

Apart from the agreements above, there are still plenty of undetermined parameter assumptions that need to be discussed.

Regarding the MIMO correlation, companies first agreed on the correlation matrix for 6Rx as follows:

Issue 2-2-1: MIMO correlation for 4 MIMO layers for Handheld UE testing - Agreement - Introduce correlation matrix for 6Rx in the specification: RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β1625*β925*β425*β125*1

As for the exact value of α and β, however, several proposals have been raised.

- Proposal - Option 1: with α = 0 and β = 0.07179 - Option 2: with α = 0.3 and β = 0.07179 - Option 3: with α = 0 and β = 0

As background, it is noted that in the spec. TS38.101-4 it has been already defined that the ULA correlation models are as follows:

Table 2.1-1 The α and β parameters for ULA MIMO correlation matrices

| Correlation Model |  |  |
| --- | --- | --- |
| Low correlation | 0 | 0 |
| Medium Correlation | 0.3 | 0.9 |
| Medium Correlation A | 0.3 | 0.3874 |
| Medium Correlation B | 0.3 | 0.005154 |
| High Correlation | 0.9 | 0.9 |

Based on this, a re-examination of the three options under consideration reveals that, taking Medium Correlation B—the medium correlation model most commonly used in those requirements defined in the spec.(TS38.101-4)—as a reference, option 2 exhibits a correlation level higher than Medium B, option 3 corresponds to the Low correlation model, while option 1, being lower than option 2, can be regarded as maintaining a relatively balanced correlation level.

Furthermore, as evidenced by our simulation results, summarized in the table below, we evaluated two distinct cases in which the only difference lies in the correlation setting, with all other conditions held constant. This setup allows for a direct performance comparison between option 1 and option 2.

Table 2.1-2 Simulation for comparing different correlations

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 4 | 10 / 15 | MCS 17 (Table 1) | TDLA30-10 | 4x6, α = 0 and β = 0.07179 | 70 | 22 |
| 2 | 4 | 10/15 | MCS 17 (Table 1) | TDLA30-10 | 4x6, α = 0.3 and β = 0.07179 | 70 | 29.5 |

From the simulation results above, it can be observed that the performance under the correlation configured with option 1 falls within a more reasonable SNR range. In contrast, the performance of option 2 is less favorable, with excessively high SNR values, making it unsuitable for defining requirements. The fact that option 3 was not simulated should not be interpreted as an intention to exclude it; rather, it reflects our confidence that, under this correlation setting, the UE would operate properly and the resulting SNR would remain at a healthy level.

Based on the above, we can conclude the following observations:

Observation 1: From the simulation perspective, performance with option 1 and option 2 are all with high SNR points

Therefore, we propose to consider the existing correlation medium B defined in the spec. for defining requirements for the case with 4 MIMO layers for Handheld UE. In particular, with respect to option 1 and 2, its consideration should be proved by potential evidence that may be provided to demonstrate whether the UE, when receiving a transmission of 4 MIMO layers using 6 receive antennas, would experience a low or high level of correlation.

Proposal 3: Consider existing medium B correlation until RAN4 has received good justification on newly proposed values.

Regarding whether one or two test cases should be defined, we consider that it depends on whether the two potential tests differ in terms of parameter settings, such as antenna correlation, MCS, or channel model. Among these factors, antenna correlation is considered the most critical. Therefore, this topic is related to Issue 2-2-1.

RAN4 has already reached the conclusion that for the 4-MIMO-layer test of FWA devices, the correlation will be assumed to be ULA low, which is exactly Option 3 in Issue 2-2-1. In that case, the 4-MIMO-layer tests for handheld UEs and FWA devices would be aligned in terms of the correlation parameter. Thus, defining a single test case to cover both UE types would be justified. On the other hand, if the correlation settings differ, then two separate test cases would be necessary.

Since MIMO correlation is strongly implementation-dependent, RAN4 would need to receive justification or even measurement data to determine whether the assumption should be low or high, and this justification should be widely agreed among companies. At present, only proposals from some companies have been received, but without detailed justification or indication that the proposals are based on measurement data.

Therefore, we consider that the decision on whether to define one or two test cases for 4-MIMO-layer depends on the conclusion of Issue 2-2-1.

Proposal 4: Postpone the discussion of the number of test cases for 4 MIMO layer scenario until there is an agreement for issue 2-2-1

As for the MCS for 4 MIMO layer cases for FWA/Handheld UE with TDLA30-10 ULA Low, following two options were proposed in the last meeting:

Issue 2-2-5: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low - Agreement: Down select the following two options in the next RAN4 meeting - Option 1: MCS 20 (Table 2) - Option 2: MCS 17 (Table 1)

According to our simulation results, it’s feasible to consider both MCS values as they are all within a reasonable range.

Table 2.1-3 Simulation for comparing different MCS values

| Test num. | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 4 | 10 / 15 | MCS 17 (Table 1) | TDLA30-10 | 4x6, ULA low | 70 | 11.5 |
| 2 | 4 | 10 / 15 | MCS 20 (Table 2) | TDLA30-10 | 4x6, ULA low | 70 | 13.5 |

Following the guidance of the agreement, we propose to down-select to option 2: MCS17: 64QAM as it’s widely used in defining performance requirements in the spec. (TS38.101-4), while MCS20(table 2): 256QAM is too high to be selected together with 4 MIMO layers.

Proposal 5: Consider MCS17(table 1): 64QAM for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

After the discussion in RAN4 #115 meeting in Malta, it’s rather clear what to be assumed for the cases of 6 MIMO layers for FWA. Many of the simulation assumptions have already reached agreement. However, two issues remain unresolved. The first concerns the test applicability rule for FWA. Although only a single proposal has been put forward thus far, our position remains consistent—namely, to deprioritize the discussion on test applicability until RAN4 has established a clear case list for either conducting simulations or more importantly, for defining requirements.

Proposal 6: Postpone the discussion of test applicability rule for FWA

The other issue concerns how to assume/configure the DMRS. To date, two options have been proposed: in addition to the commonly used Rel-15 Type 1 with DMRS 2+2, there is clearly also a preference in some quarters for selecting Rel-18 eType I DMRS 1+1. While we acknowledge that the latter may offer certain advantages over the former under specific conditions—such as lower overhead—we continue to hold the view that binding an optional feature to the testing of 6 MIMO layers would, quite evidently, narrow the test coverage.

From a technical standpoint, we are willing to consider the Rel-18 eType I DMRS configuration, provided it can be included as an additional case. In other words, if there is to be only one test case for 6 MIMO layers for FWA, then we propose to choose option 1. However, if two test cases can be broadly agreed upon, we would of course support—and be pleased to see—the Rel-18 eType I DMRS included as one of them.

Proposal 7: Consider Rel-15 Type 1 DMRS 2+2 if there will be only one case for 6 MIMO layer for FWA test

## 2.2	CSI reporting requirements

Similar to the case of Rel-18 8Rx, it is necessary to define CQI reporting requirements for both handheld UEs and FWA UEs equipped with 6Rx. For handheld UEs and FWAs that support up to 6Rx only, the existing 8Rx CQI reporting requirements do not provide adequate coverage. Accordingly, the following key assumptions are proposed:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4T6R with static channel specified in Annex B.1

Except for the above, it’s preferred in the meantime to reuse other parameter assumptions for 8Rx CSI reporting requirements.

In general, we propose to define one case and apply it to both handheld UEs and FWA UEs, which means there is no need to define additional applicability rules for 6Rx CQI reporting requirements.

Proposal 8: Define CQI reporting requirements for 6Rx reception

Proposal 9: Consider following parameter assumptions:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 10: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rule.

## 2.3	SDR requirements

The purpose of the SDR requirements is to verify that Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. It is considered essential to define the SDR requirement for the case of 6-receiver reception. The items that currently require discussion and decision are as follows:

- To determine a static channel matrix for the 8T6R case.

- To decide whether to define SDR requirements for 4 MIMO layers.

With regard to the first issue, two options have been proposed, differing in whether the given matrix should be multiplied by the factor √3/4.

Issue 3-1-1: Static channel and precoding for 6Rx … - For 8T6R (Fixed precoding with i1,1=i1,2=i2=0): - Option 1: - H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11 - Option 2: - H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

In our view, it is necessary to normalize the power of the channel matrix so that the SDR test results are consistent with other antenna configurations (such as 4T4R or 6T6R). Together with the purpose to compensate for the imbalance caused by having more Tx antennas than Rx antennas, ensuring that after layer-to-Rx mapping (which forms a diagonal matrix), the total received power per layer is not unintentionally increased. Thus, we propose to consider option 2 as for the static channel matrix for 8T6R case.

Proposal 11: Consider option 2 for the static channel matrix for 8T6R case

Regarding whether to consider 4 MIMO layers with 1024QAM, we choose to objectively inspect the simulation results of it so that to discuss the feasibility of introducing such requirement. Based on our initial simulation results, it’s been observed that the necessary SNR for achieving 85% throughput is 28.9dB, which is to be seen as feasible to establish requirements.

Figure 2.3-1 PDSCH performance with 1024QAM + 4 layers

Observation 2: It’s feasible from the simulation to define 1024QAM 4-layer SDR requirements

Thus, we propose to define SDR requirements for 1024QAM with 4 layers.

Proposal 12: Define SDR requirements for 1024QAM with 4 layers

# 3	Summary

In this contribution, we shared our views for the demodulation requirement discussion for 6Rx covering both handheld UEs and FWA. Proposals are summarized as follows:

Proposal 1: Postpone the discussion of test applicability rule until the relevant test cases are better defined

Proposal 2: Postpone the discussion on the specification structure

Observation 1: From the simulation perspective, performance with option 1 and option 2 are all with high SNR points

Proposal 3: Consider existing medium B correlation until RAN4 has received good justification on newly proposed values.

Proposal 4: Postpone the discussion of the number of test cases for 4 MIMO layer scenario until there is an agreement for issue 2-2-1

Proposal 5: Consider MCS17(table 1): 64QAM for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

Proposal 6: Postpone the discussion of test applicability rule for FWA

Proposal 7: Consider Rel-15 Type 1 DMRS 2+2 if there will be only one case for 6 MIMO layer for FWA test

Proposal 8: Define CQI reporting requirements for 6Rx reception

Proposal 9: Consider following parameter assumptions:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 10: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rule.

Proposal 11: Consider option 2 for the static channel matrix for 8T6R case

Observation 2: It’s feasible from the simulation to define 1024QAM 4-layer SDR requirements

Proposal 12: Define SDR requirements for 1024QAM with 4 layers

# References

[1]	R4-2508727: Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon, RAN4 #115, St Julian’s, Malta

[3]	R4-250xxxx: Simulation results for 6Rx UE demodulation, Ericsson, RAN4 #116, Bangalore, India

# Annex

Parameter configurations and example test cases for information:

Table 5.2.5.1.1-2: Test parameters Parameter; Unit; Value. Test case type; Only single carrier test. TDD pattern; 7DS2U, S: 6D+4G+4U. Channel bandwidth / SCS; 40 MHz / 30 kHz. Propagation channel; 4 Layers: Refer to Issue 2-2-1 and 2-2- - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low. Antenna configuration; 4 Layers: 4T6R 6 Layers: 8T6R. MCS; 4 Layers - ULA Low: Option 1: MCS20 (Table 2); Option 2: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1). PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PRB bundling type; Static. PRB bundling size; 2. TRS configuration; Symbol#{5, 9}. PT-RS; Not configured. DM-RS configuration; 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-3. Overhead; 0. Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx; Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1. (N1, N2), (O1, O2); 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1). Special slot (S slot) scheduling; Not schedule PDSCH in special slot for 6 layers cases. Number of HARQ Processes; 8. HARQ ACK/NACK bundling; Not configured. Maximum HARQ transmissions; 4. Redundancy version coding sequence; {0,2,3,1}. PDSCH & PDSCH DMRS Precoding configuration; Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Receiver type assumption; MMSE-IRC with joint 6Rx MIMO detector. Tx EVM assumption; - 6% for QPSK/16QAM/64QAM - 3% for 256QAM. Test metric; SNR@70% max TP. Table 5.2.5.1.1-3: Minimum performance for Rank 4 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA medium B; 70; xxx. 1-2; xxx; 10 / 15; MCS17; TDLC300-100; 4x6, ULA medium B; 70; xxx. 1-3; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA Low; 70; xxx. Table 5.2.5.1.1-4: Minimum performance for Rank 6 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; xxx; 10 / 15; MCS13; TDLA30-10; 8x6, ULA Low; 70; xxx.