# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511444

Bengaluru, India, August 25-29, 2025

Source:	Ericsson
Title:	LS reply on IDC of GNSS
Agenda item:	7.27.2
Document for:	Discussion

# 1	Introduction

In this paper we present our views on the questions brought by the LS.

# 2	Discussion

In LS [1], it is quoted as follows:

RAN2 would like to thank RAN4 for the original LS in R4-2419898. RAN2 has considered the simultaneous operation between GNSS and UL transmission in NR NTN and concluded not to work on the enhancements to the existing IDC solutions in Release 18.

RAN2 might consider the extension to IDC TDM solution or autonomous denial solution in Release 19, and would like to receive more information on requirements, e.g. on the periodicity and duration of IDC free time to help the UE read the GNSS in NTN.

RAN2 ask the additional information on the periodicity and duration of the IDC in LS. In TR 36.816, the required GNSS bit decoding is described and quoted as follows:

“For example, in GNSS each bit is DSSS spread over few tens of ms, i.e. 20ms bit period in case of GPS. GNSS requires some amount of interference free time every bit period depending upon GNSS receiver phase (i.e. acquisition, tracking phase). There may be no specific requirement that certain portion of bit period is more critical than other. If GNSS receiver can get sufficient percentage of interference free time out of every bit period then it can possibly recover the signal and solve the in-device co-existence issue.”

So basically, there is a need to guarantee the successfully GNSS decoding and from the above discussion, one can derive the cycle time could be corresponding to the DSSS spreading time over GNSS bit, i.e few tens of ms.  And the active duration could be X % of the cycle length.

In current IDC reporting, the cycle length is up to 10240 ms. In the active Duration, it is also flexible and X% can be 50%, 60%, 80% assuming a cycle length of 100ms, which can exceed the few tens of ms GNSS bit duration. UE can transmit the IDC assistance IE when GNSS positioning fix is needed.

- Obsevation-1 : Current IDC specification is good enough to have IDC solution for GNSS.

IDC-TDM-Assistance-r18 ::=            SEQUENCE {

cycleLength-r18                       ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30,

ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160,

ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240},

startOffset-r18                       INTEGER (0..10239),

slotOffset-r18                        INTEGER (0..31),

activeDuration-r18                    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

...

}

In IoT NTN, the GNSS fix time is specified for IoT NTN before, basically this could be referenced when providing the IDC information. GNSS positionFixDuration cover the different GNSS TTFF time including hot, warm and TTFF is defined as below:

“Time To First Fix (TTFF) refers to the time between when a GNSS unit is turned on and it is able to output a valid navigation solution with a given level of performance”

As the cold start takes very long time (30 seconds), (e) Redcap NTN UE or NRN UE should not consider this in connected mode. For hot and warm start of GNSS, we think there is no need to stop the UL/DL traffic as GNSS power on should not disturb the traffic. UE can initiate GNSS reading with IDC request and stop IDC when GNSS reading is done.

Last meeting, some company also want the autonomous denial IDC solution. In our understanding, such solution could mess up link adaptation (for PUCCH and PUSCH) and maybe power control and NW-side detection of RLF, so network implementation should take care the issues caused by IDC and therefore, the network performance may not be optimized.   (slower to detect UEs that have disappeared, slower convergence in PDCCH link adaptation and slower PUSCH link adaptation leading to less system capacity.

In 36.816, the drawback of the denial solution is stated as below, 41.6% UL throughput loss is not good and therefore together with above network impact, we have below proposal.

“The analysis indicates that without eNB knowing the denial resources, the UL throughput loss is up to 41.6% [16]. Therefore, autonomous LTE denials for ISM data packets seem not an acceptable solution for solving steady state situations e.g. voice call.”

Not considering the autonomous denial solution when discussing the IDC.

gnss-PositionFixDurationReporting

If present, this field indicates that UEs capable of performing GNSS position fix in RRC_CONNECTED are configured to include the time duration required to acquire a GNSS position in RRCConnectionSetupComplete-NB, RRCConnectionResumeComplete-NB, and RRCConnectionReestablishmentComplete-NB.

#### GNSS-PositionFixDuration

The IE GNSS-PositionFixDuration indicates the time duration required for the UE to acquire a GNSS position. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on.

GNSS-PositionFixDuration information element

-- ASN1START

GNSS-PositionFixDuration-r18 ::= ENUMERATED{

s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31}

-- ASN1STOP

# 3 	Summary

For LS [1], we have below proposals:

Proposal-1: Not considering the autonomous denial solution when discussing the IDC.

# References

[1] R2-2501568, Response LS on simultaneous operation between GNSS and NR NTN

[2] TR 36.816