3GPP TSG-RAN WG4 Meeting RAN4#116      				R4-2511489 Bengaluru, India

August 25th – 29th, 2025

Agenda Item:	6.14.2

Source:	Lenovo, Motorola Mobility

Title:	On Coexistence Requirements for the NR 4940-4990 MHz Band for US Public Safety

Document for:  Approval

- Introduction

# In RAN #108, work item [1] was agreed with the objective of defining a new 5G NR TDD operating band in the frequency range of 4940-4990 MHz in the U.S. to allow for 5G public safety operation in line with the framework identified in Amendment of Part 90 of the Commission’s Rules in FCC 24-114 WP Docket No. 07-100 [2].

# As captured in the work item description, there is to be a one quarter study phase to identify and document coexistence requirements for the new band with the following description:

# Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band.

    - Identify adjacent spectrum protection requirements

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

    - Identify other technologies operating in the 4940-4990 MHz band

      - Identify potential coexistence mechanism(s)

        - Frequency division sharing

        - Geographic separation

        - TDD synchronization

      - Identify any potential emission limits more stringent than the normal BS and UE emission masks

# In this contribution, we provide our views on these co-existence issues.

#### Adjacent spectrum protection requirements

Generally, only regional regulatory requirements are addressed in 3GPP specifications, and these emissions requirements are typically addressed during the normative phase using A-MPR.  No regulatory requirements have been identified for the spectrum adjacent to 4940 – 4990 MHz. Currently, no NS-signaled additional emission requirements exist for band n79 (4400 – 5000 MHz), which includes the 4940 – 4990 MHz spectrum as a subset.

The emissions masks for 4940 – 4990 MHz are defined in Part 90 Subpart I § 90.210.  The mask for low power emissions less than or equal to 20 dBm are defined in (l).  Specifically, we have

(l) 	Emission Mask L. For low power transmitters (20 dBm or less) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows: …

The mask for high power emissions greater than 20 dBm are defined in (m).  Specifically, we have

(m) 	Emission Mask M. For high power transmitters (greater that 20 dBm) operating in the 4940-4990 MHz frequency band, the power spectral density of the emissions must be attenuated below the output power of the transmitter as follows: …

The emission requirements defined for the band during the normative phase must satisfy the mask corresponding to the power class of the device.  Alternatively, a single emission mask can be defined which meets the requirements for both power classes.  Another alternative would be to use the general NR spectrum emission mask defined in Table 6.5.2.2-1 of TS 38.101-1 if it can be shown to satisfy the emissions requirements of Mask L and Mask M.

Based on the above, we have the following observations:

Observation 1:	No extra regulatory requirements have been identified adjacent to the 4940 – 4990 MHz spectrum in the U.S.

Observation 2:	The spectrum emission limits for the 4940 – 4990 MHz spectrum are defined in Part 90, Subpart I, § 90.210.  Emission Mask L applies for transmitters less than or equal to 20 dBm, and emission Mask M applies for transmitters greater than 20 dBm.

- Other technologies operating in the 4940-4990 MHz band

In the U.S., the spectrum from 4940 – 4990 MHz is licensed for public safety use.  Channels in this band licensed to any entity eligible under § 90.1203(a) are available on a shared basis only.  From sections (e) and (f) of § 90.1207, the licenses must provide the following site-by-site information by June 9, 2025, as indicated below.

(f) Licensees holding active authorizations for the 4940-4990 MHz band on March 30, 2023 shall file the complete site-by-site information described in paragraph (e) of this section for their existing radio systems in the Commission's Universal Licensing System by the compliance date specified in paragraph (g) of this section.

(g) Compliance with paragraphs (e) and (f) in this section shall be required as of December 9, 2024. The deadline for submissions for licensees subject to paragraph (f) in this section is Monday, June 9, 2025.

The filing date was subsequently extended to July 9, 2025.  Thus, all licensee data is now available [3].  As of Oct 27, 2024, there were 3,676 licenses issued in 4940 – 4950 MHz.  It should be noted that the licensing information is extensive and is sufficient to allow for site and frequency coordination of incumbent licensees.  The required license information is included in the Appendix.

In the Seventh Report and Order (WP Docket No. 07-100) [4], the FCC adopted “a nationwide Band Manager “framework to coordinate operations in the 4.9 GHz band, optimize public safety use, and facilitate the integration of the latest commercially available technologies, including 5G, for the benefit of public safety.”  The responsibilities of the Band Manager as given in § 90.1217 include the following:

§ 90.1217 4.9 GHz Band Manager.

The 4.9 GHz Band Manager will have the following primary responsibilities:

(a) Frequency coordination and interference protection for 4.9 GHz band incumbent public safety operations;

(b) Incentivizing the use of the latest commercially available technologies, including 5G;

Observation 3:  Currently only public safety is licensed to operate in 4940 – 4990 MHz spectrum in the U.S.

Observation 4:  All licensee data for the 4940 – 4990 MHz spectrum is available as of July 9, 2025.

Observation 5:  The 4.9 GHz Band Manager has responsibility for frequency coordination interference protection for 4.9 GHz band incumbent public safety operation.

#### Summary

Based on the observations above, it seems that co-existence issues are well understood.  In summary:

- No extra regulatory requirements have been identified adjacent to the 4940 – 4990 MHz spectrum in the U.S.

- The spectrum emission limits for the 4940 – 4990 MHz spectrum have been identified.

- The Band Manager has the responsibility to coordinate interference in 4940 – 4990 MHz and protect incumbent public safety operation.  Further, the Band Manager has both the detailed licensing information and the authority needed to coordinate interference

As a result, we have the following proposal:

Proposal:  Conclude the one-quarter study on coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band and proceed with the normative work to specify the NR band for 4940 – 4990 MHz.

# References

- RP-251887 “New WID on Introduction of NR TDD 4.9GHz Band for US Operation,” 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 09-13, 2025.

- FCC’s Eighth Report and Order, Docket 07-100 adopted October 18, 2024, FCC-24-114A1.

- FCC Universal Licensing Database, Radio Service Codes PF (Fixed Services) and PB (Base/Mobile and Temporary Fixed Operation)  License Search - Advanced License Search.

- FCC’s Seventh Report and Order, Docket 07-100 adopted January 18, 2023, FCC-23-3A1.

Appendix:  § 90.1207 Licensing

(e)	Applications for license in the 4940-4990 MHz band by an entity eligible under § 90.1203(a) must include the following technical information.

(1) The license for base/mobile, mobile-only or temporary fixed (1 year or less) stations will specify, among other parameters, the following technical information:

(i) Coordinates (base).

(ii) Antenna height-to-tip (base and temporary fixed).

(iii) Antenna height above average terrain (base).

(iv) Center frequency, emission designator, and ERP.

(v) Number of units (mobile and temporary fixed).

(vi) Area of operation (mobile and temporary fixed), which shall be limited to the geographic area encompassing the legal jurisdiction of the licensee or, in case of a nongovernmental organization, the legal jurisdiction of the state or local governmental entity supporting the nongovernmental organization. However, applicants may define their areas of operation outside of their areas of legal jurisdiction to assist public safety operations with the permission of the jurisdiction(s) in which the mobile and/or temporary fixed stations are to be operated.

(2) The license for permanent fixed point-to-point, point-to-multipoint and fixed receiver stations must include, among other parameters, the following technical information:

(i) Transmitting station coordinates.

(ii) Frequencies and polarizations.

(iii) For the transmitting equipment, the tolerance, effective isotropic radiated power, emission designator, and type of modulation (digital).

(iv) For the transmitting antenna(s), the model, gain, antenna center line height(s) above ground level and ground elevation above mean sea level.

(v) Receiving station coordinates.

(vi) For the receiving antenna(s), the model, gain, antenna center line height(s) above ground level and ground elevation above mean sea level.

(vii) Path azimuth and distance.