# 3GPP TSG-RAN WG4 Meeting #116	R4-2512546

# Bengaluru, India, August 25th – 29th, 2025

Title: 	Way Forward on NTN Ku band co-existence

Agenda Item:	7.7.3

Source: 	Ericsson

Document for:	Approval

# Way forward

This document summarizes the agreements in RAN4#116 meeting on open issues of Ku Band co-existence simulation assumptions and agreed SAN VSAT ACLR ACS values.

### #1 Simulation assumptions

#### Subtopic 1-1: Clutter loss assumptions in Path loss for Calibration and Co-existence analysis

Agreement: Refer from TR 38.811 Table 6.6.2-2 for clutter loss and shadow fading in Ka band for 35 degrees elevation angle for both calibration and co-existence simulations

#### Subtopic 1-2: ISD assumption

Agreement: Update Ku band co-existence simulation assumptions to use ISD = 450m for TN calibration.

### #2 Co-existence results suggested required ACIR values

The attached excel sheet above summarizes the ACIR values submitted by the interested companies.

- Average ACIR - The values considered are the worst between the 35 and 90 degrees elevation angles. In cases where 5th percentile cannot be achieved, 50th percentile is considered to determine the worst case.

- Average ACIR excluding outliers – The values considered are based on the principle in Clause 6b.3.2.

Agreement:

Table 1: Co-existence results suggested required ACIR values

| Co-existence results suggested ACIR values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ACIR values |  |  |  |
| Scenario | Contributing | NTN | Parabolic |  | Phased array - 45x45 | Phased array - 78x78 |
| 1 | VSAT ACLR | GEO | 20,06 | 21,18 |  | 21,08 |
|  |  | LEO600 | 17,26 | 12,45 |  |  |
| 4 | SAN ACS | GEO | 15 | 17 |  | 15 |
|  |  | LEO600 | 30 | 21,83 |  |  |
| 5 | VSAT ACS | GEO | 36,39 | 38,84 |  | 33,12 |
|  |  | LEO600 | 38,25 | 39,92 |  |  |
| 6 | SAN ACLR | GEO | 0 | 0 |  | 0 |
|  |  | LEO600 | 0 | 0 |  |  |

### #3 Co-existence suggested SAN VSAT ACLR/ ACS values

Based on the results presented in Table 1, Table 3 summarizes the SAN-VSAT ACLR ACS co-existence results suggested values assuming the following TN-UE ACLR ACS values in Table 2 (referenced from IMT studies in TR 38.921 for 10 GHz and TR 38.922 for 15 GHz) -

Table 2: ACLR and ACS of TN

|  | TN |  | Values |
| --- | --- | --- | --- |
| 11 GHz (IMT studies for 10 GHz from TR 38.921) | BS | ACLR | 37 dB |
|  |  | ACS | 40 dB |
|  | UE | ACLR | 24 dB |
|  |  | ACS | 31 dB |
| 14 GHz (IMT studies for 15 GHz from TR 38.922) | BS | ACLR | 30 dB |
|  |  | ACS | 30 dB |
|  | UE | ACLR | 24 dB |
|  |  | ACS | 24 dB |

Following SAN-VSAT ACLR ACS values are derived -

Table3: Co-existence suggested ACLR and ACS of NR-NTN Ku band

|  | SAN |  | VSAT |  |  |
| --- | --- | --- | --- | --- | --- |
|  | GEO | LEO 600 | Parabolic | Phased array 45x45 | Phased array 78x78 |
| ACLR (dBc) | Parabolic: 0 Phased 45x45: 0 Phased 78x78: 0 | Parabolic: 0 Phased 45x45: 0 | GEO: 20.52 LEO: 17.49 | GEO: 21.79 LEO: 12.52 | GEO: 21.67 |
| ACS (dBc) | Parabolic: 15.13 Phased 45x45: 17.22 Phased 78x78: 15.13 | Parabolic: - Phased 45x45: 22.54 | GEO: 45.21 LEO: - | GEO: - LEO: - | GEO: 35.40 |

### #4 Agreed SAN VSAT ACLR/ ACS values

Agreement:

Table 4: Agreed ACLR and ACS of NR-NTN Ku band

|  | SAN |  | VSAT |  |  |
| --- | --- | --- | --- | --- | --- |
|  | GEO | LEO 600 | Parabolic | Phased array 45x45 | Phased array 78x78 |
| ACLR (dBc) | TBD | TBD | TBD | TBD | TBD |
| ACS (dBc) | TBD | TBD | TBD | TBD | TBD |

### #5 CR Work split to include Ku band sections in TR 38.863

Agreement:

| Item | Company |
| --- | --- |
| Big CR to TR 38.863 | ZTE |
| Addition of simulation assumptions as Clause 6b.1, 6b.2 and 6b.3 (Based on TR 38.863 Clause 6.1, 6.2 and 6.3) | Ericsson |
| Collection of simulation results in excel and addition of simulation results as Clause 6b.4 and 6b.5 (Based on TR 38.863 Clause 6.4 and 6.5) | Qualcomm |
| Addition of Calibration results in Annex | SES |