3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513046

# Prague, CZ, 13th - 17th, Oct 2026

Source: 	Samsung

Title: 	Discussion on spectrum for 6GR

Agenda Item:	8.6

Document for:	Approval

- Introduction

Spectrum serves as a fundamental enabler for each cellular generation, and 6G is expected to utilize a broader range of frequencies than ever before. Consequently, it is essential to discuss the definition of bands, band combinations and frequency ranges. Furthermore, with the introduction of an increasing number of new bands, it becomes  more necessary to explore the potential for further simplifying band combination, and to conduct spectrum related regulatory survey.

- Discussion

## 2.1 Band and band combination definition

## 2.1.1 Approaches to secure 6G spectrum

Our general views on 6G spectrum are outlined as follow,

- New band exploration for new spectrum to be licensed:

  - Identified new band widely accepted by regulators and industry: The exploration of new bands for 6G spectrum licensing is underway, with identified bands gaining widespread acceptance from regulators and industry stakeholders. This approach mirrors the successful adoption of previous generations, such as the European Pioneer/Primary bands for 5G (700MHz n28, C band, and 26GHz n258).

  - Capacity and Coverage: The new bands should be designed to provide sufficient bandwidth to meet the anticipated capacity and coverage requirements of 6G, enabling advanced technologies and high data throughput.

  - Economy perspective: Harmonized 6G pioneer bands, supported by regulatory certainty, will establish a unified 6G ecosystem. This fosters economies of scale, driving affordability and widespread deployment of 6G technologies.

  - Spectrum management: Effective spectrum management is critical to prevent spectrum fragmentation, particularly during spectrum refarming. This ensures efficient resource utilization and creates opportunities for new market entrants, enhancing competition and innovation.

- Spectrum clearing: important for candidate bands congested with incumbent services, e.g., 7-24GHz.

  - Example: C-band auction in US, satellite operators agreed to clear spectrum to exchange for relocation cost.

- Spectrum refarming: potentially for all spectrum used for 2G, 3G and 4G, and smooth migration from 5G to 6G.

Observation 1: Our general views on approaches to secure 6G spectrum are outlined as above, includes new band exploration, spectrum clearing and spectrum refarming.

## 2.1.2 Band prefix

For 6G, it is proposed to adopt “s” or “t” as band prefix, resulting in designations such as s77 or t77, s256 or t256 for single band, and CA_s1-s77 or CA_t1-t77 for 6G-6G carrier aggregation.

Proposal 1: It is proposed to adopt “s” or “t” as 6G band prefix.

  - Band combination simplification

Since Rel-15, tens of thousands of band combination have been introduced for NR, yet only less than 10% have been implemented. Significant efforts have been dedicated to new BC requests, requirements analysis(including MSD, UE-UE co-existence, ΔTIB and ΔRIB), TP/CR preparation and review, errors correcting, and new rules adaption. Particularly in Rel-19, RAN4 spectrum work explode with tremendous of new band combinations, leading to a common consensus that a change is necessary for 6G. We elaborate our views in the following sections.

## 2.2.1 Band group concept

PA/LNA/Transceiver ports are already “band-group” basis, whereas duplexer/filter are implemented on a per-band basis to ensure emissions conformance. The addition of multi-plexer is necessary for carrier aggregation support with bands in same “band group” such as Mid-Mid, Low-Low. As illustrated in Figure 1, band combination implementation increases the complexity of UE RFFE design. Additionally, it leads to more complex conformance test and increased signaling overhead.

Fig1. Illustration for UE RFFE for CA_n1-n3 and CA_n25-n30-n66

The so called “Band group concept” has been discussed offline among UE vendors for some time. It seems intrigue the reconsideration of whether carrier aggregation is implementation favourably in 6G day-1 considering that the UE primarily operates in single CC mode for most of the time.

The fundamental idea is to divide FRs into several band groups, such as 5 band groups for FR1 and only 1 band group For FR2-1. Within one band group, only one band is allowed to operate at a time. The anticipated advantages include improved UE performance and reduced cost due to less insertion loss, a lighter RFFE design, more room for FR3, which are benefit from the removal of the multi-plexer.

Further, within one band group, there are two alternatives to be further considered.

- Alt 1) CA via switching

- Alt 2) CA is allowed by decreased Rx Chains per band

Observation 2: The fundamental idea for band group concept is to divide FRs into several band groups. Within one band group, only one band is allowed to operate at a time. Further, within one band group, CA via switching and/or decreased Rx chains per band can be considered.

Observation 3: The anticipated advantages include improved UE performance and reduced cost due to less insertion loss, a lighter RFFE design, more room for FR3, which are benefit from the removal of multi-plexer.

We are open to discuss the band group concept, with the understanding that it should be thoroughly checked with all stakeholders, particularly the operators. In the meanwhile, traditional CA, like what we have for 5G-5G carrier aggregation, should not be precluded for 6G.

Proposal 2: We are open to discuss the band group concept, with the understanding that it should be thoroughly checked with all stakeholders, particularly the operators. In the meanwhile, traditional CA(like what we have for 5G-5G carrier aggregation) should not be precluded for 6G.

## 2.2.2 Further simplification for MSD

In Rel-19 HPUE evolution, successful work [1] has been accomplished to replace case-by-case HPUE MSD analysis with HPUE MSD LUTs (Look up table) that utilizing PC3 MSD values as input, which could significantly reduce the workload for band combination specification. This approach applies to DL inter-band CA with UL as 1UL 1CC (for harmonic, harmonic mixing, cross band isolation MSD) and 2UL 2CC (IMD MSD), where the eligibility criteria are clearly defined.

Moreover, in RAN4#116, proposals [2,3] have been made for DL inter-band CA with UL as TDD 1UL 2CC (such as UL CA_n77(2A)) and 2UL 3CC (such as UL CA_n1A-n77C, which is subject to triple beat MSD) suggesting the adoption of the same HPUE values as PC3. However, the decision on these proposals (dCR) was postponed for further review. Pure intra-band CA, where both UL and DL are intra-band, has not been discussed across the release due to the time constrains. Nevertheless, the LUT mechanism is theoretically applicable to such configurations as well.

Given the limited scope of 5G-A Rel-20, it is less likely that further discussion on MSD simplification will take place. Therefore, it is recommended to continue the further simplification work in 6G out of the common interest. One potential direction is to extend the MSD LUT to cover HPUE cases that were not addressed in Rel-19 work, and maintaining MSD requirements for each band combination. Another potential direction is to explore the possibility to define MSD as frequency ranges basis.

Proposal 3: For MSD simplification, continue the further exploration in 6G. Two potential directions are:

- Extend the MSD LUT approach to cover HPUE cases that were not addressed in Rel-19 work, and maintaining MSD requirements for each band combination

- Have MSD requirements as frequency ranges basis

## 2.2.3 General rules for ΔTIB, ΔRIB

For ΔTIB and ΔRIB, in principle we believe simpler/general rules should be defined to replaced case-by-case analysis in 6G.

Some simplified rules have already been captured in 3GPP RAN4 PRD 01 v0.4.0 (2025-08), as copied below:

----------------------------------------------------------- 3GPP RAN4 PRD 01 v0.4.0----------------------------------------------------

## 5.4	Guidelines on ΔTIB and ΔRIB

### 5.4.1		General

The ΔTIB and ΔRIB values are introduced to allow relaxation to the UEs transmit power (ΔTIB) and reference sensitivity (ΔRIB) to compensate for hardware complexity of supporting a given band combination.

### 5.4.2		2 band CA/DC ΔTIB,c and ΔRIB,c

To define ΔTIB and ΔRIB when introducing new two band CA/DC band combinations the following guidelines can be followed.

Grouping of bands:

- L-bands are below 1 GHz.

- H-bands are above 1 GHz and below 3GHz

- VH-bands are above 3 GHz

Delta values apply as follows for two band CA/DC:

- VH bands ΔTIB =0.8 and ΔRIB =0.5

- H bands ΔTIB =0.5 and ΔRIB =0

  - Except for n1 and n3 which have ΔTIB =0.3

- H-L: Both gets ΔTIB =0.3 and ΔRIB =0

- L-L,H-H: Both gets ΔTIB =0.5 and ΔRIB =0

  - Except for n1 and n3 which have ΔTIB =0.3

- L-VH: L ΔTIB =0.3 and ΔRIB =0.2

- H-VH: H ΔTIB =0.5 and ΔRIB =0.2

- Carriers with any uplink harmonic falling onto the DL:

  - L-H: L ΔTIB =0.6 dB H ΔTIB =0.3 dB

  - L-VH: L ΔTIB =0.6 dB VH ΔTIB =0.8 dB

  - H-VH: H ΔTIB =0.6 dB VH ΔTIB =0.8 dB

- For SDL combinations SDL band gets “N/A” and  L,H ΔTIB =0.3 and VH ΔTIB =0.8

### 5.4.3			More than 2 band CA/DC ΔTIB,c and ΔRIB,c

Delta values for more than two band CA/DC are found as the MAX of the included delta values defined for the two bands CA/DC combinations.

### 5.4.4			Exceptions of ΔTIB and ΔRIB for specific band combinations

Due to multiple reasons the delta values captured together with the band combinations in the specifications may differ from the presented guidelines in Clause 5.4.1 and Clause 5.4.2. These values are found on a case-by-case basis when they are introduced.

----------------------------------------------------------- 3GPP RAN4 PRD 01 v0.4.0----------------------------------------------------

As shown above, the general rules for ΔTIB and ΔRIB are based on the grouping of bands, while case-by-case study is not precluded for some exceptional cases. We believe similar approach should be applied in 6G in the spirit of band combination simplification.

Proposal 4: For ΔTIB and ΔRIB, it is proposed to specify general simplified rules to replace case-by-case analysis unless it is found necessary for some exceptional combos.

- The rules outlined in RAN4 PRD can serve as a starting point

- -ΔTIB and ΔRIB tables are only for exceptional cases where the general rules are not suitable

- General rules are expected to be captured in 6G spec

## 2.2.4 Removal of BCS concept

In 5G era, BCS has been introduced for NR-CA, but not for EN-DC. For EN-DC, one band within a band combination are supposed to support all CBWs that a single band supports, unless indicated otherwise. In contrast, for NR-CA, BCS(s) shall be indicated to NW. Multiple BCSs have been introduced for many combos due to the addition of new channel bandwidths in later releases, which significantly exploding the band combination specification work and increasing the signalling overhead. Though BCS 4/5 was introduced in Rel-17 motivated by the desire for band combination simplification which alleviates the specification workload to some extent, UE indication still remains necessary.

In 6G, we are open to discuss the removal of BCS concept, which means that one band within a band combination is supposed to support all CBWs that a single band supports, unless otherwise indicated. If certain channel bandwidths, such as the maximum channel bandwidth, cannot be supported due to interference concerns to other bands, this can be indicated additionally (e.g., via supportedBandwidthDL and supportedBandwidthUL).

Proposal 5: It is proposed to discuss the removal of BCS concept for band combination.

- This means that one band within a band combination is supposed to support all CBWs that a single band supports, unless otherwise indicated

- If certain channel bandwidths, such as the maximum channel bandwidth, cannot be supported due to interference concerns to other bands, this can be indicated additionally

## 2.3 Definition of frequency ranges

In 5G NR, spectrum usage has been structured into two ranges: Frequency Range 1 (FR1, below 7.125 GHz) and Frequency Range 2 (FR2, 24.25–71 GHz). This categorization was mainly driven by propagation characteristics, device implementation constraints, and regulatory allocation at the time of standardization.

As 6G evolves, there is growing consensus that a new Frequency Range, covering the 7–24 GHz mid-band spectrum, will play a critical role in bridging coverage and capacity between FR1 and FR2. This range offers more favorable propagation compared to FR2 while enabling significant wider bandwidth than FR1. In addition, discussions have emerged on whether further refinements (e.g., sub-ranges within FR3 or extensions toward sub-THz spectrum) are necessary to address diverse deployment needs.

In our view, a key consideration in defining frequency ranges for 6G is the appropriate choice of subcarrier spacing (SCS) and numerology. The propagation delay spread, phase noise, and hardware limitations vary significantly across frequency ranges, making it impractical to adopt same SCS configuration across all bands. For example, smaller SCS values are desirable in lower bands to improve coverage and robustness, while larger SCS values are necessary in higher bands to manage phase noise and support wideband channels.

For new spectrum for 6GR including 15GHz, three options can be considered:

- Option A: Extend FR1 to 8.4 GHz and define a separate FR for the mid-high band (8.4–24.25 GHz) with its own single numerology.

- Option B: Extend FR1 to 15.35 GHz and merge 15.35–24.25 GHz into FR2 family.

- Option C: Define a separate FR for 7.125 – 24.25 GHz.

In either case, spectrum at or above 7.125 GHz should adopt a single SCS, suitable for wider bandwidths and the phase noise environment of higher frequencies.

Among the three options, we would first propose to extend FR1 up to 8.4 GHz considering the channel propagation characteristics and the objective to have similar coverage as for 3.5 GHz.

Proposal 6: For new spectrum for 6GR including 15GHz, among the three options, our recommendation for FR1 is to extend it to 8.4 GHz at least, considering the channel propagation characteristics and previous studies of IMT SI.

## 2.4 Spectrum related regulatory survey

The frequency ranges to be figured out are summarized as following. The selection of potential 6G band candidates is subject to further discussion of ITU.

- 4.4-4.8GHz:

- 7.125-8.4GHz:

- 14.8-15.35GHz:

The ITU status for 4.4-8.4 GHz range is summarized as following.

Observation 4: The selection of potential 6G example bands should take the ITU status into account.

# 3. Conclusion

Observation 1: Our general views on approaches to secure 6G spectrum are outlined as above, includes new band exploration, spectrum clearing and spectrum refarming.

Proposal 1: It is proposed to adopt “s” or “t” as 6G band prefix.

Observation 2: The fundamental idea for band group concept is to divide FRs into several band groups. Within one band group, only one band is allowed to operate at a time. Further, within one band group, CA via switching and/or decreased Rx chains per band can be considered.

Observation 3: The anticipated advantages include improved UE performance and reduced cost due to less insertion loss, a lighter RFFE design, more room for FR3, which are benefit from the removal of multi-plexer.

Proposal 2: We are open to discuss the band group concept, with the understanding that it should be thoroughly checked with all stakeholders, particularly the operators. In the meanwhile, traditional CA(like what we have for 5G-5G carrier aggregation) should not be precluded for 6G.

Proposal 3: For MSD simplification, continue the further exploration in 6G. Two potential directions are:

- Extend the MSD LUT approach to cover HPUE cases that were not addressed in Rel-19 work, and maintaining MSD requirements for each band combination

- Have MSD requirements as frequency ranges basis

Proposal 4: For ΔTIB and ΔRIB, it is proposed to specify general simplified rules to replace case-by-case analysis unless it is found necessary for some exceptional combos.

- The rules outlined in RAN4 PRD can serve as a starting point

- ΔTIB and ΔRIB tables are only for exceptional cases where the general rules are not suitable

- General rules are expected to be captured in 6G spec

Proposal 5: It is proposed to discuss the removal of BCS concept for band combination.

- This means that one band within a band combination is supposed to support all CBWs that a single band supports, unless otherwise indicated

- If certain channel bandwidths, such as the maximum channel bandwidth, cannot be supported due to interference concerns to other bands, this can be indicated additionally

Proposal 6: For new spectrum for 6GR including 15GHz, among the three options, our recommendation for FR1 is to extend it to 8.4 GHz at least, considering the channel propagation characteristics and previous studies of IMT SI.

- Option A: Extend FR1 to 8.4 GHz and define a separate FR for the mid-high band (8.4–24.25 GHz) with its own single numerology.

- Option B: Extend FR1 to 15.35 GHz and merge 15.35–24.25 GHz into FR2 family.

- Option C: Define a separate FR for 7.125 – 24.25 GHz.

Observation 4: The selection of potential 6G example bands should take the ITU status into account.

# 4. Reference

[1] R4-2511752, Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification, RAN4#116

[2] R4-2509759, Proposal on 1UL band 2CC and 2UL band 3CC HPUE MSD, Skyworks Solutions Inc., RAN4#116

[3] R4-2511523, Draft CR to R19 38101-1 to correct HPUE MSDs due to one UL band 2CC IMDs, Skyworks Solutions Inc., RAN4#116