3GPP TSG-RAN WG4 Meeting #116-bis														R4-2513050

Prague, Czech, 13th – 17th October, 2025

Agenda item:			8.10

Source:	Samsung

Title:	Discussion on spectrum sharing for 6GR

Document for:	Approval

# 1	Introduction

In RAN#109 plenary meeting, the objectives of 5G and 6G migration for the work group level 6G SI were updated in [1, RP-252912]

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

For 5G-6G migration, both MRSS and additional migration option(s) study are within RAN4’s scope. In this paper, we share our initial view on MRSS and dual stack as additional option, and potential RAN4 impacts.

# 2	Discussion

## 2.1	General discussion on MRSS vs dual stack

A smooth migration is always an important issue when designing a new generation of IMT communication. In 4G-5G migration, both RAN-level coordination (e.g., DSS) and RAN-level aggregation (e.g., ENDC) were introduced. In 5G-6G migration, Multi-RAT Spectrum Sharing is considered as an effective way to provide 6G coverage on existing 5G bands, especially low band. Given the RAN-level aggregation in 5G NR was not so successful, more migration options need to be studied e.g., dual stack with CN-level aggregation/coordination.

Fig. 1 Illustration of MRSS vs Dual stack

The multi-RAT spectrum sharing enables network operators to simultaneously use a single legacy carrier for both RAT’s services without the need for spectrum re-farming. To achieve high spectrum utilization, spectrum resources between the two RATs can be dynamically coordinated according to the traffic load on the two RATs. The DSS (Dynamic Spectrum Sharing) between LTE and NR enables flexible resource allocation but the NR DL throughput is reduced by around ~15-25% due to fixed overhead from LTE. In 5G-6G migration, it is expected that MRSS would be more efficient because of less always-on signals in 5G NR compared with LTE CRS, though the overhead may not be zero.

Multi-RAT spectrum sharing can not solve all migration issues. MRSS is a migration scheme focusing on network side lacking of UE assistance. MRSS is only applicable for the same band between two RATs. In the transition period from 5G to 6G, there could be many 6G coverage hole and there could be the case 6G has been deployed in new 6G spectrum while 5G exists in FR1 band. Inter-RAT mobility is the basic function for 6G but the service interruption will be inevitable, e.g., voice call. In 6G there will be many other services requiring low latency like immersive service and cloud gaming. Dual stack can provide better user experience with relative low complexity.

Dual stack is a migration scheme at CN-level coordination. A dual stack capable UE can have dual active protocol stack for dual RATs, with simultaneous registration/connection to the network of each RAT. With much less implementation and deployment complexity compared with DC, dual stack provides smooth migration with better user experience in terms of service interruption and latency while exploiting full use of operator’s spectrum and network resources.

Based on above discussion, it can be observed that MRSS is network-centric migration scheme for 5G and 6G at same band enabling flexible spectrum allocation while there might be some overhead. Dual stack is an integrated migration scheme involving both UE and network applicable for more deployment scenarios and provides better user experience and spectrum utilization with less complexity compared with DC.

Observation 1:	Dual stack and MRSS are not contradicted to each other. MRSS is network-centric migration scheme for 5G and 6G at same band enabling flexible spectrum allocation while there might be some overhead. Dual stack is an integrated migration scheme involving both UE and network applicable for more deployment scenarios and provides better user experience and spectrum utilization with less complexity compared with DC.

It is consensus to only consider limited options in 6G. Compared with NSA DC, dual stack is a promising balance between complexity and performance.

Proposal 1:	RAN4 needs to study both dual stack and MRSS for RAN4 related impact.

## 2.2	Dual Stack

In this section, RAN4 impacts of dual stack are discussed. Depending on different implementation options, the RAN4 impacts can be different.

There can be different traffic handling for the different RATs with dual active stack. According to ATSSS (Access Traffic Steering, Switching, Splitting) in TS 23.501 [2], there could be steering based dual stack, switching based dual stack and splitting based dual stack.

The switching based dual stack is most easy implementing from hardware perspective. Dual stacks are both active and UE switches to most suitable RAT for a data flow transmission/reception. There is no power splitting or power sharing so there is no impact on uplink coverage for either RAT. It is operation without inter-RAT handover so the service interruption is minimized which could not be addressed by MRSS. There is minimum spec impact for this option. Most RF requirements are expected to be the same as that of standalone single carrier.

Observation 2:	The switching based dual stack is expected to have minimum RAN4 spec impact. Most RF requirements of standalone single carrier can be reused.

The steering based dual stack may require simultaneous transmission/reception between the two RATs. The UE can steer to different RAT for different services or applications according to core network aggregation principle. One example is that the voice call is steered to 5G RAT and 6G data services are steered to 6G RAT. This option enables better user experience by steering data flow to best suitable RAT respectively.

The splitting based dual stack definitely requires simultaneous transmission/reception between the two RATs. The UE splits a data flow into both 5G RAT and 6G RAT for all services. This option enables peak throughput performance.

Both steering based dual stack and splitting based dual stack are expected to have simultaneous transmission and reception at UE side. There would be more RAN4 impacts and the RF requirements are expected to be similar as that of ENDC.

Observation 3:	Both steering based dual stack and splitting based dual stack are expected to have simultaneous transmission and reception at UE side, so more RAN4 spec impacts are expected.

Proposal 2:	RAN4 needs to study both switching based dual stack and steering/splitting based dual stack respectively for RAN4 related impact.

## 2.3	MRSS

In this section, RAN4 impacts of MRSS are discussed.

MRSS is a solution for the smooth migration in the re-farming bands. So MRSS is not needed for the new 6G spectrum. FR2-1 is existing bands but in our understanding the necessity of MRSS for FR2 is not clear. Usually, FR2 deployment has no full coverage so the most practical deployment is likely to be based on CA or other means with FR1. Therefore, MRSS discussion can be focused on FR1 bands.

Proposal 3:	RAN4 to focus on FR1 bands in MRSS discussion.

As aforementioned, there might be overhead for MRSS. A sharing-based MRSS can save overhead, i.e., reusing NR signals/channels for 6GR, e.g., SSB. However, re-use of NR SSB may lead to multiple SSB designs for 6GR which is not aligned with 6G design principle. Actually, SSB is expected to have minimal overhead, in our understanding, MRSS should avoid to reuse NR signals/channels. This will also minimize the RAN4 RRM impacts.

Proposal 4:	NR signals/channels (e.g., SSB) should not be reused for 6GR in MRSS

The MRSS deployment can be based on either semi-static or dynamic manner. Semi-static MRSS has no resource sharing between NR and 6GR, but with RRC-based rate matching. Generally speaking, semi-static MRSS has less spec impact. FDM or TDM between NR and 6GR is expected and correspondingly there might be different RRM impacts. Dynamic MRSS is similar to DSS with dynamic resource sharing between NR and 6GR by dynamic rage matching without FDM or TDM between the two.

Proposal 5:	RAN4 to discuss the spec impacts for semi-static MRSS and dynamic MRSS respectively before down-selection is made if needed.

# 3	Conclusion

Observation 1:	Dual stack and MRSS are not contradicted to each other. MRSS is network-centric migration scheme for 5G and 6G at same band enabling flexible spectrum allocation while there might be some overhead. Dual stack is an integrated migration scheme involving both UE and network applicable for more deployment scenarios and provides better user experience and spectrum utilization with less complexity compared with DC.

Proposal 1:	RAN4 to recommend dual stack as additional 5G-6G migration scheme.

Observation 2:	The switching based dual stack is expected to have minimum RAN4 spec impact. Most RF requirements of standalone single carrier can be reused.

Observation 3:	Both steering based dual stack and splitting based dual stack are expected to have simultaneous transmission and reception at UE side, so more RAN4 spec impacts are expected.

Proposal 2:	RAN4 to take switching based dual stack as starting point, and further study the RAN4 impacts for steering based dual stack and splitting based dual stack.

Proposal 3:	RAN4 to focus on FR1 bands in MRSS discussion.

Proposal 4:	NR signals/channels (e.g., SSB) should not be reused for 6GR in MRSS

Proposal 5:	RAN4 to discuss the spec impacts for semi-static MRSS and dynamic MRSS respectively before down-selection is made if needed.

# Reference

[1] RP-252912, Revised SID: Study on 6G Radio, RAN#109

[2] TS 23.501, System architecture for the 5G System (5GS)