3GPP TSG RAN4 Meeting #116bis						R4-2513070

Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	6GR Considerations on Spectrum aspects

Agenda item:	8.6

Document for:	Approval

Introduction

In the current Study on 6G Radio [1] the following topics are listed under 6GR.

- UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations

Furthermore, at RAN4#116 a plan for the 6G study in RAN4 was presented by the RAN4 chair as captured in [2]. One of the topics was “Spectrum” with a scope listed as:

- Band/band combination definition and simplification

- Definition of frequency ranges

- Spectrum related regulatory survey

In this contribution we discuss our considerations on the item listed above.

Discussion

Definition of frequency ranges

During NR the concept of frequency ranges were introduced to the specifications to address/quantify the significantly different properties and thereby related requirements in the newly adopted so-called cm/mm-wave spectrum. As a result, RAN4 introduced frequency range 1 (FR1) for the “legacy” spectrum and frequency range 2 (FR2) for the new spectrum. As a result, NR separates the frequency ranges as shown from TS 38.101-1 in Table 2.1-1.

Table 2.1-1: Definition of frequency ranges in NR

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

It can be noted in Table 2.1-1 that FR2 was separated into two sub-ranges as FR2 covers spectrum used for licensed NR deployment while FR2-2 currently covers only unlicensed deployment of NR.

With the introduction of NTN to NR there was a need for expanding the usable spectrum ranges even further. However, to keep the impact to terrestrial NR at a minimum, it was decided to introduce frequency ranges specific for NTN as shown from TS 38.101-5 in Table 2.1-2.

Table 2.1-2: Definition of NTN frequency ranges in NR

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500 MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

From Table 2.1-2 it is noted that FR1-NTN and FR2-NTN is not aligned to FR1 and FR2 respectively. With the latest updates to TS 38.101-5 FR1-NTN and FR2-NTN are even overlapped in the central range. The aim with this was to allow different device types to operate in different manner based on the frequency range.

NR currently has multiple frequency range definitions dependent on not only frequency range but also deployment type.

Since the intent when defining the 6G SI is that NTN operation shall be a part of the generic requirements it is needed considered whether the frequency ranges for TN and NTN use still is to be separated.

RAN4 shall discuss whether to have separated or united frequency range definitions for TN and NTN operation in 6GR.

Additionally, a question for 6G is whether 3GPP wants to adapt similar/same frequency ranges as NR or would make sense to reconsider the use of frequency ranges and their relation to specified requirements more actively.

As also discussed in [3] the frequency ranges can as a suggestion be paired to system parameter attributes as sub-carrier spacing and maximum/minimum channel bandwidth.

RAN4 shall consider paring frequency ranges to specified assumptions and requirements in 6GR.

Furthermore, RAN4 needs to consider the potential new 6GR TN spectrum around 7GHz, 12GHz and 15GHz and its inclusion into frequency ranges or basis for new ranges. This together with the multiple NTN bands which have led to the extension of the NTN ranges shown in Table 2.1-2. All to address the gab and/or overlap between the two existing frequency ranges. Here it shall be noted that even some are referring to some of these regions as FR3, this definition does not exist. To avoid any confusion on what it referred to it is suggested to at least within in 3GPP only to refer to agreed defined frequency ranges.

There shall be no references to “FR3” before an agreed definition of this exist within RAN4.

From our perspective it would makes sense to consider multiple ranges spanning from around 400MHz until around 52GHz. These new ranges can be influenced by the current NR ranges but not necessarily follow the same naming convention.

RAN4 shall discuss a split into multiple frequency ranges spanning from around 400MHz to around 52GHz in 6GR.

Definition of bands and band combinations

Operating band numbering

NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G use same band number if frequency arrangement is same. LTE uses Arabic numeral, WCDMA uses Roman numeral and NR has prefix on “n” in front of Roman numeral. For 6G we propose to re-use prefix concept and just pic a new letter, for example “s”.

NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G had used same band number if frequency arrangement is same. LTE uses Arabic numeral, WCDMA uses Roman numeral and NR has prefix on “n” in front of Roman numeral.

For 6GR operating band naming re-use prefix concept from NR but replace “n” with “s”.

In Rel-21 both new 5G and 6G (and perhaps even 4G) operating bands will be defined. In those cases, the number is chosen from same number pool 65 – 256 on first come first served basis. This pool is currently shared also with 5G NTN bands which select band numbers from top of the range. This is summarized in Table 2.2.1-1 which also includes a suggestion for how to handle new 6G bands.

Table 2.2.1-1: Operating band numbering scheme

As shown in Table 2.2.1-1 a similar approach as used in NR for denoting bands can be used for 6G bands.

Keep using numbering space 1-512 for new bands according to the division shown in Table 2.2.1-1.

If the range from 1-512 is deemed not sufficient in 6GR then it can be considered to add a ranged starting from 513 for use for new 6GR bands as indicated also in Table 2.2.1-1.

Carrier aggregation notation

Band combinations, i.e Carrier Aggregation (CA) or Dual Connectivity (DC), have its own notation. Reading a CA or DC declaration tells exactly the number of carriers and their frequency location through an agreed syntax. For CA syntax is shown on high level in Figure 2.2.1-1.

Figure 2.2.2-1 Inter and intra-band CA syntax of 5G

While the dash in the notation is known always to separate inter-band combinations the underscore is used in many relations and easily leads to confusion. This is because the underscore has the following purposes in the notation for 5G:

- Separates the band combination Type (CA/DC/V2X/SUP/ULSUP_TDM/SL) from the band declaration

- Separates RAT in EN/DC and NE/DC

- Separates downlink from uplink carriers in CA downlink configuration, where SUL is part of the uplink (Not in the Uplink CA configurations)

- Separates band combination Types to allow a second Type declaration in both EN/DC downlink (_SUP) and EN/DC uplink (_ULSUP_TDM)

- V2X and SL are not obligated to use an underscore or at least they are not doing so in current specifications.

The underscore seems not to have a clearly unified defined purpose in current band combinations.

In the band and band combination syntax there is also lots of types. These are CA/ DC/ V2X/ SUP/ ULSUP_TDM/ SL /SLCA.

CA is most frequently used syntax, indicating a combination of carriers, not limited to bands.

DC is introduced as a RAT indication but shares the use of underscore before band declaration. A combination consisting of bands with no letter in front of the band number and those with the letter “n” separated by underscore indicates exactly the same. So, it can be asked why use “DC”, when the bands already show the mix of RATs

The band prefix already can be used to identify DC combinations.

V2X or SL concerns specific bands, but are rather features or capability, than a Type. Most commonly linked to band n47 but has one rare case of n79-n79. These special combination indications are a result of limiting the adaptation of these for a specific device type.

From our perspective every band combination could be created with CA and underscores as the only syntax. This even with a distinguishing for SUL, V2X and SL as this remains a part of the declaration of the bands (e.g. n14, n38, n46, n47, n79, n96, n102).

There is no need to declare the type, but use only CA, when more carriers are combined.

Therefore, it can be questioned whether RAN4 shall adopt similar in 6G timeframe. We believe this is not needed.

RAN4 shall consider not to use RAT or device type indications for band combinations in 6GR.

If possible, to eliminate the special band combination declarations it would allow simplification of band combination notation in general.

Removing special band combination declarations would allow simplification of band combination notation.

The syntax for band combination notation must cover all aspects as simple as possible. This means on CA is used as the Type, and no DC, SUL or other types are supported, since it is absorbed in the new syntax. The CA syntax must underscore only to separate the parts of the declaration as Type_DL_UL. An example is shown in Figure 2.2.1-2.

Figure 2.2.1-2 CA syntax in 6GR.

The band combination (CA) covers the downlink bands of s1, s46 and s78, with the uplink bands of s1 and s46. This compact notation is easy to read, it considers flexible channel BW, so no BCS needed, and it has both uplink and downlink in the same string. For Basket Notation we allow use of DL and UL as types.

- In case of DC -> CA_n1-s3_n1-s3 (“n” and “s” easily identifies RAT)

- In case of SUL ->CA_s1_s1-s81 (SUL is in the band enumeration)

- In case of Sidelink -> s47 (Sidelink is in the band enumeration)

- In case of V2X -> CA_s1-s47_s1-s47 (V2X is in the band enumeration)

In 6G CA syntax consists of a single type (“CA”), underscore separators for Type, DL and UL, one or more downlink bands separated by dash and one or more uplink bands separated by dash.

In 6GR the band combination syntax shall consists of a single type (“CA”), underscore separators for Type, DL and UL, one or more downlink bands separated by dash and one or more uplink bands separated by dash.

Carrier aggregation and switching

In 5G NR the various switching schemes, like UL TxSwitching was introduced and broke with the notation of radio configuration through CA notation. A dedicated UE capability group allowed multiple switchedUL or DualUL declarations related to the carriers of the downlink operation. This means that the CA notation no longer serves for identifying the radio configurations state, and not only carrier deactivation, but also switching may change the active radio configuration at any point during operation.

In NR UL and DL switching schemes means that the CA notation is no longer enough to identify a radio state configuration.

As an example, it is not obvious reading the notation CA_n12A-n29A that a hidden feature of LBCAsw is applicable for making that combination work. The switching is covered in UE capabilities and may even not be read from the UE radio specification, but only in the system parameter specification.

In 6GR there’s a motivation to improve the CA framework, especially considering flexibility in of the configurations. In relation to switching this is also discussed in [5]. Allowing this additional flexibility for CA is desirable but it also means there is even more reason to carefully consider how CA configuration is identified. This to ensure all attributes of the individual configurations are clearly identifiable.

RAN4 shall consider how CA attributes as DL/UL switching can be clearly identifiable for 6GR band combinations

Band combination specific requirements simplification

Definition of UE relaxations per band combination

During NR time frame RAN4 have been investigating how to improve the specifications. As proposed in [3], RAN4 was encouraged to investigate if some of the information/requirements now spread across multiple tables listing band combinations can be merged into a single table listing the band combination.

In the current NR specification multiple tables are now listing band combination specific requirements  meaning to understand requirements for a single band combination one will have to consult numerous long tables in the specification.

A general thing for the band combination specific requirements is that most of these are UE relaxations for either Tx or Rx requirements. UE relaxations per band combination, e.g. ΔTIB,c and ΔRIB,c and even MSD in many cases, have the same value dependent on the type of issue, e.g. harmonics, creating the need for the relaxation. Even so, these relaxation values are currently listed per band combination in multiple tables for each type of issue.

Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonica mixing issues.

The work currently ongoing to move listing of supported band combinations to a database instead of listing them in the DOCX  based specification is not addressing the band combination specific requirements but only the configurations and their supported channel bandwidth etc.

The to move listing of supported band combinations to a database instead of listing them in the DOCX specification is not addressing the band combination specific requirements.

To simplify the specification and the reading of it a suggestion is to list relaxation per UE issue/type in a single table and then list all the types of issues for the specific band combinations in list of supported band combinations. By grouping a list of issues per band combination would provide an overview of the individual band combinations combined need for UE relaxation. This was not done during NR times as the specification impact was deemed too large but for 6G the change principle can be used from the beginning.

RAN4 shall consider for 6GR to providing a single list of supported band combinations together with their impairments requiring relaxation to provide an overview instead of spreading the information over multiple tables in the specification.

In a practical example from 5G NR for TS 38.101-1 only targeting Clause 7 is presented in [3].

Spectrum related regulatory survey

Use of spectrum for 6GR is based on the availability provided by international designations for IMT and regional regulations for the specific use.

Spectrum for 6GR is based on the availability provided by international designations for IMT and regional regulations.

The World Radiocommunication Conference 2023 (WRC-23) made positive decisions about additional bands for 5G/6G, opening the door to the regulatory discussions ongoing globally.

- WRC-23 took the decision that the U6 GHz  can be used for 5G/6G in EMEA, a few Asian countries, and Brazil and Mexico. Additional countries have indicated they would back the band in the coming years and are expected to formalize this during the next radio conference, WRC-27. Currently, Europe is studying technical and regulatory conditions of whether all 700 MHz of the band to be made available for 5G/6G, or only a portion, or if a potential shared use by mobile and Wi-Fi would require limitations which would reduce the appeal for CSPs to deploy 5G/6G using this band. Regulation for the band is expected for 2027, making upper 6 GHz band potentially the pioneer band for the launch of 6G. Under the technology neutral principle, it can also be used, if needed, to support advanced cases of 5G and further be repurposed to 6G.

- The WRC-23 also agreed – for the WRC-27 cycle (AI 1.7) – to study 7-8.4 GHz  for 6G, with the potential of identifying up to 1275 MHz in this range in 2027. Additional bands – 4.4-4.8 GHz and 14.8-15.35 GHz – are also being studied. For the 7-8.4 GHz range, the USA is a key country as it has limited spectrum available in mid-bands following FCC’s 2020 decision to allow unlicensed operations, including Wi-Fi in the full 6 GHz range.

In general, the use and availability of spectrum can be summarized via Figure 2.4-1. From Figure 2.4-1 it can be noted that the potential new 6GR TN spectrum around is focused around 7GHz, 12GHz and 15GHz.

Figure 2.4-1 Potential use of new and existing spectrum bands with 6G technology

Potential new 6GR TN spectrum around is focused around 7GHz, 12GHz and 15GHz

In relation to the potential new 6GR bands in specific, at the ITU-R level, stakeholders are engaging in defining the studies to evaluate the potential of the bands 4.4-4.8, 7.1-8.4 GHz and 14.8-15.35 GHz (under AI 1.7) to ensure that building economically viable and high-performing 6G networks using this spectrum is possible. The findings of the studies will be presented to the ITU-R which will make the final decision of whether to assign the spectrum to 6G at the World Radiocommunication Conference 2027 (WRC-27).

ITU-R is engaging in defining the studies to evaluate the potential of the bands 4.4-4.8, 7.1-8.4 GHz and 14.8-15.35 GHz

Outside of the ITU-R process and given the shortage of spectrum in the mid-bands, the USA is considering additional spectrum in the 3.1-3.45 GHz and in 12 GHz range. The successful use of these bands depends on defining the right operational conditions, i.e. an optimal sharing mechanism in the 3.1-3.45 GHz band to assure coexistence between military radars and mobile with least restrictive conditions for mobile deployment; efficient technologies to allow using the exclusively licensed 12 GHz range for wide area coverage.

FCC is assessing the potential of the bands 3.1-3.45 GHz 12.7-13.25 GHz.

Figure 2.4-2 provides a snapshot of mid-band frequencies that are considered at regional level for 6G based on harmonized conditions as per the WRC-23 decisions, new bands under study for WRC-27, and additional bands considered by the USA for its internal market.

Figure 2.4-2 Mid-band spectrum considered for 6G: 
Harmonized bands as per WRC-23 and new bands under study for WRC-27

From Figure 2.4-2 it can be noted that the upper 6GHz band (1) is already decided for unlicensed use in some regions while it is under discussion in the remaining. For the around 7GHz band (2) this is potentially available for 6GR in all regions. The same in principle applies for the around 15GHz band (3). However, we note that for (3) the incumbents in this bands make coexistence with 6G systems challenging. For (4) this has limited geographical footprint and also faces coexistence challenges. The USA specific bands around 3 GHz and 12 GHz (5) currently no other geographical adaptation.

In conclusion from the above the currently most attractive band, in our view, for 6GR studies within RAN4 is the around 7GHz band.

The currently most attractive band, in our view, for 6GR studies within RAN4 is the around 7GHz band.

Even ITU-R will not make the final decision of whether to assign the around 7GHz spectrum to 6G before WRC-27 (October, 2027 - Shanghai, China) we still believe RAN4 should focus on this band initially for 6GR development.

RAN4 shall initially focus on a new 6GR band around 7GHz.

Conclusion

In this contribution we present our view on spectrum aspects for 6G. The following observations have been made:

Observation 1: NR currently has multiple frequency range definitions dependent on not only frequency range but also deployment type.

Proposal 1: RAN4 shall discuss whether to have separated or united frequency range definitions for TN and NTN operation in 6GR.

Proposal 2: RAN4 shall consider paring frequency ranges to specified assumptions and requirements in 6GR.

Proposal 3: There shall be no references to “FR3” before an agreed definition of this exist within RAN4.

Proposal 4: RAN4 shall discuss a split into multiple frequency ranges spanning from around 400MHz to around 52GHz in 6GR.

Observation 2: NR concept for band numbering has proven successful and very usable. This concept means that 3G, 4G, and 5G had used same band number if frequency arrangement is same. LTE uses Arabic numeral, WCDMA uses Roman numeral and NR has prefix on “n” in front of Roman numeral.

Proposal 5: For 6GR operating band naming re-use prefix concept from NR but replace “n” with “s”.

Proposal 6: Keep using numbering space 1-512 for new bands according to the division shown in Table 2.2.1-1.

Observation 3: The underscore seems not to have a clearly unified defined purpose in current band combinations.

Observation 4: The band prefix already can be used to identify DC combinations.

Observation 5: There is no need to declare the type, but use only CA, when more carriers are combined.

Proposal 7: RAN4 shall consider not to use RAT or device type indications for band combinations in 6GR.

Observation 6: Removing special band combination declarations would allow simplification of band combination notation.

Proposal 8: In 6GR the band combination syntax shall consists of a single type (“CA”), underscore separators for Type, DL and UL, one or more downlink bands separated by dash and one or more uplink bands separated by dash.

Observation 7: In NR UL and DL switching schemes means that the CA notation is no longer enough to identify a radio state configuration.

Proposal 9: RAN4 shall consider how CA attributes as DL/UL switching can be clearly identifiable for 6GR band combinations

Observation 8: In the current NR specification multiple tables are now listing band combination specific requirements  meaning to understand requirements for a single band combination one will have to consult numerous long tables in the specification.

Observation 9: Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonica mixing issues.

Observation 10: The to move listing of supported band combinations to a database instead of listing them in the DOCX specification is not addressing the band combination specific requirements.

Proposal 10: RAN4 shall consider for 6GR to providing a single list of supported band combinations together with their impairments requiring relaxation to provide an overview instead of spreading the information over multiple tables in the specification.

Observation 11: Spectrum for 6GR is based on the availability provided by international designations for IMT and regional regulations.

Observation 12: Potential new 6GR TN spectrum around is focused around 7GHz, 12GHz and 15GHz

Observation 13: ITU-R is engaging in defining the studies to evaluate the potential of the bands 4.4-4.8, 7.1-8.4 GHz and 14.8-15.35 GHz

Observation 14: FCC is assessing the potential of the bands 3.1-3.45 GHz 12.7-13.25 GHz.

Observation 15: The currently most attractive band, in our view, for 6GR studies within RAN4 is the around 7GHz band.

Proposal 11: RAN4 shall initially focus on a new 6GR band around 7GHz.

- References

RP-250858, New SID: Study on 6G Radio,

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

R4-2513144, 6GR System parameters, Nokia

R4-2409190, On UE RF specifications table improvement, Nokia

R4-2513145, 6GR General RF and UE RF, Nokia