3GPP TSG-RAN WG4 #116bis	R4-2513074

Prague, Czech Republic, October 13-17, 2025

Agenda item:	7.10.3		RRM core requirement

Source:	Qualcomm Incorporated

Title:	On the RRM Impact of NR MIMO Phase 6

Document for:	Discussion

# Introduction

In RAN4 #116bis RAN4 will start working on the RAN1-led work item for MIMO Phase 6. The objectives of the work item as defined in [1] are as follows.

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

      - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements:

  - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively)

    - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking

    - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively

  - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs

    - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

The status of the work item is reported in [2]. In this paper, we share our views on the major topics of NR-MIMO Phase 6 and provide a preliminary assessment of the RRM impact.

# Discussion

The work item has two main topics, i.e., improvement of SRS capacity and coverage and enhancing the downlink CSI acquisition. In the following we share our view on the impact of core requirements.

## Improvement of SRS Capacity and Coverage

The purpose of this topic in NR MIMO Phase 6 is to enhance SRS capacity and coverage. RAN1 discusses two sub-topics under this main topic. The first one is to introduce more contiguous symbols for SRS by introducing SRS transmission across two adjacent slots (S and U slots). The second topic is to enhance RB-level partial frequency sounding further by allowing multiple-frequency domain starting positions within each SRS frequency hop.

Regarding the SRS transmission across two adjacent slots the following scenario has been agreed so far in RAN1

Figure 2-1: SRS resource transmitted across two slots (Scenario 1)

Scenario 1 in RAN1 is defined as an SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S-slot and a second consecutive U-slot. That means that the SRS in the U-slot can be transmitted in the beginning of the slot. However, this is already supported since Rel-16 as an optional feature. Requirements in TS 38.133 for SRS antenna port switching always assume that the SRS is in the last six symbols of a slot. Hence, there are no requirements if the SRS comes earlier in a slot than in the last six symbols. We also do not think that new requirements for this case should be introduced now

Observation 1: 5G NR only supports SRS antenna port switching in case SRS is in the last six symbols of a slot.

Proposal 1: RAN4 should not introduce additional RRM requirements for SRS antenna port switching in case of cross-slot SRS.

Another question is whether cross-slot SRS can be applied for SRS carrier switching. RAN1 #122 made the following agreement.

RAN1 #122 Agreement:

When there is a cross-slot SRS resource in the end of a S slot and the beginning of a U slot in a serving cell, support transmitting PUSCH with a priority index 0 and corresponding DMRS after this cross-slot SRS in the U slot in the serving cell.

This agreement is related to the question whether PUSCH can be transmitted after SRS in the U-slot and says that SRS in the S-slot and the U-slot and PUSCH in the U-slot are in the same serving cell. Hence, cross-slot SRS is not attractive for SRS carrier switching. Based on this RAN1 agreement, we also do not think that RAN4 should start discussing requirements for SRS carrier switching.

Observation 2: Based on current RAN1 agreements, cross-slot SRS is not an attractive feature for SRS carrier switching since SRS and PUSCH in both slots are transmitted in the same serving cell.

RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

Proposal 2: RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

Regarding RB-level partial frequency sounding, we do not expect impact to the RRM core requirements by multiple frequency-domain starting positions for SRS frequency hopping.

Observation 3: Multiple frequency-domain starting positions for SRS frequency hopping do not have impact to the RRM core requirements based on current RAN1 agreements

However, it is premature to close this discussion already now and RAN4 can wait for further progress in RAN1.

Proposal 3: RAN4 should wait for further progress in RAN1 to finally decide that multiple frequency-domain starting positions for SRS frequency hopping does not have RRM impact.

## Enhancing Downlink CSI Acquisition

RAN1 discusses two sub-topics under the main topic of enhancing DL CSI acquisition. The first topic is about providing early SRS/CSI/CSI/-RS triggering during transition from IDLE/INACTIVE to CONNECTED mode, from SCell deactivation to activation or from SCell dormancy to active state. The second topic deals with reducing the CSI-RS frequency-domain density for larger number of CSI-RS ports aggregated across multiple CSI resources.

Regarding the early triggering of aperiodic SRS, aperiodic CSI-RS and aperiodic CSI report in the transition from IDLE/INACTIVE to CONNECTED mode, RAN1 made the following high-level agreements that are illustrated in Figure 2-2.

RAN1 #122 Agreement:

For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting):

- Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3.

  - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering.

- Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3.

- Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on  the capability reported by the UE.

- Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting.

  - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting.

Note: The term “capability” above does not mean legacy RRC based UE capability.

Figure 2-2: Timeline for early SRS/CSI/CSI/-RS triggering in IDLE/INACTIVE mode

The UE sends a capability report through msg3 of a 4-step RACH procedure and the network triggers SRS/CSI-RS/CSI through msg4. Our assumption is that any timelines of msg3 and mg4 follow legacy timelines since msg3 and msg4 belong to legacy RACH. The question of interest is how quickly the UE can send aperiodic SRS-AS and receive aperiodic CSI-RS after the triggering through msg4. The second question of interest is how quickly the UE can process CSI-RS and send the aperiodic CSI report to the network. In our view these questions should be addressed first in RAN1 and RAN4 only needs to get involved in case that RAN1 has specific questions to RAN4. Hence, RAN4 can wait for further progress in RAN1 before it looks at early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode.

Observation 4: The question of interest is how quickly the UE can send aperiodic SRS-AS and receive aperiodic CSI-RS after the triggering through msg4. The second question of interest is how quickly the UE can process CSI-RS and send the aperiodic CSI report to the network.

Observation 5: In our view these questions should be addressed first in RAN1 and RAN4 only needs to get involved in case that RAN1 has specific questions to RAN4.

Proposal 4: RAN4 should wait for further progress in RAN1 for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode.

We do not expect impact to RRM core requirements by the reduction of frequency domain density for larger number of CSI-RS ports above 32. However, reduction of frequency domain density impacts demodulation performance.

Observation 6: Reduction of frequency domain density for larger number of CSI-RS ports does not have impact to the RRM core requirements.

Proposal 5: RAN4 should not further discuss RRM impact of frequency domain density reduction for CSI-RS.

Regarding the early trigger of SRS-AS/CSI-RS/CSI, RAN1 made the agreement that the definition of the requirement is on RAN4. That means that the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI must be defined by RAN4.

Proposal 6: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after Scell activation command is received.

RAN1 #122 Agreement:

For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell.

- FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4)

- FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command

- FFS: Whether/what new information is provided in SCell activation command?

RAN1 #122 Agreement:

For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell.

- The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

- FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command?

- FFS: Whether/what new information is provided in SCell activation command?

However, certain details are not yet clear in RAN1 that will impact the timeline. For example, it is still open in RAN1 how to trigger the aperiodic SRS-AS and the aperiodic CSI-RS and aperiodic CSI report based on legacy Scell activation command. This triggering can be either done by a MAC CE or by a DCI.

Observation 7: It is still open in RAN1 how to trigger the aperiodic SRS-AS and the aperiodic CSI-RS and aperiodic CSI report based on legacy Scell activation command.

Figure 2-3 illustrates the current timeline of Scell activation for FR1 in case the Scell is known and the SCell measurement cycle is less than or equal to 160 ms.

Figure 2-3: Timeline for early SRS/CSI/CSI/-RS triggering in SCell activation procedure

It is seen that there is a fixed time of TFirstSSB + 5 ms that includes MAC CE processing and SSB processing times. This first SSB is required for fine time and frequency tracking. This time and frequency tracking impacts both uplink transmission as well as downlink reception. Hence, aperiodic SRS-AS and aperiodic CSI-RS has to come after the first SSB. The figure also shows the P-CSI-RS after the first SSB. Legacy SCell activation makes use of this P-CSI-RS and its associated CSI report. Based on the figure it seems that the gain in activation delay is mainly coming from the fact that now an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15, and it is a simple task for the network to accelerate SCell activation by Rel-15 features. Hence, the benefit of this feature is not clear and should be discussed in RAN4.

Observation 8: The gain in activation delay is mainly coming from the fact that an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15. The same holds for aperiodic SRS-AS. Also, this can be done already today.

Proposal 7: RAN4 should discuss the use case and benefit of early triggering of aperiodic CSI-RS and aperiodic SRS-AS in SCell activation procedure.

Proposal 8: RAN4 should discuss sending an LS to RAN1 asking for clarification of the use case.

# Conclusions

In this paper we provided our views on the major topics of NR MIMO Phase 6 and a preliminary assessment of the RRM impact. Our observations and proposals are summarized as follows:

Improvement of SRS Capacity and Coverage

Observation 1: 5G NR only supports SRS antenna port switching in case SRS is in the last six symbols of a slot.

Proposal 1: RAN4 should not introduce additional RRM requirements for SRS antenna port switching in case of cross-slot SRS.

Observation 2: Based on current RAN1 agreements, cross-slot SRS is not an attractive feature for SRS carrier switching since SRS and PUSCH in both slots are transmitted in the same serving cell.

Proposal 2: RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed.

Observation 3: Multiple frequency-domain starting positions for SRS frequency hopping do not have impact to the RRM core requirements based on current RAN1 agreements

Proposal 3: RAN4 should wait for further progress in RAN1 to finally decide that multiple frequency-domain starting positions for SRS frequency hopping does not have RRM impact.

Enhancing Downlink CSI Acquisition

Observation 4: The question of interest is how quickly the UE can send aperiodic SRS-AS and receive aperiodic CSI-RS after the triggering through msg4. The second question of interest is how quickly the UE can process CSI-RS and send the aperiodic CSI report to the network.

Observation 5: In our view these questions should be addressed first in RAN1 and RAN4 only needs to get involved in case that RAN1 has specific questions to RAN4.

Proposal 4: RAN4 should wait for further progress in RAN1 for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode.

Observation 6: Reduction of frequency domain density for larger number of CSI-RS ports does not have impact to the RRM core requirements.

Proposal 5: RAN4 should not further discuss RRM impact of frequency domain density reduction for CSI-RS.

Proposal 6: RAN4 to define the timeline of the aperiodic SRS-AS transmission and the timeline of the aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI after Scell activation command is received.

Observation 7: It is still open in RAN1 how to trigger the aperiodic SRS-AS and the aperiodic CSI-RS and aperiodic CSI report based on legacy Scell activation command.

Observation 8: The gain in activation delay is mainly coming from the fact that an aperiodic CSI-RS is used instead of a periodic CSI-RS. However, an aperiodic CSI-RS can already be used in Rel-15. The same holds for aperiodic SRS-AS. Also, this can be done already today.

Proposal 7: RAN4 should discuss the use case and benefit of early triggering of aperiodic CSI-RS and aperiodic SRS-AS in SCell activation procedure.

Proposal 8: RAN4 should discuss sending an LS to RAN1 asking for clarification of the use case.

# References

- RP-252936, Revised WID: NR MIMO Phase 6, RAN #109, September 2025

- RP-252850, Status Report NR MIMO Phase 6, RAN #109, September 2025