3GPP TSG-RAN WG4 Meeting # 116bis                                       R4-2513084

Prague, CZ, Oct. 13 – 17, 2025

Agenda Item: 6.17.2.1

Source: MediaTek Inc.

Title: Discussion on test cases for event triggered L1 measurement reporting

Document for: Discussion

# Introduction

In this contribution, we would like to discuss the RRM performance requirements to be defined for R19 mobility.

# Discussion

Last meeting, RAN4 discussed the test cases to be defined for event-triggered L1 reporting and reached the following progress.

Recommended WF: - Further discuss based on the following directions - For SSB based L1 RSRP measurement reporting: - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple) - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW) - For CSI-RS based L1 RSRP measurement reporting: - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. - For UE supporting event triggered reporting, UE only needs to pass 2) - For UE not supporting event triggered reporting, UE only needs to pass 1)

For L3 measurement, there are periodic reporting, event-triggered reporting and event-triggered periodic reporting. RAN4 only defined test cases for event-triggered reporting. We think the reason behind is that UE measurement behaviour is almost the same for periodic reporting, event-triggered reporting and event-triggered periodic reporting.

Observation 1: For L3 measurement, RAN4 only defined test cases for event-triggered reporting.

For SSB L1 measurement, RAN4 had defined test cases for intra-f, inter-f without gap and inter-f with MG in R18 for both FR1 and FR2. The only difference between periodic reporting and event-triggered reporting is that UE needs to compare the measurement results and determines whether to report. Such an additional comparison is a pure baseband calculation and is agnostic to scenarios like intra-/inter-f or w/wo gap. Therefore, we don’t think it is necessary to test all the cases for event-triggered reporting. We understand that there were many proposals to test different LTM events. Approach 1 may be a good compromise. But we don’t think a UE will fail test case for LTM4 if UE can pass the test case for LTM3 as it is just a simple comparison of the measured results with the threshold. Considering the workload, we still prefer approach 2, i.e., keep R18 test cases, develop necessary new R19 test to verify reporting behavior.

Proposal 1: For event-triggered L1 reporting, define the following two test cases:

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

# Summary

In this paper, we provide some view on performance requirements of R19 mobility. We have the following proposals:

Observation 1: For L3 measurement, RAN4 only defined test cases for event-triggered reporting.

Proposal 1: For event-triggered L1 reporting, define the following two test cases:

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

- Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2