# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513099

# Prague, Czech Republic, 13 – 17 Oct 2025

Agenda item:	4.20.2

Source:	OPPO

Title:	Discussion on RRM core requirements for A-IoT

Document for:	Approval

- Introduction

This contribution will provide our considerations on RRM core requirements for A-IoT.

- Discussion

Issue 1-2-2: The value for TD2R_min Agreement: - The value of TD2R_min is max {3* TD2R_chip, 10 us}, where TD2R_chip is the chip length of D2R transmission

CR R4-2512237

Last RAN4 meeting agreed the value of TD2R_min in [1] and the related CR [2]. As shown above, the value of TD2R_min is related with TD2R_chip, which is the duration of a D2R chip as specified in TS 38.291. However, it is observed that TD2R_chip is different with different small frequency shift factor in case of FDM, i.e. TchipD2R=TbitD2R/2×RSFS as illustrated in the figure below. To avoid time misalignment in this case, RAN1 defined time offset (e.g. TR→D) using the largest value of TchipD2R, which is highlighted in green below. The same method could be applied for TD2R_min, and we proposed to update TD2R_chip above as Tchip'. Otherwise, devices transmitting D2R on different frequency resources may have different TD2R_min.

Observation 1: TD2R_chip is different with different small frequency shift factor in case of FDM.

Proposal 1: For TD2R_min, update TD2R_chip as Tchip' defined in clause 7.1.2 of TS 38.219.

Figure 1, illustration of TD2R_chip in case of FDM

### 7.1.1 Device procedure for D2R generation A device shall generate the D2R transmission using the following parameters provided by higher layers: - the duration in microseconds of each D2R bit, TbitD2R - the block repetition number, Rblock - the small frequency shift factor to be used, RSFS - the interval in bits for D2R midamble insertion, Ibit - sequence length indicator for D2R-ambles, Lamble - the additional D2R midamble insertion indicator, Iadd - the channel coding indicator, Rcode The device shall: - set TchipD2R=TbitD2R/2×RSFS - if Lamble indicates a short D2R amble sequence, set lamble=7; otherwise set lamble=31. ### 7.1.2 Device procedure for transmission time determination A device shall upon receiving a PRDCH intended for the device in an R2D transmission ending in chip χendR2D=MchipR2D-1, perform a corresponding D2R transmission with chip χD2R=0 starting an amount of time TR→D after the end of chip χR2D= χendR2D according to the configuration received from higher layers. If the D2R transmission is for a Random ID message (Msg1) or corresponds to a Random ID Response message (Msg2) - the device shall determine the D2R transmission starting time using the following parameters determined by higher layers: - the set of NSFS≥1 potential small frequency shift factors Ri,SFS', i=1, 2, …, NSFS - the D2R transport block size in bytes, NTBSD2R - Tchip' is equal to the largest value among TbitD2R/(2×Ri,SFS') otherwise - Tchip'=TchipD2R. If the D2R transmission is for a Random ID message - if after chip χR2D= χendR2D there are potential access occasion(s), as defined in TS 38.391 [3], for the transmission which are earlier in time than the access occasion selected for the transmission - the device shall set TR→D=1.25Toffset+MchipD2RTchipD2R - otherwise - the device shall set TR→D=Toffset else if the D2R transmission corresponds to a R2D Random ID Response message or to a contention-free random access procedure - the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-1 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used Table 7.1.2-1 Tchip' [μs]; Δcode [μs]. τ; 0. τ/2. τ/4. τ/8. τ/16. τ/32; τ. τ/64. τ/128. τ/192. otherwise - the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-2 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used Table 7.1.2-2 D2R block size [bytes]; Δcode [μs]. NTBSD2R≤32; 2τ. 32<NTBSD2R≤64; 4τ. 64<NTBSD2R≤125; 8τ. where Toffset has the value given in Table 7.1.2-3 and Mchipsymb is as determined for the corresponding PRDCH. Table 7.1.2-3 Tchip' [μs]; Toffset [μs]. τ; 10τ. τ/2. τ/4; 5τ. τ/8. τ/16. τ/32; τ. τ/64; τ, if Mchipsymb=2 τ/4, if Mchipsymb=6 or 12 or 24. τ/128. τ/192.

- Conclusion

This contribution gave our considerations on TD2R_chip and the following proposals.

Observation 1: TD2R_chip is different with different small frequency shift factor in case of FDM.

Proposal 1: For TD2R_min, update TD2R_chip as Tchip' defined in clause 7.1.2 of TS 38.219.

References

- R4-2512134, WF on RRM for Ambient_IoT_Solutions, CMCC, RAN4 #116

- R4-2512237, DraftCR on D2R Timing requirements, CMCC, RAN4 #116

- TS 38.291, Ambient IoT Physical layer (Release 19)