3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513115

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Inputs to RAN4 6G study on general RF and UERF

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.4 General RF and UE RF [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#109, the RAN-led 6G study objectives were updated in [1] adding the RAN4 related topics. In this contribution, we share our views on general RF and UERF.

# Discussion

## 2.1 6G SI objectives relevant to UE RF

The portion of the 6G SI objective most relevant to the UE RF requirements is reproduced below in italics and highlighted in yellow.

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

- Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

- Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

6GR core and performance requirements

- General RF aspects [RAN4]

- Intra-3GPP co-existence studies

- Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

- BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

- Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

- UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

- Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

- RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

- Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

- Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

- Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

- Handling irregular channel bandwidths including the definition [RAN4, RAN1]

- Definition of ‘frequency range(s)’[RAN4]

Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

- PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

- Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

- Aspects of integration with communication services [RAN1]

- higher layer procedures and protocol aspects [RAN2]

- RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In this contribution, we will cover general RF and UERF per the RAN4 chairman note associated with the corresponding agenda item:

spectrum aggregation framework, UE RF related coverage and energy efficiency, and other joint UE-BS RF issues.

## 2.2 6G UE RF related coverage, power class concept

For 5G the latest releases all focused on higher UE output power for UL coverage. For 6G, we think this issue will be further exacerbated as the cell edge performance should improve.

This means that the minimum 6G UE power capability should start from the current state of the art which is based on PC2 capable PAs used in 1Tx (PC2), 2Tx (PC1.5) and 4Tx (PC1).

5G also has different set of power classes that do not necessarily align:

- The single band power class

- The intra-band CA power class

- The inter-band CA power class

- The per band per BC power class

There are artificial limitations to the power that the UE can reach:

- Boosting is only allowed optionally and only for a limited set of allocations and modulation order.

- It was shown that many other boosting options may exist:

  - For lower modulation order w/wo shaping

  - For inner third allocations

  - In cases where the OOB emissions are further away (extended bandwidth cases)

Proposal on UE power class for 6G:

- Default power class (for smartphone UE) is PC2 (26dBm nominal maximum power) unless limited by regulation (FDD bands in Japan, specific in-band PSD/power regulation)

- Intra-band CA power class is the same than the band power class (note that intra-band CA should not be specified if the spectrum can be addressed by a single CC)

- No inter-band power class: the UE declares the maximum power in each band and the total power can reach between the power of the lowest power class band and the power sum of the two bands’ power class.

- Upper limit of power class always allows UEs to boost their power up to 3dB.

  - This could be enabled with setting power class tolerance as +5/-2dB

    - FFS if this optional or it is signaled by the UE

    - FFS if boosted value is specified (UE boosts what it can up to its best level based on emission margins, impairments, thermal or DC current limitations)

    - The BS can still change the upper limit with PUmax (e.g. to meet regulation)

  - Then power classes would overlap and thus a power class every 3dB may prove sufficient

- Power class definition should be based on a target nominal power, a reference waveform and MPR for a given emission requirement.

  - For 6G, a CBW wider than 20MHz for the reference waveform would be more adequate like 50MHz <2.3GHz and 100MHz >2.3GHz.

Proposal on simplification of UL related work:

- To simplify MPR specification across #Tx, antenna coupling assumptions could be varied across frequency ranges and #Tx to have a single requirement.

- To simplify A-MPR, it could be defined only for the CBW that are relevant to the deployment in the region where the additional emissions apply and only tested for those. With wider channel bandwidths, the -50dBm/MHz band protection maty become more difficult and be subject to A-MPR.

For increased UL coverage and cell edge performance, when multiple antennas are available multi-stream MIMO is not the best approach as the required SNR is not available. However, single stream MIMO based on different phases on the different antennas, although not providing beamforming as in phase arrays, may prove useful to shape the radiation pattern and gain a few dB in one direction. To fully exploit this, it may be of interest to have more phase states than every 90 degrees as of today, at least for more than two UL antennas. This may prove practical for NTN or combat hand effect.

Proposal on UE digital beamforming for multiple antennas: Investigate higher phase step granularity to improve radiation pattern directivity for above two discrete UL/DL antennas.

On the DL side, improved coverage is also related to NF and RFFE losses. It is possible to improve REFSENS for 6G while simplifying the RAN4 work.

Proposal on DL performance:

- Consider simplifying (eliminating?) DeltaT and DeltaR tables but embed reasonable losses in REFSENS and assume multi-band support.

- Consider improved NF and filter losses depending on frequency ranges and FDD/TDD

- For FDD bands:

- Consider 55dB Tx/Rx isolation and 50dB TX to antenna rejection for FDD bands

- Use fixed main path to diversity path imbalance (fixed MRC gain, potentially versus frequency ranges)

- Consider revisiting UL allocation position and size: worst case/best-case?

## 2.3 Spectrum aggregation framework

At this time, it is still unclear what spectrum aggregation should be considered within 6G and between 5G and 6G. Also, it is unclear whether DC/DL/UL CA/simultaneous Tx/Rx should be considered. However, based on the 5G experience, these aspects need to be clarified early on and, if needed, the MSD rules and framework should be simplified and adopted early.

Proposal on spectrum aggregation: for 5G and 6G aggregation consider the following simplifications:

- No intra-band 5G + 6G support

- Favor inter-band 5G+6G with 5G and 6G in different band groups and avoid in band-group 5G+6G

- No support for non-contiguous ULCA and non-contiguous allocations for contiguous CA in 6G

- 6G contiguous CA only considered if the spectrum cannot be supported with the maximum CBW (no BW class B)

- Consider if some band-group combination should have no MSD as default (e.g. between bands <1GHz and bands >3.3GHz).

# Conclusion

In this contribution, we provide our views on the Release 20 RAN4 6G SI general RF and UERF aspects.

Proposal on UE power class for 6G:

- Default power class (for smartphone UE) is PC2 (26dBm nominal maximum power) unless limited by regulation (FDD bands in Japan, specific in-band PSD/power regulation)

- Intra-band CA power class is the same than the band power class (note that intra-band CA should not be specified if the spectrum can be addressed by a single CC)

- No inter-band power class: the UE declares the maximum power in each band and the total power can reach between the power of the lowest power class band and the power sum of the two bands’ power class.

- Upper limit of power class always allows UEs to boost their power up to 3dB.

  - This could be enabled with setting power class tolerance as +5/-2dB

    - FFS if this optional or it is signaled by the UE

    - FFS if boosted value is specified (UE boosts what it can up to its best level based on emission margins, impairments, thermal or DC current limitations)

    - The BS can still change the upper limit with PUmax (e.g. to meet regulation)

  - Then power classes would overlap and thus a power class every 3dB may prove sufficient

- Power class definition should be based on a target nominal power, a reference waveform and MPR for a given emission requirement.

  - For 6G, a CBW wider than 20MHz for the reference waveform would be more adequate like 50MHz <2.3GHz and 100MHz >2.3GHz.

Proposal on simplification of UL related work:

- To simplify MPR specification across #Tx, antenna coupling assumptions could be varied across frequency ranges and #Tx to have a single requirement.

- To simplify A-MPR, it could be defined only for the CBW that are relevant to the deployment in the region where the additional emissions apply and only tested for those. With wider channel bandwidths, the -50dBm/MHz band protection maty become more difficult and be subject to A-MPR.

Proposal on UE digital beamforming for multiple antennas: Investigate higher phase step granularity to improve radiation pattern directivity for above two discrete UL/DL antennas.

Proposal on DL performance:

- Consider simplifying (eliminating?) DeltaT and DeltaR tables but embed reasonable losses in REFSENS and assume multi-band support.

- Consider improved NF and filter losses depending on frequency ranges and FDD/TDD

- For FDD bands:

- Consider 55dB Tx/Rx isolation and 50dB TX to antenna rejection for FDD bands

- Use fixed main path to diversity path imbalance (fixed MRC gain, potentially versus frequency ranges)

- Consider revisiting UL allocation position and size: worst case/best-case?

Proposal on spectrum aggregation: for 5G and 6G aggregation consider the following simplifications:

- No intra-band 5G + 6G support

- Favor inter-band 5G+6G with 5G and 6G in different band groups and avoid in band-group 5G+6G

- No support for non-contiguous ULCA and non-contiguous allocations for contiguous CA in 6G

- 6G contiguous CA only considered if the spectrum cannot be supported with the maximum CBW (no BW class B)

- Consider if some band-group combination should have no MSD as default (e.g. between bands <1GHz and bands >3.3GHz).

# References

[1] RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN4#109