3GPP TSG-RAN WG4 Meeting #116bis                      R4-2513141                                                                                                       Prague, CZ, 13th – 17th October 2025

Source:	China Telecom

Title:	Discussion on UE requirements for FR1 SU-MIMO with spatial channel model

Agenda Item:	7.5.3

Document for:	Discussion

Introduction

In RAN #109 meeting, the new WID on NR demodulation performance: Phase6 was approved [1], with the following objectives for the normative work for spatial channel model (SCM):

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

In this contribution, our views on UE requirements for FR1 SU-MIMO with spatial channel model is provided.

Discussion

According to the WID, the normative work includes mainly 3 parts: update of rCDL-C1 channel model considering the outcome in SI FS_NR_7_24GHz_Chmod (if necessary), test scope and test parameter selection, test applicability.

Possible update to rCDL-C1 channel model

In the current rCDL-C1 channel generation of TR38.753, there remains editor note for the angle scaling part that ‘The angle scaling procedure included in this section was used during the work in Rel-19 study item, and it is based on the companies’ agreed interpretation of the procedure, not captured in the version of TR 38.901 (v18)’.

Therefore, we propose to resolve this editor note and discuss the possible update to the current rCDL-C1 channel model.

Proposal 1: RAN4 should first resolve this editor note for the angle scaling for rCDL-C1 and discuss the possible update to the channel model.

Frequency generalization

According to the current TR38.753, FR1 3.5GHz is used as the default carrier frequency for the simulation study. And frequency generalization is one of the discussion points in the WID.

The purpose of the existing RAN4 performance requirements definition is to define band-agnostic requirements that can be applicable for all supporting bands in the same frequency range.

Therefore, while discussing the possible rCDL-C1 channel model update, RAN4 should ensure there should not be big performance difference while performing the test under difference carrier frequency within FR1, and we do not think carrier frequency should not be captured as one of the test parameters in the requirements.

Proposal 2: While discussing the possible rCDL-C1 channel model update, RAN4 should ensure there should not be big performance difference while performing the test under difference carrier frequency within FR1.

Proposal 3: Carrier frequency should not be captured as one of the test parameters in the requirements.

Test scope and test parameters

Duplex mode and SCS

In the Rel-19 SI, all simulation results are based on TDD 30kHz SCS with 40MHz CHBW. As most of the existing UE demodulation requirements have covered both TDD and FDD, we think RAN4 needs to discuss whether there is additional need to cover FDD 15kHz SCS with 10MHz CHBW, which means additional simulation workload is required.

Proposal 4: Cover TDD 30kHz SCS with 40MHz CHBW and discuss whether to also cover FDD 15kHz SCS with 10MHz CHBW.

Whether to define eType II PMI reporting requirements based on SCM

According to the WID, it has been approved that RAN4 will define requirements for PDSCH demodulation performance requirements for the 8T8R rank8 and 4T4R rank4 case as studied in TR38.753. Same time, it remains undecided whether to define eType II PMI test cases, given that the the simulation results for eType II PMI is not well aligned in terms of the SNR @70% and @90% max TP, as captured in Table 7.1-3 in TR38.753.

Table 7.1-3 Simulation result summary for FR1 SU-MIMO PMI 8Tx4Rx with 4 layers, follow PMI SNR at Norm. Throughput [dB]; Source #1; Source #2; Source #3; Source #4; Source #5; Source #6; Source #7; Source #8; Source #9; Average; Span. Type-I; 70%; 12.4; 11.0; 9.7; 10.3; 10.2; 9.7; 9.5; 10.2; 9.8; 10.4; 2.9. 90%; 14.4; 13.1; 12.5; 12.8; 12.6; 12.6; 12.0; 12.5; 12.3; 12.7; 2.4. eType-II; 70%; 14.0; 14.3; 8.4; 11.6; 10.1; 10.8; 8.2; 13.8; 10.8; 11.5; 6.1. 90%; 15.9; 17.1; 12.3; 14.0; 12.6; 13.9; 10.6; 16.8; 12.9; 14.0; 6.5.

We also observe that companies’ results for Type I PMI as well as SU-MIMO PDSCH are well-aligned, which means that the rCDL-C1 channel modeling has been aligned between companies.

eType II PMI enables much more accurate CSI feedback compared with Type I PMI. Thus it is naturally difficult for companies to align their simulation results especially under SCM condition, considering different vendors will implement different channel estimation accuracy and different CSI calculation algorithms.

Since Rel-16 Type II PMI, for original TDL model, it was hard for companies to align simulation results. RAN4 defines enhanced PMI requirements by selecting the lowest value among the ones companies provided.

Observation 1: According to TR38.753, companies’ results for Type I PMI as well as SU-MIMO PDSCH are well-aligned.

Observation 2: Since Rel-16 Type II PMI, for original TDL model, it was hard for companies to align simulation results. RAN4 defines enhanced PMI requirements by selecting the lowest value among the ones companies provided.

We believe PMI reporting is one of the critical aspects for UE performance verification under SCM, thus we propose to specify requirements for PMI performance with 8T4R eType II precoding.

For 4T4R eType II as captured in the WID, we notice that it was not studied in the Rel-19 SI, which means additional simulation work will be required.

Proposal 5: Specify requirements for PMI performance with 8T4R eType II precoding by selecting the lowest value among the ones companies provided.

Test metric for eType II PMI reporting

As for the test metric for eType II PMI reporting, we prefer to follow the same rule for the existing eType II requirement based on TDL, i.e., following PMI (eType II)/Random PMI (Type I codebook).

Proposal 6: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Test parameters for eType II PMI reporting and PDSCH demodulation

As for the test parameters, we propose to reuse the simulation assumptions for the Rel-19 SI as much as possible to avoid duplicate simulation workload, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Proposal 7: Reuse the simulation assumptions/test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Test metric for PDSCH demodulation requirements

In the Rel-19 study, the SNR at both 30% and 70% of max throughput are covered. Based on our observation of companies’ simulation results as in Table 7.1-1 and Table 7.1-2 of TR38.753, the results are well-aligned for 30% and 70%.

We propose to use the same metric as for TDL channel model, i.e., SNR @ 70% max TP.

Moreover, for 8T8R rank8, from Table 7.1-2 of TR38.753, we observe huge performance difference between the 2 codewords. (SNR difference of 5.9~11.8dB). Since different performance for different codewords is one of the main motivations to study spatial channel model, we propose to define SNR requirement values for each codeword separately.

Proposal 8: Use the same test metric as for TDL channel model for PDSCH demodulation, i.e., SNR @ 70% max TP.

Observation 3: For 8T8R rank8, the performance difference between the 2 codewords is at the range of 5.9~11.8dB as captured in TR38.753.

Proposal 9: For 8T8R rank8, define SNR requirement values for each codeword separately.

Test applicability

It is clearly captured in the WID that all SCM new requirements are optional. Therefore, RAN4 needs to introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration.

It remains unclear that if RAN4 will define one single declaration on SCM requirements, or define multiple declarations for each individual test cases.

In addition, RAN4 is now defining requirements for the existing UE features, e.g., 4T4R rank4 requirement applies to Rel-15 UEs and 8T8R rank8 requirement applies to Rel-17 8Rx UEs supporting max 8 layers. Therefore, RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Proposal 10: Introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration. RAN4 needs to discuss whether to define one single declaration on SCM requirements, or define multiple declarations for each individual SCM test cases.

Proposal 11: RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Conclusion

Proposal 1: RAN4 should first resolve this editor note for the angle scaling for rCDL-C1 and discuss the possible update to the channel model.

Proposal 2: While discussing the possible rCDL-C1 channel model update, RAN4 should ensure there should not be big performance difference while performing the test under difference carrier frequency within FR1.

Proposal 3: Carrier frequency should not be captured as one of the test parameters in the test requirements.

Proposal 4: Cover TDD 30kHz SCS with 40MHz CHBW and discuss whether to also cover FDD 15kHz SCS with 10MHz CHBW.

Observation 1: According to TR38.753, companies’ results for Type I PMI as well as SU-MIMO PDSCH are well-aligned.

Observation 2: Since Rel-16 Type II PMI, for original TDL model, it was hard for companies to align simulation results. RAN4 defines enhanced PMI requirements by selecting the lowest value among the ones companies provided.

Proposal 5: Specify requirements for PMI performance with 8T4R eType II precoding by selecting the lowest value among the ones companies provided.

Proposal 6: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Proposal 7: Reuse the simulation assumptions/test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Proposal 8: Use the same test metric as for TDL channel model for PDSCH demodulation, i.e., SNR @ 70% max TP.

Observation 3: For 8T8R rank8, the performance difference between the 2 codewords is at the range of 5.9~11.8dB as captured in TR38.753.

Proposal 9: For 8T8R rank8, define SNR requirement values for each codeword separately.

Proposal 10: Introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration. RAN4 needs to discuss whether to define one single declaration on SCM requirements, or define multiple declarations for each individual SCM test cases.

Proposal 11: RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Reference

RP-252957, WID: NR demodulation performance: Phase 6, Apple, RAN#109, Sep 2025