3GPP TSG-RAN WG4 # 116-bis	R4-2513143

Prague, Czech Republic, 13th October 2025 – 17th October 2025

Agenda item:	8.8

Source:	MediaTek Inc.

Title:	Discussion on 6G RAN4 driven non-AI demod topics

Document for:	Discussion

# Introduction

This contribution provides an overview of our recommendations on the technology directions and pain points to address for 6G RAN4 driven non-AI demodulation and CSI topics to fulfil the 6G demodulation and performance related study objectives.

# Discussion

Study on 6G radio work item description [1] gives high level objective for demodulation and performance aspects.

- Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

- Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

When evaluating existing 5G demodulation and CSI reporting requirements, our high‑level observation is that test configurations are usually set to minimum settings to maximise applicability across devices under test (DUTs). Furthermore, tests are designed with a specific scope; for example, CSI reporting requirements are intended to be decoupled from demodulation performance. These principles verify basic functionalities but result in unnecessarily complicated test procedures. More importantly, they are not necessarily a good indicator of overall performance because they rely on minimum‑configuration testing within a limited scope.

Our high‑level objective for the 6G demodulation and CSI reporting requirements study is to challenge the traditional testing approach. In our view, we should define requirements that better represent the real‑world performance of DUTs. In real deployments, overall radio performance is a combination of demodulation performance and CSI reporting performance, with additional impact from transmitter behaviour. Therefore, rather than limiting the test scope, we encourage greater use of integrated tests that jointly assess demodulation and CSI reporting under scenarios that better reflect real‑world radio conditions and configurations. More specifically, we propose not to decouple CSI reporting performance from demodulation performance. In 4G and 5G, this decoupling principle has resulted in cumbersome testing procedures that have little or no connection to real-world scenarios.

Observation #1: In real-world scenarios, link performance is the result of CSI reporting and demodulation operating jointly.

Proposal #1: In 6G testing requirements, we propose not to decouple CSI reporting performance from demodulation performance.

In the following sections we discuss several different demodulation and CSI testing related aspects with observations and proposals for study issues.

## 2.1 General issues

### 2.1.1 Receiver assumptions

In 5G UE downlink demodulation and CSI requirements the default baseline receiver is assumed as MMSE linear receiver. Additionally, it can be assumed in include noise whitening capability (IRC). In TR38.878 [2], where study of advanced receiver to cancel inter-user interference for MU-MIMO was documented, more advanced receiver types like E-MMSE-IRC and R-ML are also documented.

Observation #2: 5G UE downlink demodulation and CSI requirements the default baseline receiver is assumed as MMSE linear receiver.

Observation #3: 5G UE downlink demodulation and CSI requirements can assume other advanced receiver types for specific tests.

In scenarios where the form factor of a device is not a limiting factor, such as Fixed Wireless Access (FWA) devices, it is feasible to equip the device with multiple antennas featuring low correlation to support high number of MIMO layers. However, as the number of MIMO layers increases, the UE processing complexity and power consumption rise significantly, posing challenges for efficient operation. Recognizing this, in Rel-18 and Rel-19, RAN4 has already considered the concept of an 8Rx FWA simplified receiver, which leverages two joint 4Rx MIMO detectors within an 8Rx device to reduce complexity. Building on this approach, in 6G, RAN4 could also explore the requirements of low-complexity receiver designs that balance performance with reduced UE processing demands and power consumption.

Observation #4: When the UE is equipped with large number of antennas, it is possible to use several small number Rx MIMO detectors for data processing, e.g., 8Rx UE can use two joint 4Rx MIMO detectors instead of one 8Rx MIMO detector for data processing.

We propose to study what would be most feasible and practical receiver assumption for 6G requirements. We should check if we could assume more advanced receiver as a new baseline receiver for UE and BS requirements.

Proposal #2: We propose to study what would be most feasible and practical receiver assumption.

### 2.1.2 EVM assumptions

There has been discussion to revisit simulation assumption of EVM in several 5G work items.

5G TX EVM UE downlink simulation assumptions are listed in Table 2.1.2-1. 5G EVM requirements for BS type 1-C and BS type 1-H carrier in TS38.104 [3] are listed in Table 2.1.2-2 (Table 6.5.2.2-1 in TS38.104 [3]). We can see that 5G UE downlink simulation assumptions are not well aligned with 5G BS EVM requirements.

Observation #5: 5G TX EVM UE downlink simulation assumptions are not well aligned with 5G BS EVM requirements.

Furthermore, as discussed in Views on 6G BS Tx EVM requirements paper [4], modulation specific EVM requirement is fine only when the number of MIMO layers is not very high. However, when the number of MIMO layers is higher, such an approach becomes problematic. With higher MIMO ranks, even with the same modulation order, the required SNR will increase correspondingly. However, if we keep the same EVM value, the maximum achievable SNR will be upper bounded, as well as the throughput.

Observation #6: With the EVM values based on current TS 38.104, the max throughput cannot be achieved with more MIMO layers, and the degradation compared to EVM 0% can be huge in some scenarios.

We should study what EVM simulations assumptions should be used in demodulation and CSI requirements.

Proposal #3: Study what EVM simulations assumptions should be used in demodulation and CSI requirements.

Table 2.1.2-1: UE downlink simulation assumptions of EVM

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 6 % |
| 16QAM | 6 % |
| 64QAM | 6 % |
| 256QAM | 3 % |
| 1024QAM | 2.5 % |

Table 2.1.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM | 8 % |
| 256QAM | 3.5 % |
| 1024QAM | 2.5 %1 2.8 %2 |
| Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz. |  |

## 2.2 Channel model

6G channel model preparations have already started in RAN4 Rel19 spatial channel model study item, and its outcome is documented in TR38.753 [5]. Additionally, RAN1 has updated TR38.901 [6] with 6G related updates. Also, RAN4 is going to continue spatial channel model study in Rel20 with extended scope from Rel19 work.

RAN4 TR38.753 considers following channel models.

- Legacy TDL (The current baseline channel for all 5G requirements in TS38.101)

- Extended TDL (Multi-cluster variant of legacy TDL)

- CDL (TR38.901 based link-level CDL with randomness reduction for better alignment)

Observation #7: After Rel-19 study RAN4 has 3 channel model options available for 6G requirements.

The current TR38.753 considers all 3 channel model options feasible and can be aligned but do not give recommendations for specific use cases. We propose to further check in which use cases each channel model should be used in 6G requirements.

Proposal #4: We propose to check in which use cases each channel model should be used in 6G requirements.

The legacy TDL channel antenna correlation definition is not well suited for large antenna arrays. If legacy TDL would be preferred for some 6G test use cases we would prefer to further study if usual definition of correlation values could be revisited. With TDL based approaches it would be better to define (α, β, γ) to match test case instead of fixed "medium" and "high" in several legacy requirements.

Observation #8: The legacy TDL channel antenna correlation definition is not well suited for large antenna arrays.

Proposal #5: We propose to further study how to define antenna correlation values for different antenna arrays.

## 2.3 Demodulation test methodologies

### 2.3.1 General test metrics

In 5G UE downlink demodulation requirements the default test point is defined as 70% fraction of maximum throughput. In addition, there is rare exception of 30% fraction of maximum throughput to test HARQ. UE reporting in link adaptation is defined to target less than 10% BLER operation point corresponding to more than 90% fraction of maximum throughput.

Observation #9: There is misalignment of operation points in demodulation requirements and actual system target.

Common claim related to demodulation requirement test point is that it is easier to align 70% fraction of maximum throughput than 90%. This claim is usually observed in the legacy demodulation test assumptions with legacy TDL channel and random precoding. For 6G requirements there will be 3 channel model options available that may impact earlier alignment observations. The spatial channel model study item has been working to find more realistic channel models that correspond to physical environments seen in the field. We believe it does not make sense to combine realistic channel models with unrealistic transmission schemes. Therefore, instead of random precoding more realistic and practical precoding should be applied.

Observation #10: New channel models and precoding methodologies may impact demodulation alignment.

Proposal #6: We propose to study if test point of demodulation requirements could be better aligned with realistic transmission scenarios and operation points.

### 2.3.2 Demodulation requirements with fixed Rank and MCS

The legacy demodulation requirements use test assumptions with legacy TDL channel and random precoding. Especially in a case of spatial channel models random precoding may not make so much sense but more practical and realistic precoding should be applied. We would have several options to achieve more practical precoding.

- Fixed precoding that matches spatial properties of the chosen channel (assumes spatial channel model)

  - Finding good precoder case by case is additional work in test definition.

- PMI reporting

  - Mixes demodulation test with partial CSI feedback

- SRS based precoding

  - New methodology in RAN4 and applicable only in TDD

  - Requires baseline definition of precoding processing in TE to enable aligned simulation assumptions.

Observation #11: There are several methods to apply more realistic precoding in demodulation tests with spatial channel models.

Observation #12: SRS based precoding would be new methodology in RAN4.

Observation #13: SRS based precoding would require baseline definition of precoding processing in TE to enable aligned simulation assumptions.

We propose to study different precoding options for more realistic test scenarios.

Proposal #7: We propose to study different precoding options.

Proposal #8: We propose to define baseline precoding procedure in TE to enable aligned simulation assumptions.

### 2.3.3 Demodulation requirements with link adaptation

In the legacy demodulation tests, there are few demodulation tests with full link adaptation in addition to fixed reference channel testing. In these tests number of layers, precoding, and MCS are selected based on UE CSI reports. In 5G test coverage of these tests is very limited because of 2 antenna port configuration causing maximum of number of layers to be just 2 and precoder selection is trivial.

Observation #14: In 5G there is very limited set of demodulation requirements with link adaptation.

The existing demodulation requirements with link adaptation utilize HARQ for retransmission for more practical system behaviour. Also, as TE is strictly following reported CQI, that should correspond to 10% BLER, we can assume operation point to be close to normal system operation point. However, in the real system network do not fully rely on UE CQI reports but has outer loop link adaptation (OLLA). As part of 5G PDSCH absolute physical layer throughput requirements discussions some companies provided reference baseline OLLA implementation.

Observation #15: In 5G demodulation requirements with link adaptation missed OLLA.

Observation #16: One possible reference baseline OLLA implementation has been shared in 5G WI.

We see demodulation requirements with link adaptation as more realistic and complete measure of performance. Therefore, we see that implementing such tests in larger scale in more complete set of configurations could help to reduce the number of simple tests with limited scope of only demodulation or CSI reporting. We propose to study extending scope of demodulation tests with link adaptation for wider use. Also, we propose to study adding OLLA for improved realism. Earlier OLLA implementation proposal [7] can be used as a starting point for TE implementation.

Proposal #9: We propose to study extending scope of demodulation tests with link adaptation.

Proposal #10: We propose to study demodulation tests with link adaptation replacing simple demodulation or CSI tests.

Proposal #11: We propose to study adding OLLA to demodulation tests with link adaptation.

## 2.4 CSI test methodologies

In this section, we discuss CSI test methodologies corresponding to CQI, PMI, and RI reporting in 5G. We recognise that the 6G CSI reporting framework may ultimately diverge from the 5G framework, so some observations and proposals grounded in the legacy framework may be superseded. Nonetheless, we can adopt the legacy framework as a baseline until the new framework is finalised.

### 2.4.1 CQI reporting requirement related test methodologies

In the legacy CQI reporting requirements test metrics are defined as

- Testing under AWGN conditions

  - The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90 % of the time.

  - If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to BLER target, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than BLER target. If the PDSCH BLER using the transport format indicated by the median CQI is greater than BLER target, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to BLER target.

- Testing under fading conditions (assuming wideband reporting)

  - A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Reference Table;

  - The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Reference Table;

  - When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

The test procedure in AWGN and fading conditions assume two sets of predefined SNR points with 1dB difference. UE is assumed to pass requirements at least in one SNR in both SNR sets. Also, the test procedure measures median CQI as the first step and with measured median CQI + 1 evaluates BLER in the second step.

Observation #17: CQI reporting requirements use 2-step approach to evaluate all pass conditions.

We believe the current test methodology for CQI reporting is unnecessarily complicated. Therefore, we propose to consider a simpler and more realistic 1-step approach. We believe it suffices to test only in follow-CQI mode in a number of SNR points and set requirements for throughput and BLER.  More specifically, the pass criterion could be that a minimum throughput requirement is achieved so that BLER remains within allowed limits. The allowed BLER limits could depend on the channel. For example, in AWGN, BLER should fall between 0% and 10%, while in fast fading BLER could fall between 2% and 20%. Alternatively, lower limit for BLER could even be omitted since the throughput requirement should be impossible to reach by reporting too low CQI values.

Proposal #12: We propose streamlining CQI reporting testing into 1-step approach and setting requirements in terms of throughput/SNR and BLER limits.

Finally, we propose to study the necessity of dedicated CQI reporting requirements in addition to combined demodulation and link adaptation testing.

Proposal #13: We propose to study the necessity of CQI reporting requirements in addition to combined demodulation and link adaptation testing.

### 2.4.2 PMI reporting requirement related test methodologies

In the legacy PMI reporting, test metrics are defined as

- γ = tfollow / trnd

where in the definition of throughput ratios γ, tfollow is 90 % of the maximum throughput obtained at SNRUE using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRUE with random precoding. In addition, frequency domain granularity of random precoding depends on codebook under testing, and random precoding used always Single Panel Type I codebook.

We see that the intention to define throughput ratio γ as test metric was to exclude demodulation performance in PMI testing. On the other hand, when the channel has high spatial selectivity, demodulation with random precoding may perform poorly in SNRUE operation point that can make throughput ratio γ unreliable test metric.

Observation #18: Likely original intention of throughput ratio γ test metric was to make testing independent of demodulation performance.

Observation #19: Throughput ratio γ can be unreliable test metric in highly spatially selective channels.

We are wondering if throughput ratio γ based test metric is the best method to test precoding reporting. We propose to study alternative test metrics like directly using 90% of the maximum throughput obtained at SNRUE as the test metric. It would also shorten the testing time when random precoding reference would not be needed.

Proposal #14: We propose simplifying the PMI reporting testing process and setting requirements directly in terms of throughput/SNR instead of measuring γ.

Finally, we propose to study the necessity of dedicated PMI reporting requirements in addition to combined demodulation and link adaptation testing.

Proposal #15: We propose to study the necessity of PMI reporting requirements in addition to combined demodulation and link adaptation testing.

### 2.4.3 RI reporting requirement related test methodologies

In the legacy RI reporting requirements test metrics are defined as

- The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ γ1;

- The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ γ2;

There are just four 4RX RI requirements for FDD and TDD each and γ requirements are as {0.9, 1.05, 0.9, 0.9} per test. This means that in 3 tests out of 4 rank reporting can perform worse than fixed rank and still pass the test. This testing methodology does not seem very credible and beneficial. Also, 3 or out 4 tests use 2 AP Single Panel Type I codebook, and the last test uses 4 AP Single Panel Type I codebook. Maximum number of layers tested is just 2.

Observation #20: There are very few legacy rank reporting tests.

Observation #21: Maximum number of layers tested is just 2 in legacy rank reporting tests.

Observation #22: Test requirements are very loose in legacy rank reporting tests.

We propose to study alternative test metrics and test methodologies to define rank reporting requirements.

Proposal #16: We propose to study RI reporting requirements test metrics and test methodologies.

Finally, we propose to study the necessity of dedicated RI reporting requirements in addition to combined demodulation and link adaptation testing.

Proposal #17: We propose to study the necessity of RI reporting requirements in addition to combined demodulation and link adaptation testing.

# Summary

In this paper we provided our views to 6G RAN4 driven non-AI demodulation and CSI topics. The following observations and proposals are made.

Observation #1: In real-world scenarios, link performance is the result of CSI reporting and demodulation operating jointly.

Proposal #1: In 6G testing requirements, we propose not to decouple CSI reporting performance from demodulation performance.

Observation #2: 5G UE downlink demodulation and CSI requirements the default baseline receiver is assumed as MMSE linear receiver.

Observation #3: 5G UE downlink demodulation and CSI requirements can assume other advanced receiver types for specific tests.

Observation #4: When the UE is equipped with large number of antennas, it is possible to use several small number Rx MIMO detectors for data processing, e.g., 8Rx UE can use two joint 4Rx MIMO detectors instead of one 8Rx MIMO detector for data processing.

Proposal #2: We propose to study what would be most feasible and practical receiver assumption.

Observation #5: 5G TX EVM UE downlink simulation assumptions are not well aligned with 5G BS EVM requirements.

Observation #6: With the EVM values based on current TS 38.104, the max throughput cannot be achieved with more MIMO layers, and the degradation compared to EVM 0% can be huge in some scenarios.

Proposal #3: Study what EVM simulations assumptions should be used in demodulation and CSI requirements.

Observation #7: After Rel-19 study RAN4 has 3 channel model options available for 6G requirements.

Proposal #4: We propose to check in which use cases each channel model should be used in 6G requirements.

Observation #8: The legacy TDL channel antenna correlation definition is not well suited for large antenna arrays.

Proposal #5: We propose to further study how to define antenna correlation values for different antenna arrays.

Observation #9: There is misalignment of operation points in demodulation requirements and actual system target.

Observation #10: New channel models and precoding methodologies may impact demodulation alignment.

Proposal #6: We propose to study if test point of demodulation requirements could be better aligned with realistic transmission scenarios and operation points.

Observation #11: There are several methods to apply more realistic precoding in demodulation tests with spatial channel models.

Observation #12: SRS based precoding would be new methodology in RAN4.

Observation #13: SRS based precoding would require baseline definition of precoding processing in TE to enable aligned simulation assumptions.

Proposal #7: We propose to study different precoding options.

Proposal #8: We propose to define baseline precoding procedure in TE to enable aligned simulation assumptions.

Observation #14: In 5G there is very limited set of demodulation requirements with link adaptation.

Observation #15: In 5G demodulation requirements with link adaptation missed OLLA.

Observation #16: One possible reference baseline OLLA implementation has been shared in 5G WI.

Proposal #9: We propose to study extending scope of demodulation tests with link adaptation.

Proposal #10: We propose to study demodulation tests with link adaptation replacing simple demodulation or CSI tests.

Proposal #11: We propose to study adding OLLA to demodulation tests with link adaptation.

Observation #17: CQI reporting requirements use 2-step approach to evaluate all pass conditions.

Proposal #12: We propose streamlining CQI reporting testing into 1-step approach and setting requirements in terms of throughput/SNR and BLER limits.

Proposal #13: We propose to study the necessity of CQI reporting requirements in addition to combined demodulation and link adaptation testing.

Observation #18: Likely original intention of throughput ratio γ test metric was to make testing independent of demodulation performance.

Observation #19: Throughput ratio γ can be unreliable test metric in highly spatially selective channels.

Proposal #14: We propose simplifying the PMI reporting testing process and setting requirements directly in terms of throughput/SNR instead of measuring γ.

Proposal #15: We propose to study the necessity of PMI reporting requirements in addition to combined demodulation and link adaptation testing.

Observation #20: There are very few legacy rank reporting tests.

Observation #21: Maximum number of layers tested is just 2 in legacy rank reporting tests.

Observation #22: Test requirements are very loose in legacy rank reporting tests.

Proposal #16: We propose to study RI reporting requirements test metrics and test methodologies.

Proposal #17: We propose to study the necessity of RI reporting requirements in addition to combined demodulation and link adaptation testing.

# References

[1] RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO

[2] TR38.878, “NR demodulation performance evolution (Release 18), V18.4.0 (2024-09)”

[3] TS38.104, “Base Station (BS) radio transmission and reception (Release 18), V18.10.0 (2025-06)”

[4] R4-2513076, “Views on 6G BS Tx EVM requirements”, MediaTek inc.

[5] TR38.753, “Study on spatial channel model for demodulation performance Requirements for NR (Release 19), V2.0.0 (2025-09)”

[6] TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19), V19.0.0 (2025-06)”

[7] R4-2300703, “On Absolute physical layer throughput requirements with link adaptation”, Nokia, Nokia Shanghai Bell