3GPP TSG-RAN WG4 Meeting #116bis                                                                                                      R4-2513154

Prague, CZ, 13rd –17th October, 2025

Source: 	CATT

Title: 	Discussion on SAN demodulation performance requirements for NR NTN Ku-band

Agenda item:	5.10.3

Document for:	Approval

- Introduction

In this contribution, we provide our views on SAN demodulation performance requirements for Ku-band for NR NTN.

- Discussion

Referring to the demodulation requirement in TS 38.108, PUSCH, PRACH, and PUCCH demodulation requirement are defined for FR1-NTN and FR2-NTN. So, for Ku-band NTN, the PUSCH, PRACH, and PUCCH demodulation requirement shall be defined.

2.1 PUSCH

Propagation conditions and correlation matrix for PUSCH

The NTN-TDLA100-200 Low and NTN-TDLC5-200 Low were defined for FR1-NTN, and NTN-TDLC5-1200 Low was defined for FR2-NTN. The channel parameters for FR1-NTN and FR2-NTN in TS 398.108 are copied as below:

Table D.2.2-1: Channel model parameters for FR1-NTN

| Combination name | Tapped delay line model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-200 | NTN-TDLA100 | 200 Hz |
| NTN-TDLC5-200 | NTN-TDLC5 | 200 Hz |

Table D.2.2-2: Channel model parameters for FR2-NTN

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLC5-1200 | NTN-TDLC5 | 1200 Hz |

As above, For Ku-band, since it was defined as FR1-NTN Ku-band and FR2-NTN Ku-band, so for tapped delay line model, the NTN-TDLC5 used for FR1-NTN and FR2-NTN can be reused for FR1-NTN Ku-band and FR2-NTN Ku-band. Since for FR1-NTN, the frequency is about 2GHz, and for FR2-NTN, the frequency is about 30GHz, and 200Hz maximum Doppler frequency is used for 2GHz, and 1200Hz maximum Doppler frequency is used for 30GHz, so for Ku-band 14.5GHz, maximum Doppler frequency for Ku-band 14.5GHz can be set to 650Hz as starting point. So it is sufficient to define NTN-TDLC5-650 low as propagation conditions and correlation matrix for PUSCH  for FR1-NTN Ku-band and FR2-NTN Ku-band.

Proposal 1: Define NTN-TDLC5-650 low for PUSCH for FR1-NTN Ku-band and FR2-NTN Ku-band as starting point.

CBW and SCS

The SAN channel bandwidths and SCS per operating band for Ku-band in TS 38.108 are copied as below:

Table 5.3.5-1: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
| n248 | 15 |  |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 15 |  |  | 10 | 15 |  | 25 |  | 35 | 50 |  |  |
|  | 30 |  |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.5-2: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

5MHz CBW/15kHz SCS and 10MHz CBW/30kHz SCS were defined for FR1-NTN PUSCH, and 50MHz CBW/120kHz SCS was defined for FR2-NTN PUSCH. As above Table 5.3.5-1, although the 5MHz CBW/15kHz SCS was not defined for FR1-NTN Ku-band, we can still define 5MHz CBW/15kHz SCS for FR1-NTN Ku-band, and applicability of requirements for different channel bandwidths for PUSCH can solve this issue. So, CBW and SCS for FR1-NTN can be reused for FR1-NTN Ku-band, and CBW and SCS for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 2: CBW and SCS for PUSCH for FR1-NTN can be reused for FR1-NTN Ku-band, and CBW and SCS for PUSCH for FR2-NTN can be reused for FR2-NTN Ku-band.

Test parameters for PUSCH demodulation

From our understanding, except the channel model, the test parameters for PUSCH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PUSCH for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 3: The test parameters for PUSCH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PUSCH for FR2-NTN can be reused for FR2-NTN Ku-band.

2.2 PUCCH

Propagation conditions and correlation matrix for PUCCH

As above discussion for propagation conditions and correlation matrix for PUSCH for Ku-band, we think the same channel model can be used for PUSCH and PUCCH for Ku-band. So we propose,

Proposal 4: Define NTN-TDLC5-650 low for PUCCH for FR1-NTN Ku-band and FR2-NTN Ku-band as starting point.

PUCCH format

The PUCCH formats for FR1-NTN can be reused for FR1-NTN Ku-band, and the PUCCH formats for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 5: The PUCCH formats for FR1-NTN can be reused for FR1-NTN Ku-band, and the PUCCH formats for FR2-NTN can be reused for FR2-NTN Ku-band.

Test Parameters for PUCCH.

From our understanding, except the channel model, the test parameters for PUCCH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PUCCH for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 6: The test parameters for PUCCH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PUCCH for FR2-NTN can be reused for FR2-NTN Ku-band.

2.3 PRACH

Propagation conditions and correlation matrix, frequency offset for PRACH

For FR1-NTN, the following propagation conditions and correlation matrix, frequency offset was defined for PRACH.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| AWGN | 0 |
| NTN-TDLA100-200 Low | 200 Hz |

For FR2-NTN, the following propagation conditions and correlation matrix, frequency offset was defined for PRACH.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| NTN-TDLC5-1200 Low | 3000 Hz |

As above discussion for propagation conditions and correlation matrix for PUSCH for Ku-band, we think that the following propagation conditions and correlation matrix, frequency offset can be defined for PRACH for FR1-NTN Ku-band.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| AWGN | 0 |
| NTN-TDLC5-650 Low | 1500 Hz |

The following propagation conditions and correlation matrix, frequency offset can be defined for PRACH for FR2-NTN Ku-band.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| NTN-TDLC5-650 Low | 1500 Hz |

Proposal 7: Define the following propagation conditions and correlation matrix, frequency offset for PRACH for FR1-NTN Ku-band as starting point.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| AWGN | 0 |
| NTN-TDLC5-650 Low | 1500 Hz |

And define the following propagation conditions and correlation matrix, frequency offset for PRACH for FR2-NTN Ku-band as starting point.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| NTN-TDLC5-650 Low | 1500 Hz |

PRACH preamble Burst format

The PRACH preamble Burst format for FR1-NTN can be reused for FR1-NTN Ku-band, and the PRACH preamble Burst format for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 8: The PRACH preamble Burst format for FR1-NTN can be reused for FR1-NTN Ku-band, and the PRACH preamble Burst format for FR2-NTN can be reused for FR2-NTN Ku-band.

Test Parameters for PRACH.

From our understanding, except the channel model, the test parameters for PRACH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PRACH for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 9: The test parameters for PRACH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PRACH for FR2-NTN can be reused for FR2-NTN Ku-band.

- Conclusion

This contribution provides analysis on SAN demodulation for NR NTN Ku-band, and the following proposal is concluded as follows:

Proposal 1: Define NTN-TDLC5-650 low for PUSCH for FR1-NTN Ku-band and FR2-NTN Ku-band as starting point.

Proposal 2: CBW and SCS for PUSCH for FR1-NTN can be reused for FR1-NTN Ku-band, and CBW and SCS for PUSCH for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 3: The test parameters for PUSCH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PUSCH for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 4: Define NTN-TDLC5-650 low for PUCCH for FR1-NTN Ku-band and FR2-NTN Ku-band as starting point.

Proposal 5: The PUCCH formats for FR1-NTN can be reused for FR1-NTN Ku-band, and the PUCCH formats for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 6: The test parameters for PUCCH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PUCCH for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 7: Define the following propagation conditions and correlation matrix, frequency offset for PRACH for FR1-NTN Ku-band as starting point.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| AWGN | 0 |
| NTN-TDLC5-650 Low | 1500 Hz |

And define the following propagation conditions and correlation matrix, frequency offset for PRACH for FR2-NTN Ku-band as starting point.

| Propagation conditions and correlation matrix (annex D) | Frequency offset |
| --- | --- |
| NTN-TDLC5-650 Low | 1500 Hz |

Proposal 8: The PRACH preamble Burst format for FR1-NTN can be reused for FR1-NTN Ku-band, and the PRACH preamble Burst format for FR2-NTN can be reused for FR2-NTN Ku-band.

Proposal 9: The test parameters for PRACH for FR1-NTN can be reused for FR1-NTN Ku-band, and the test parameters for PRACH for FR2-NTN can be reused for FR2-NTN Ku-band.

- References

- RP-250799, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107