3GPP TSG RAN WG4 Meeting #116bis		R4-2513168

Prague, Czech,  13 – 17 October 2025

Title: 	Discussion on RRM impacts on non-concurrent n5 DL and n8 UL in Rel-19 to enable UL CA_n5-n8

Agenda Item:	5.9.3

Source: 	CATT

Document for:	Approval

1 Background

In the RAN4#116 meeting, an agreement was captured in the chair note to discuss the RRM impacts on the solution which can enable UL CA_n5-n8.

Agreement: For CA_n5-n8, in RAN4#116bis, an agenda item in RRM session will be introduced to discuss and determine if there is RRM impact. If RRM impact is identified and agreed, this topic will not be further pursued. If not, the discussion will continue in main session under R19 maintenance. - When n5DL is operated, n8UL RF path needs to be switched off and the same for the other way around. - Proponent companies are encouraged to bring detailed anlysis, including the possible gain/loss. Before the conclusion on RRM impact is made, no further discussion is needed in the main session.

In this paper, we’d like to further elaborate our solution on enabling UL CA_n5-n8 and analyse whether there is RRM impact on this solution. In the end, we’d like to provide the investigation about the gain of the proposed solution.

2 Discussion

2.1 The elaboration about the proposed solution

Generally, the RF implementation could be achieved for different RRC configurations as below.

|  | RRC configuration | RFFE configuration |
| --- | --- | --- |
| Case 1 | 2UL/2DL | - #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx - #B: n5-UL:Tx, n8-UL:Tx, n8-DL:Rx |
| Case 2 | 1UL/2DL (like SDL) | - #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx |

Based on companies’ contributions, there are two assumed RF implementations for 2UL/2DL RRC configuration at least.

RF Implementation 1: the RFFE configuration can be switched between RFFE#A and RFFE#B under RRC configuration 2UL/2DL.

RF Implementation 2: the RFFE configuration can be configured as RFFE#B under RRC configuration 2UL/2DL.

The proposed solution needs UE being capable of RF Implementation 1. An example is illustrated without RRM impacts as follows to enable UL CA_n5-n8 and balance the time-frequency domain resources between UL and DL:

| RRC configuration case 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RF Conf. | CBW (MHz) | 10 |  | 15 |  |
|  | Slot Index | PCell |  | SCell |  |
|  |  | UL band n5 | DL band n5 | UL band n8 | DL band n8 |
| RFFE#A (k+1 Slots) | Slot N | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+1 | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+2 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+3 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+4 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+k-1 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+k | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
| RF Switching | Slot N+k+1 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |
| RFFE#B (17-k Slots) | Slot N+k+2 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+15 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+16 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+17 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+18 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
| RF Switching | Slot N+19 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |

Step 1: Access n5 as PCell

Step 2: RRC reconfiguration to Case 2: Add n8 with only n8-DL as SCell  UL Tx in SCell via n5-UL

  - RFFE configuration is #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx

Step 3: RRC reconfiguration to Case 1: PCell remains at n5, reconfigure SCell n8 by adding n8-UL

  - Freedom of RFFE configuration

    - Alt.#1: Not changed, remaining as #A: (n5-UL: Tx, n5-DL:Rx, n8-DL:Rx)

    - Alt#2: Or reconfigured to #B: (n5-UL:Tx, n8-UL:Tx, n8-DL:Rx)

Step 4: When n8-UL needs to be scheduled from Slot #N+k+2 to Slot #N+m (where m=18 for the illustration),

  - NW to choose a slot #N+k or before slot #N+k, where DCI issued to the UE that n8-UL transmission happens at slot #N+k+2

    - k is chosen

      - one slot switching period is large enough so the UE can make successful RFFE reconfiguration to Alt. #2

      - There will be no SSB/PDCCH/CSI-RS on n5-DL to be received by the UE between Slot #N+k+1 and #N+m+1 (where m=18 for the illustration)

    - The DCI also schedules n5-UL between Slot #N+k+1 and #N+m+1 (where m=18 for the illustration) via 1 TB mapping to multiple slots via features:

      - Repetitive (Rel-15)

      - Non-repetitive (Rel-17)

Hence n5-UL transmission during this period is not impacted.

  - UE to complete reconfiguration of RFFE between Slot #N-k and #N (k is chosen to guarantee that no n5-DL reception is required)

  - UE to make UL transmission at n8-UL at Slot #N+k+2 until Slot #N+m (where m=18 for the illustration).

  - Once UE complete n8-UL transmission, reconfigure RFFE from #B to #A immediately with one slot switching period.

  - There is no uplink transmission on n8-UL for the UE between Slot #N and Slot #N+k

Step 5: Repeat Step 4 if new n8-UL transmission is required.

NOTES:

- PCell is always n5, and n8 always serves as SCell.

- SCell is active

  - No SCell activation issue

- PCell and SCell are collocated, and have the same frame timing

  - No UL timing issue

- Only RFFE Alt. #A works (i.e., when RRC reconfigured from Case 2 to Case 1, RFFE configuration does not change), otherwise, NW does not configure the UE with concurrent n5-DL and n8-UL (scheduling restriction)

- UE capability required

  - NW needs to know the UE capability : (1) No RFFE reconfiguration when adding n8-UL to SCell; (2) RFFE reconfigured back to 1UL/2DL when finishing UL transmission on n8-UL.

Besides the UE capability, UE just follows NW instructions, no need for change of behaviours, signalling, etc., and no additional requirements.

It’s noted that UE is not expected to transmit in the UL band n8 when any reception is needed in the DL band n5.

Observation 1: No SCell activation issue, No UL timing issue and no impacts on SSB/CSI-RS measurement are observed.

Proposal 1: RRM session conclude that there is no RRM impact on the proposed solution which can enable UL CA_n5-n8.

2.2 The Gain investigation for this solution

The Time-Frequency Scheduling Occasion are summarized below for RRC configuration case 1 and case 2.

| RRC configuration case 1 (Where k=9) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RF Conf. | CBW (MHz) | 10 |  | 15 |  |
|  | Slot Index | PCell |  | SCell |  |
|  |  | UL band n5 | DL band n5 | UL band n8 | DL band n8 |
| RFFE#A (10 Slots) | Slot N | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+1 | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+2 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+3 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+4 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+5 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+6 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+7 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+8 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
| RF Switching | Slot N+9 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |
| RFFE#B (8 Slots) | Slot N+10 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+11 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+12 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+13 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+14 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+15 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+16 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+17 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+18 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
| RF Switching | Slot N+19 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |
| Time Domain Scheduling Occasion | Unit / (ms) | 20 | 8 | 8 | 18 |
| Time-Frequency Scheduling Occasion | Unit / (MHz*ms) | 200 | 80 | 120 | 270 |
| Total TFSO (UL+DL) | Unit / (MHz*ms) | 670 |  |  |  |
| Total TFSO (UL) | Unit / (MHz*ms) | 320 |  |  |  |
| Total TFSO (DL) | Unit / (MHz*ms) | 350 |  |  |  |

| RRC configuration case 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RF Conf. | CBW (MHz) | 10 |  | 15 |  |
|  | Slot Index | PCell |  | SCell |  |
|  |  | UL band n5 | DL band n5 | UL band n8 | DL band n8 |
| RFFE#A | Slot N | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+1 | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+2 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+3 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+4 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+5 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+6 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+7 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+8 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+9 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+10 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+11 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+12 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+13 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+14 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+15 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+16 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+17 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+18 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+19 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
| Time Domain Scheduling Occasion | Unit / (ms) | 20 | 18 |  | 18 |
| Time-Frequency Scheduling Occasion | Unit / (MHz*ms) | 200 | 180 |  | 270 |
| Total TFSO (UL+DL) | Unit / (MHz*ms) | 650 |  |  |  |
| Total TFSO (UL) | Unit / (MHz*ms) | 200 |  |  |  |
| Total TFSO (DL) | Unit / (MHz*ms) | 450 |  |  |  |

For the exemplary two cases, it’s observed that the total Time-Frequency Scheduling Occasion for the RRC configuration case 2 (1UL/2DL: 650 MHz*ms) is less than the RRC configuration case 1(2UL/2DL: 670 MHz*ms). The system gain is observed for the proposed solution. In addition, the proposed solution can help enable UL CA_n5-n8 and balance the UL and DL time-frequency for one UE which can support UL CA_n5-n8 with non-concurrent n5 downlink and n8 uplink.

Observation 2: For the exemplary two cases, it’s observed that the total Time-Frequency Scheduling Occasion for the RRC configuration case 2 (1UL/2DL: 650 MHz*ms) is less than the RRC configuration case 1(2UL/2DL: 670 MHz*ms). The system gain is observed for the proposed solution (RRC configuration case 1).

Observation 3: In addition, the proposed solution can help enable UL CA_n5-n8 and balance the UL and DL time-frequency for one UE which can support UL CA_n5-n8 with non-concurrent n5 downlink and n8 uplink.

Proposal 2: The gain is investigated and observed for the proposed solution.

3 Summary

Observation 1: No SCell activation issue, No UL timing issue and no impacts on SSB/CSI-RS measurement are observed.

Proposal 1: RRM session conclude that there is no RRM impact on the proposed solution which can enable UL CA_n5-n8.

Observation 2: For the exemplary two cases, it’s observed that the total Time-Frequency Scheduling Occasion for the RRC configuration case 2 (1UL/2DL: 650 MHz*ms) is less than the RRC configuration case 1(2UL/2DL: 670 MHz*ms). The system gain is observed for the proposed solution (RRC configuration case 1).

Observation 3: In addition, the proposed solution can help enable UL CA_n5-n8 and balance the UL and DL time-frequency for one UE which can support UL CA_n5-n8 with non-concurrent n5 downlink and n8 uplink.

Proposal 2: The gain is investigated and observed for the proposed solution.

Reference

[1] R4-2506064, draft reply LS to RAN2 on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, CATT

[2] TR 38.872, Study on enhancement for 700/800/900MHz band combinations, v18.4.0

[3] R4-2417125, WF on UL CA_n5-n8, CATT, China Telecom, Huawei, HiSilicon

[4] R4-2420410, LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, CATT

[5] R2-2501572, Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx, CATT

[6] RP-251879, WF for RAN4 led REL-20 topics, RAN4 Chair (Apple)

[7] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei)

[8] R4-2507910, WF on simultaneous Rx/Tx requirements, Huawei

[9] R4-2511850, WF on CA_n5_n8, CATT, Huawei

[10] RP-242532, “Motivation for a new work item on low band carrier aggregation via switching in Rel-19”, AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, HiSilicon, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Samsung, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE

[11] R4-2502840, “WF on simultaneous Rx-Tx”, Huawei