3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513199

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on performance requirements for RedCap UEs with NTN

Source: 	CATT

Agenda item:	6.19.3.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, RAN4 has reached good progress in defining the core RRM requirements and all of Draft CRs have been endorsed [1]. In this contribution, we discuss and provide our initial views on the performance requirements for RedCap UEs with NTN based on the agreed core requirements.

- Discussion

  - 2.1 Testing configuration and parameters

For duplex mode in test configuration, considering that only FDD duplex mode was considered for NR NTN and HD-FDD mode has been introduced for RedCap UEs with NTN, so both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

Proposal 1: Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

For SCS and bandwidth in test configuration, we believe that the legacy configurations in NTN can be reused, as RAN4 didn’t define any new test cases for eRedCap UE in Rel-18, so 15 kHz SCS and 10MHz bandwidth can be reused for RedCap UE with FR1-NTN.

Considering that both GSO and NGSO are configured in NTN test cases, so the test cases defined RedCap UE with FR1-NTN also need to consider GSO and NGSO. Also, if UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

According to the above, for example, the following testing configuration can be supported for RedCap UE with FR1-NTN for cell reselection requirements:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Observation 1: RAN4 didn’t define any new test cases for eRedCap UE in Rel-18.

Proposal 2: 15 kHz SCS and 10MHz bandwidth can be configured in test cases.

Proposal 3: Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN.

Proposal 4: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

For the above test configuration, the existing RMC related parameters, Io values, SSB configuration and SMTC configurations can be reused for RedCap UE with FR1-NTN.

Since NCD-SSB and related requirements were not defined in core part, so there is also no need to consider NCD-SSB related configuration in test cases for (e)RedCap UE with NTN.

In addition, RAN4 didn’t define BWP specific serving cell MO and RedCap specific initial UL/DL BWP related core requirements for (e)RedCap UE with FR1-NTN, so there is also no need to consider RedCap specific UL/DL BWP configuration for (e)RedCap UE with NTN.

Proposal 5: For the proposed test configuration, the existing RMC related parameters, Io values, SSB configuration and SMTC configurations can be reused for RedCap UE with FR1-NTN.

Proposal 6: RAN4 not to consider NCD-SSB related configuration in test cases for (e)RedCap UE with NTN.

Proposal 7: RAN4 not to consider RedCap specific UL/DL BWP configuration for (e)RedCap UE with NTN.

Regarding how and where to capture the test cases for 1Rx and 2Rx (e)RedCap UE with NTN, we believe that the existing test cases defined for FR1 NTN in A.14 can be reused as baseline for RedCap UEs with FR1-NTN, and RAN4 will further adapt the test cases for RedCap UE, e.g. test configuration, 1Rx/2Rx and so on.

In addition, we prefer to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents, as in core requirements. Also, new sections of test cases with suffix ‘A.X’ and measurement accuracy requirements with suffix ‘10.X’ will be defined for (e)RedCap UE with NTN.

For test principle and setups, the testing related to satellite access and RedCap defined in A.3.35 and A.3.36 can be a reference.

Proposal 8: The existing test cases defined for FR1 NTN in A.14 can be reused as baseline for RedCap UEs with FR1-NTN, and RAN4 will further adapt the test cases for RedCap UE, e.g. test configuration, 1Rx/2Rx and so on.

Proposal 9: RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents, as in core requirements.

Proposal 10: RAN4 to define new sections for test cases with suffix ‘A.X’ and measurement accuracy requirements with suffix ‘10.X’ for (e)RedCap UE with NTN.

Proposal 11: For test principle and setups, the testing related to satellite access and RedCap defined in A.3.35 and A.3.36 can be a reference.

  - 2.2 Test scope

- Test cases to verify the core requirements reusing NTN

For the following core requirements, the existing requirements defined for NTN are reused for (e)RedCap UEs with FR1-NTN bands, and there is also no need to differentiate 1Rx and 2Rx (e)RedCap UEs.

- Enhanced cell reselection measurement

- UE transmit timing

- UE timer accuracy

- Timing Advance

- Active BWP switch delay

- Active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

Therefore, we think that there is also no need to differentiate 1Rx and 2Rx (e)RedCap UEs in the test cases for the above requirements.

For UE timer accuracy and active TCI state switching delay, since RAN4 didn’t define any test cases for the core requirements in NTN, so RAN4 will also not to define the test cases for UE timer accuracy and active TCI state switching delay requirements for RedCap UE with NTN.

Proposal 12: RAN4 to define new test cases without differentiating 1Rx and 2Rx (e)RedCap UEs with NTN for the following requirements:

- Enhanced cell reselection measurement

- UE transmit timing

- Timing Advance

- Active BWP switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

- Test cases to verify the new core requirements

For the following core requirements, legacy NTN requirements can’t be reused directly for (e)RedCap UEs with FR1-NTN bands, so we believe that the new test cases need to be added for (e)RedCap UEs with FR1-NTN bands, and the legacy test cases defined for NTN can be a baseline.

- Cell Re-selection

- Configured Grant based Small Data Transmissions (CG-SDT)

- Random access based Small Data Transmissions (RA-SDT)

- Handover/CHO/ satellite switching with re-synchronization

- RRC Connection Mobility Control

- Radio Link Monitoring

- Link Recovery Procedures

- NR intra-frequency measurements

- NR inter-frequency measurements

- L1-RSRP measurements for Reporting

- NR measurements for positioning

In addition, since RAN4 agreed to defined the requirements for SDT including CG-SDT and RA-SDT, so the new test cases for SDT need to be defined for (e)RedCap UEs with FR1-NTN compared with NTN. However, there were no test cases defined for RA-SDT requirements in RedCap WI, so we also think that only CG-SDT test case need to be defined for (e)RedCap UEs with FR1-NTN.

Proposal 13: RAN4 to define new test cases for the following requirements:

- Cell Re-selection

- Configured Grant based Small Data Transmissions (CG-SDT)

- Handover/CHO/ satellite switching with re-synchronization

- RRC Connection Mobility Control

- Radio Link Monitoring

- Link Recovery Procedures

- NR intra-frequency measurements

- NR inter-frequency measurements

- L1-RSRP measurements for Reporting

- NR measurements for positioning

- Test cases list

To be more specific, we give the following test cases list as a baseline, which are based on existing NTN measurement cases, and CG-SDT test cases have been added. It is worth noting that 1Rx and 2Rx need to be defined separately in some test cases. In order to reduce the redundancy of test cases, RAN4 can further discuss which test cases can be removed based on the following table and the FFS part can be considered to be removed to reduce test effort.

| Test categories/configurations |  | Test cases (as baseline) |
| --- | --- | --- |
| RRC_IDLE state mobility | 4.2E Cell Re-selection for NR RedCap UE with Satellite Access - Intra/inter-f - 1Rx/2Rx - with the feature for enhanced requirements (without differentiating 1Rx and 2Rx) - time-based/location-based - fulfilling low mobility/ not-at-cell edge relaxed measurement criterion - inter-RAT - with TN carrier | - Cell reselection to FR1 intra-frequency NR case for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements - Time-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - FFS: Location-based measurement initiation to FR1 intra-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for 1Rx/2Rx RedCap UE with NTN - FFS: Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements - FFS: Time-based measurement initiation to FR1 inter-frequency cell reselection for 1Rx/2Rx RedCap UE with NTN - Location-based measurement initiation to FR1 inter-frequency NR cell reselection for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion for 1Rx/2Rx RedCap UE with NTN - Cell reselection to FR1 inter-RAT for NR NTN carrier for 1Rx/2Rx RedCap UE with NTN - Cell re-selection to FR1 inter-frequency NR case with TN carrier for 1Rx/2Rx RedCap UE with NTN |
| RRC_IDLE state mobility | 5.5E Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UE with satellite access - 1Rx/2Rx | - NR UE CG-SDT Test in FR1 for 1Rx/2Rx RedCap UE with NTN |
| RRC_CONNECTED state mobility | 6.1F.1 NR SAN Handover - Intra/inter-f - 1Rx/2Rx - RACH-based/RACH-less | - Intra-frequency SAN Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency SAN Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Intra-frequency SAN RACH-less Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN |
|  | 6.1F.2 NR SAN Conditional Handover - Intra/inter-f - 1Rx/2Rx - time-based/distance-based CHO - CHO without L3 measurement criteria | - Intra-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Intra-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency SAN distance-based conditional Handover from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN - FFS: Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN |
|  | 6.1F.3 NR SAN Satellite switching with re-synchronization - 1Rx/2Rx - RACH-based/RACH-less - Hard/soft | - RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN for 1Rx/2Rx RedCap UE with NTN - RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1 for 1Rx/2Rx RedCap UE with NTN for 1Rx/2Rx RedCap UE with NTN |
|  | 6.2E RRC Connection Mobility Control for RedCap UE with Satellite Access - Intra/inter-f - contention based/ non-contention based - only 4-step RA type (follow NTN) | SA: RRC Re-establishment for SAN - Intra-frequency RRC Re-establishment in FR1 for 1Rx/2Rx RedCap UE with NTN - Inter-frequency RRC Re-establishment in FR1 for 1Rx/2Rx RedCap UE with NTN Random Access - 4-step RA type contention based random access test in FR1 for NR standalone for 1Rx/2Rx RedCap UE with NTN - 4-step RA type non-contention based random access test in FR1 for NR standalone for 1Rx/2Rx RedCap UE with NTN RRC Connection Release with Redirection - Redirection from NR in FR1 to NR in FR1 for 1Rx/2Rx RedCap UE with NTN |
| Timing | 7.1E UE transmit timing for Redcap with Satellite Access - without differentiating 1Rx and 2Rx | - NR UE Transmit Timing Test for FR1 |
|  | 7.3E Timing advance for Redcap with Satellite Access - without differentiating 1Rx and 2Rx | - SA FR1 timing advance adjustment accuracy |
| Signalling characteristics | 8.1E Radio Link Monitoring for RedCap UE with Satellite Access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - FFS: Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN - Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS based RLM in DRX mode for 1Rx/2Rx RedCap UE with NTN |
|  | 8.5E Link Recovery Procedures for RedCap UE with Satellite Access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in non-DRX mode for 1Rx/2Rx RedCap UE with NTN - Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS based BFD and LR in DRX mode for 1Rx/2Rx RedCap UE with NTN |
|  | 8.6E Active BWP switch delay for RedCap UE with satellite access - without differentiating 1Rx and 2Rx | - DCI-based and Timer-based Active BWP Switch - RRC-based Active BWP Switch |
|  | 8.13E UE-specific CBW change for RedCap UE with satellite access - without differentiating 1Rx and 2Rx | - UE specific CBW change on PCell in FR1 in non-DRX |
|  | 8.14E Pathloss reference signal switching delay for RedCap UE with satellite access - without differentiating 1Rx and 2Rx | - MAC-CE based pathloss reference signal switch delay |
| Measurement procedure | 9.2E NR intra-frequency measurements for RedCap with satellite access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | - SA event triggered reporting tests without gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests without gap under DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with single measurement gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with FNO concurrent gaps under DRX for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for 1Rx/2Rx RedCap UE with NTN |
|  | 9.3E NR inter-frequency measurements for Redcap UEs with satellite access - 1Rx/2Rx - Other aspect will follow legacy NTN test cases | - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for 1Rx/2Rx RedCap UE with NTN - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overlapping for 1Rx/2Rx RedCap UE with NTN - Event triggered reporting test without gap under non-DRX for 1Rx/2Rx RedCap UE with NTN - Event triggered reporting tests without gap under DRX for 1Rx/2Rx RedCap UE with NTN |
|  | 9.5E L1-RSRP measurements for Reporting for RedCap UE with satellite access - SSB/ CSI-RS based - DRX/non-DRX - 1Rx/2Rx | - SSB based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN - SSB based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement for satellite access when DRX is not used for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement for satellite access when DRX is used for 1Rx/2Rx RedCap UE with NTN |
| Measurement Performance | SS-RSRP/SS-RSRQ/SS-SINR Measurement Performance - Intra/inter-f - 1Rx/2Rx | SS-RSRP - SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN SS-RSRQ - SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN SS-SINR - SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN - SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1Rx/2Rx RedCap UE with NTN |
|  | L1-RSRP measurement Performance for beam reporting - SSB/ CSI-RS based - 1Rx/2Rx | - SSB based L1-RSRP measurement for 1Rx/2Rx RedCap UE with NTN - CSI-RS based L1-RSRP measurement on resource set with repetition off for 1Rx/2Rx RedCap UE with NTN |

Proposal 14: The test cases listed above can be a baseline, and RAN4 can further discuss which test cases can be removed to reduce the redundancy of test cases.

- FFS part can be considered to be removed to reduce test effort.

- Conclusion

This document discussed performance requirements for RedCap UEs with NTN and presented following observation and proposals:

Proposal 1: Both FDD and HD-FDD duplex mode should be considered in test configuration for (e)RedCap UE with FR1-NTN.

Observation 1: RAN4 didn’t define any new test cases for eRedCap UE in Rel-18.

Proposal 2: 15 kHz SCS and 10MHz bandwidth can be configured in test cases.

Proposal 3: Both GSO and NGSO are configured in test cases for RedCap UE with FR1-NTN.

Proposal 4: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

Proposal 5: For the proposed test configuration, the existing RMC related parameters, Io values, SSB configuration and SMTC configurations can be reused for RedCap UE with FR1-NTN.

Proposal 6: RAN4 not to consider NCD-SSB related configuration in test cases for (e)RedCap UE with NTN.

Proposal 7: RAN4 not to consider RedCap specific UL/DL BWP configuration for (e)RedCap UE with NTN.

Proposal 8: The existing test cases defined for FR1 NTN in A.14 can be reused as baseline for RedCap UEs with FR1-NTN, and RAN4 will further adapt the test cases for RedCap UE, e.g. test configuration, 1Rx/2Rx and so on.

Proposal 9: RAN4 to use the ‘refer to’ method when drafting the performance spec to avoid duplicated contents, as in core requirements.

Proposal 10: RAN4 to define new sections for test cases with suffix ‘A.X’ and measurement accuracy requirements with suffix ‘10.X’ for (e)RedCap UE with NTN.

Proposal 11: For test principle and setups, the testing related to satellite access and RedCap defined in A.3.35 and A.3.36 can be a reference.

Proposal 12: RAN4 to define new test cases without differentiating 1Rx and 2Rx (e)RedCap UEs with NTN for the following requirements:

- Enhanced cell reselection measurement

- UE transmit timing

- Timing Advance

- Active BWP switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

Proposal 13: RAN4 to define new test cases for the following requirements:

- Cell Re-selection

- Configured Grant based Small Data Transmissions (CG-SDT)

- Handover/CHO/ satellite switching with re-synchronization

- RRC Connection Mobility Control

- Radio Link Monitoring

- Link Recovery Procedures

- NR intra-frequency measurements

- NR inter-frequency measurements

- L1-RSRP measurements for Reporting

- NR measurements for positioning

Proposal 14: The test cases listed above can be a baseline, and RAN4 can further discuss which test cases can be removed to reduce the redundancy of test cases.

- FFS part can be considered to be removed to reduce test effort.

- Reference

[1]	R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)