3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513200

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on performance requirements for other RRM requirements

Source: 	CATT

Agenda item:	6.19.3.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the WF on RRM requirements for NR_NTN_Ph3_Part1 including performance requirement was approved [1], and the WI NR_NTN_Ph3 was completed on time. In this contribution, we discuss and provide our initial views on the performance requirements for other RRM requirements based on the agreed core requirements.

- Discussion

For this part, only SSB periodicity larger than 20 ms had some RRM impacts in core requiremnets, so RAN4 only need to consider the related test cases for SSB periodicity larger than 20 ms.

For SSB configuration, according to RAN1 agreement, we believe that RAN4 need to introduce new 160ms periodicity SSB pattern for DL enhancement NTN UE. According to the agreements in core part, RAN4 need to consider defining the test cases for RRC Re-establishment for Serving cell SSB Ês/Iot (dB)<-8 and unknown NR cell with 160ms SSB periodicity.

Proposal 1: RAN4 to introduce new 160ms periodicity SSB pattern for DL enhancement NTN UE.

Proposal 2: RAN4 to define the test case for RRC Re-establishment for DL enhancement NTN UE for Serving cell SSB Ês/Iot (dB)<-8 and unknown NR cell with 160ms SSB periodicity.

For SMTC enhancement, RAN4 discussed the impact on the core requirements. For more than 4 SMTCs per frequency (i.e. 6) in RRC Idle/Inactive mode, no update in the core requirements, so RAN4 doesn’t need to define the test case.

For two different SMTC periodicities (with different offsets) in one frequency layer, the following requirements are impacted:

- Maximum interruption in paging reception

- Handover requirement

- RRC Connection Release with Redirection requirement

- RRC Connected mode intra/inter-frequency measurement requirement

- FFS: RRC IDLE/INACTIVE mode intra/inter-frequency measurement requirement

Therefore, the test cases for the above requirements need to be defined. For the test configuration,  the different periodicities with different offsets need to be considered.

Proposal 3: For more than 4 SMTCs per frequency (i.e. 6) in RRC Idle/Inactive mode, RAN4 to not define the test case.

Proposal 4: For two different SMTC periodicities (with different offsets) in one frequency layer, RAN4 to define the test cases for the following requirements:

- Maximum interruption in paging reception

- Handover requirement

- RRC Connection Release with Redirection requirement

- RRC Connected mode intra/inter-frequency measurement requirement

- FFS: RRC IDLE/INACTIVE mode intra/inter-frequency measurement requirement

Proposal 5: Regarding the test configuration for the above test case,  the different periodicities with different offsets need to be considered.

For other unchanged test configurations, parameters and procedures, RAN4 can use the reference method to define the corresponding test cases to avoid lots of duplication.

Proposal 6: For other unchanged test configurations, parameters and procedures, RAN4 can use the reference method to define the corresponding test cases.

- Conclusion

This document discussed the issues for performance requirements for SSB periodicity larger than 20 ms and presented following proposals:

Proposal 1: RAN4 to introduce new 160ms periodicity SSB pattern for DL enhancement NTN UE.

Proposal 2: RAN4 to define the test case for RRC Re-establishment for DL enhancement NTN UE for Serving cell SSB Ês/Iot (dB)<-8 and unknown NR cell with 160ms SSB periodicity.

Proposal 3: For more than 4 SMTCs per frequency (i.e. 6) in RRC Idle/Inactive mode, RAN4 to not define the test case.

Proposal 4: For two different SMTC periodicities (with different offsets) in one frequency layer, RAN4 to define the test cases for the following requirements:

- Maximum interruption in paging reception

- Handover requirement

- RRC Connection Release with Redirection requirement

- RRC Connected mode intra/inter-frequency measurement requirement

- FFS: RRC IDLE/INACTIVE mode intra/inter-frequency measurement requirement

Proposal 5: Regarding the test configuration for the above test case,  the different periodicities with different offsets need to be considered.

Proposal 6: For other unchanged test configurations, parameters and procedures, RAN4 can use the reference method to define the corresponding test cases.

- Reference

[1]	R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)