3GPP TSG-RAN WG4 Meeting # 116bis                                                         R4-2513204

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on RF conformance testing of new requirements for SBFD operation

Source: 	CATT

Agenda item:	6.13.2.1

Document for:	Discussion

- Background

According to the WF[1] of RAN4#116, the RF conformance testing for SBFD operation requires further study, including both system parameters and new requirements. This paper provides additional analysis on these aspects.

- System parameters of conformance testing

## 2.1 Restriction on DL subband size

##### Issue 1-2-4: Restriction on DL subband size

Way forward:

- FFS the following aspect in conformance part for testing applicability:

  - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required.

Issue 5-1-4: Testing applicability for supported SBFD configuration

Way forward:

- Further study on the following proposals on test configuration:

  - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. For the restriction on guardband size of SBFD

  - Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations.

For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required, as the SSB can occupy only one “D” within the DUD configuration.

Proposal 1: For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required.

##### Issue 5-1-5: RF channels to be tested for SBFD operation

Way forward:

- Further study on the following proposals on RF channels to be tested for SBFD operation:

  - When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

2.2 RF channels to be tested for SBFD operation

When testing SBFD modes, the SBFD carrier can be configured at both the bottom and top positions within the operating frequency band, as these locations are the positions with the strongest out-of-band interference.

Proposal 2: When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

- Conformance testing for new RF requirements

3.1 Additional manufacture declarations and updates

Following requirements are newly introduced, and need to be added to both TS 38.141-1 and TS 38.141-2.

•Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

Proposal 3: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2.

•	Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

3.2 Additional test models for SBFD capable BS

Transmitter output power in UL subband/DL subband(s)

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 1: Transient period between SBFD and non-SBFD

For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD, as shown in figure 1.

Proposal 4 For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

For SBFD capable BS, the desired test model and frequency location of wanted signal for in-channel adjacent subband leakage requirement testing are the same as those used for BS ACLR absolute basic limit test, but with different test frequency ranges. These contents need to be added to TS 38.141-1 and TS 38.141- 2

Proposal 5: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

Proposal 6: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

For SBFD capable BS, the desired Rx input signal and its frequency location for in-channel adjacent subband blocking testing are the same as those used for in-band blocking requirement. When test this requirement, the transmitters must be turned on and the test frequency range is different from in-band blocking. These contents need to be added to TS 38.141-1 and TS 38.141- 2

Proposal 8: For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

Proposal 9: For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

Proposal 10: For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

- Summary

In this contribution, we have the following proposals.

Proposal 1: For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required.

Proposal 2: When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

Proposal 3: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2.

•	Transient period between SBFD and non-SBFD

•	Absolute limit for in-channel adjacent subband leakage

•	In-channel adjacent subband blocking

Proposal 4: For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

Proposal 5: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

Proposal 6: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

Proposal 8: For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

Proposal 9: For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

Proposal 10: For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

Reference

[1] R4-2512530, “Way Forward for [116][306] NR_duplex_evo_General”, Samsung, RAN4#116