3GPP TSG RAN WG4 Meeting #116-bis		R4-2513210

Prague, Czech, Oct 13th – 17th, 2025

Title: 	Discussion on 6G OTA and testabilities

Agenda Item:	8.12

Source: 	CATT

Document for:	Discussion

# 1	Introduction

OTA testing has made great developments in 5G, but there are some aspects still remains open. Further work to develop OTA test methodologies in 6G is essential, particularly considering the fact that AI will play a key role in 6G. In this paper we share our views on 6G OTA methodologies and the related AI testability challenges.

# 2.	Discussion

## 2.1	FR1 OTA test methodology scope

Although FR1 terminals have traditionally relied on conducted testing, it has increasingly becoming an industry consensus that OTA test methodology and performance requirements are more closely aligned with real-world conditions.

In 5G phase, the work on FR1 OTA testing has started since Rel-17. The 1Tx TRP / TRS requirements and MIMO OTA requirements are specified in [2] and [3] respectively only for handheld UE. In Rel-19, FR1 OTA test methodology for handheld NTN UE is specified in [4].

Throughout the development of FR1 OTA test method, the work on FR1 OTA test methodology and associated performance requirements usually takes handheld UE / smartphone form factor as the first priority, and limited progress has been made on other UE categories due to time constraints. In 6G, it's necessary to extend the FR1 OTA test methodology studies to cover a broader range of non-handheld device types, such as FWA UE, CPE, laptop, UAV, vehicular UE, etc.

Proposal 1:	RAN4 to study and extend the OTA test methodology and performance requirements for more non-handheld device types in 6G era.

Through the requirements defined in [2], [3] and [4], it can be seen that FR1 OTA testing mainly focus on TRP/TRS and MIMO OTA metrics in 5G phase. Since it’s recognised that OTA testing better reflects real-world conditions, RAN4 could consider to investigate OTA-based requirements and test methodology for more FR1 Tx/Rx characteristics in 6G, such as Output power dynamics, Transmit signal quality, etc.

Proposal 2:	RAN4 to investigate and extend OTA-based requirements and test methodology for more FR1 transmitter and receiver characteristics in 6G era.

Moreover, in 5G phase the FR1 OTA testing and performance metric definition are mainly focusing on single band testing. With larger aggregated bandwidth and more flexible spectrum usage will be defined in 6G, it's time to consider the OTA testing aspects for multi carrier combinations for FR1 UE.

Proposal 3:	RAN4 to study the OTA test methodology and performance requirements for multi frequency range combinations for FR1 UE in 6G era.

## 2.2	FR2 OTA test methodology scope

In 5G, there have been some considerations on the testing of FR1+FR2 configuration, such as providing FR1 link to the DUT. Taking DFF method as an example, following is the extract of the DFF description in 5.2.1.1 of [5]:

The key aspects of the DFF setup are: - Far-field measurement system in an anechoic chamber …… - For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

But the fact is that the conformance test for FR1+FR2 band combinations has never been executed in practice. The main reason is the absence of an applicable test system that can simultaneously perform conducted measurements in FR1 and OTA measurements in FR2.

In 6G, UL/DL spectrum decoupling will be considered. UE is likely to be configured with FR1 frequency in the UL and FR2 frequency in the DL. It’s essential to study the pure OTA test methodology for UE configuring mixed FR1-FR2 operation in 6G.

Proposal 4:	RAN4 to study pure OTA test methodology for UE configuring mixed FR1-FR2 operation in 6G era.

## 2.3	AIML based beam management testability issue

In 5G, the testability and interoperability for AI/ML-based beam-management has been discussed in Rel-19. However, RAN4 hasn’t defined an OTA test method that can jointly verify UE beam-sweeping performance and AI/ML inference capability. Considering that more and more AI/ML use cases will be studied in 6G and their performances are expected to be verified in an environment that is similar to the real-world conditions, it is necessary to continue investigating the OTA test system for UE AI/ML-based use cases performance verification in 6G.

Proposal 5:	RAN4 to continue to study the OTA test system for AI/ML-based beam-management in 6G era.

# 3 Conclusion

In this paper, we submit following proposals on the scope consideration of OTA test methodology and testability issues in 6G:

Proposal 1:	RAN4 to study and extend the OTA test methodology and performance requirements for more non-handheld device types in 6G era.

Proposal 2:	RAN4 to investigate and extend OTA-based requirements and test methodology for more FR1 transmitter and receiver characteristics in 6G era.

Proposal 3:	RAN4 to study the OTA test methodology and performance requirements for multi frequency range combinations for FR1 UE in 6G era.

Proposal 4:	RAN4 to study pure OTA test methodology for UE configuring mixed FR1-FR2 operation in 6G era.

Proposal 5:	RAN4 to continue to study the OTA test system for AI/ML-based beam-management in 6G era.

# Reference

[1]	RP-251881 New SID: Study on 6G Radio, NTT DOCOMO

[2]	TS 38.161 User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios

[3]	TS 38.151 Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs

[4]	TR 38.870 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)

[5]	TR 38.810 Study on test methods