3GPP TSG-RAN WG4 Meeting #116bis				                                           R4-2513212

Prague, Czech, October 13th – October 17th, 2025

Title:	Discussion on the maintenance of RRM core requirements of NR RRM Ph5

Source:	CATT

Agenda Item:	4.15.1

Document for:	Discussion

1 Introduction

In last meeting, RAN4 discussed the enhancement for NR RRM Ph5 including FR2-1 SSB based measurement delay reduction and FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the core part of this WI was completed. The agreements were captured in the approved WF [1].

In this contribution, we would like to discuss some maintenance issues on the core part and present our views.

2 Discussion

RAN4 discussed the quitting condition and re-evaluation mechanism of FBS in last meeting and the following agreements were achieved:

RAN4#116: ### Issue 1-1-1: Value for T1 Agreement: - A fixed value of 90 seconds. ### Issue 1-1-2: Re-evaluation mechanism Agreement: FFS following options in the maintenance stage: - Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be: - Alt 1: Use the values for T1 as a starting point. - Alt 2: 5 s, which is predefined in RAN4 spec. - Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS. - Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS. Agreement in Friday online: - No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

Regarding the re-evaluation mechanism after UE falls back to normal measurements, companies have different views as above. And based on the discussion situation, it seems difficult converging views in this meeting. Therefore in order to facilitate progress in this work item, we are okay that it is up to UE implementation when to re-enter the FBS after quitting the last FBS. But in the following two cases, UE needs to evaluate the entering thresholds, i.e., fbs-ThresholdP-r19 and/or fbs-ThresholdQ-r19, and re-enter the FBS mode immediately if the thresholds are satisfied:

Case 1: Entering thresholds are reconfigured by network and they are satisfied;

Case 2: For unchanged entering thresholds, when entering thresholds that are unsatisfied become satisfied, UE needs to re-enter the FBS mode immediately.

Figure 1 illustrates the two cases:

Figure 1 – Two cases where UE needs to evaluate the triggering conditions and re-enters the FBS mode if they are met (Left: case 1; Right: case 2)

Proposal 1: It is up to UE implementation when to re-enter the FBS after quitting the last FBS with RAN4 confirming that UE needs to evaluate the triggering conditions and re-enter the FBS mode immediately if the thresholds are satisfied in the following two cases:

Case 1: Triggering conditions are reconfigured by network and they are satisfied;

Case 2: For unchanged entering thresholds, when entering thresholds that are unsatisfied become satisfied, UE needs to re-enter the FBS mode immediately.

Another issue is how to specify requirements when UE reports different BSF values for different bands [1]:

Way forward: Further discuss this issue in the maintenance stage: - Option 1: - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

According to the applicability requirements [2], for intra-frequency measurements, the target carrier is the serving carrier, in which case there is no such issue. For inter-frequency measurements, there will be two carriers, one is the serving carrier and the other one is the target carrier. These two carriers may belong to the same band or different bands. When they belong to the same band, there is no such issue either. But when they belong to different bands, RAN4 need to clarify which BSF value should be used in the requirements. In our understanding, the BSF value reported on the band to which the target carrier belongs should be used in the requirements.

Observation 1: This issue only exists for inter-frequency measurements and when serving carrier and target carrier are in different bands.

Proposal 2: For inter-frequency measurements and when serving carrier and target carrier are in different bands, the BSF value reported on the band to which the target carrier belongs should be used in the requirements.

3 Conclusion

In this paper, we discussed some maintenance issues on the core part of NR RRM Ph6 WI and the following proposals are provided:

Proposal 1: It is up to UE implementation when to re-enter the FBS after quitting the last FBS with RAN4 confirming that UE needs to evaluate the triggering conditions and re-enter the FBS mode immediately if the thresholds are satisfied in the following two cases:

Case 1: Triggering conditions are reconfigured by network and they are satisfied;

Case 2: For unchanged entering thresholds, when entering thresholds that are unsatisfied become satisfied, UE needs to re-enter the FBS mode immediately.

Observation 1: This issue only exists for inter-frequency measurements and when serving carrier and target carrier are in different bands.

Proposal 2: For inter-frequency measurements and when serving carrier and target carrier are in different bands, the BSF value reported on the band to which the target carrier belongs should be used in the requirements.

4 References

- R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, Moderator (CATT), RAN4#116.

- R4-2512348, Draft big CR for NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RAN4#116.