3GPP TSG-RAN WG4 Meeting #116bis	R4-2513267

Prague, CZ, 13th – 17th October 2025

Agenda item:	8.5

Source: 		NTT DOCOMO, INC.

Title: 	6G RAN4 Study on BS RF

Document for:	Discussion

- Introduction

In RAN4#116, the plan and meeting arrangement for 6G RAN4 study were introduced through contribution R4-2511652[1]. Starting from this meeting, discussions on 6G begin within each RAN4 topic. In this contribution, we present our view on 6G in the context of RAN4 BS RF.

- Discussion

According to R4-2511652[1], the scope of BS RF as 6G topics was shared. We contribute the 6G concept from each RAN4 perspective.

- Frequency Ranges, BS type

For 6G BS RF, the initial focus should be on the partitioning of Frequency Ranges (FR) and the definition of BS types. At this stage, it is anticipated that sub-10 GHz will be the main focus, with bands around 15 GHz also under consideration. The significance of each BS functional block to the overall RF performance changes across these frequency ranges. This shift is due to factors such as the choice of beamforming architecture (e.g., digital, analog, or hybrid) and the increasing significance of antenna system effects. Therefore, the requirement reference points for each BS type need to be set to enable meaningful evaluation in their respective frequency ranges. Also, FR partitions should align with practical usage and the specific evaluation domains tied to each BS type. Following this logic, a prospective FR3 (i.e., the range between FR1 and FR2) would need to be sub-divided where its RF characteristics change significantly. Furthermore, we note that the current FR1 already groups bands with different usage patterns (e.g., lower and upper FR1). This raises a fundamental question for the 6G era: what is the intended purpose of the FR framework? If the framework should reflect technical characteristics, a re-evaluation of its definitions becomes necessary. If, however, its purpose is simply to indicate 'frequencies added in 6G,' then following the precedent and placing all new bands into a single FR3 bucket would be a straightforward option.

Observation 1: For the candidate frequency ranges for 6G, establishing appropriate requirement reference points for each frequency range is crucial for a meaningful evaluation of RF performance.

Proposal 1: As a first step, we propose establishing a clear principle for the 6G FR framework, deciding whether to align FR partitions with technical characteristics or to use them simply as labels for new spectrum.

- Co-location requirement

The current BS-to-BS co-location framework is based on assumptions such as fixed coupling loss, which were defined before beamforming became widely adopted. However, beamforming-capable BS are now increasingly deployed, and future 6G systems are expected to rely heavily on advanced beamforming technologies, especially in higher frequency bands. This evolution in hardware and deployment strategy introduces a mismatch between the legacy assumptions and the actual behavior of modern systems. Without updating the framework, there is a risk of inefficient site planning.

Observation 2: The growing adoption of beamforming suggests a potential misalignment between the current co-location framework and the deployment realities expected in future 6G systems.

Proposal 2: To avoid inefficiencies in deployment, co-location requirements need to be defined with careful consideration of the actual characteristics of beamforming-capable BS.

- Test model

In 5G NR, the current specifications define Test Models (TM) only for single-layer transmission. This limitation has previously caused practical issues during the conformance testing of beamforming BS. When the same single-layer TM was applied across multiple transmission layers, the signals from each layer could combine in-phase. This phenomenon, in some cases, led to an inability to accurately measure the base station output power, as the resulting PAPR differed significantly from actual operating conditions.

Observation 3: The 5G experience shows that the exclusive use of single-layer Test Models can lead to inaccurate power measurements for beamforming BS due to in-phase signal combining.

Furthermore, the existing 5G NR Test Models are defined with a fixed number of Physical Resource Blocks (PRB) and lack scalability [2]:

4.9.2.2 FR1 test models The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test. The following general parameters are used by all NR test models: - Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms) - The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier spacing - NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-1 in TS 38.104 [2]. - Normal CP - Virtual resource blocks of localized type

In practice, however, 5G NR operational scenarios sometimes require a BS to transmit using a narrower PRB allocation than the fixed values specified in the TMs, due to factors like specific band utilization or co-existence problems. This has created a gap between test conditions and realistic operational needs.

Observation 4: The 5G experience shows that fixed-PRB Test Models can create a mismatch between testing conditions and the practical needs of deployment and certification.

Proposal 3: For 6G, the specification of Test Models needs to reflect the scenarios where they will be utilized.

- Conclusion

In this contribution, we offer our 6GR view as follows.

Observation 1: For the candidate frequency ranges for 6G, establishing appropriate requirement reference points for each frequency range is crucial for a meaningful evaluation of RF performance.

Proposal 1: As a first step, we propose establishing a clear principle for the 6G FR framework, deciding whether to align FR partitions with technical characteristics or to use them simply as labels for new spectrum.

Observation 2: The growing adoption of beamforming suggests a potential misalignment between the current co-location framework and the deployment realities expected in future 6G systems.

Proposal 2: To avoid inefficiencies in deployment, co-location requirements need to be defined with careful consideration of the actual characteristics of beamforming-capable BS.

Observation 3: The 5G experience shows that the exclusive use of single-layer Test Models can lead to inaccurate power measurements for beamforming BS due to in-phase signal combining.

Observation 4: The 5G experience shows that fixed-PRB Test Models can create a mismatch between testing conditions and the practical needs of deployment and certification.

Proposal 3: For 6G, the specification of Test Models needs to reflect the scenarios where they will be utilized.

References

[1] 3GPP, R4-25116522, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple).

[2] 3GPP, TS38.141-1 V19.1.0, “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.