3GPP TSG-RAN WG4 Meeting #116bis	R4-2513292

Prague, CZ, 13th – 17th October 2025

Agenda item:	8.8

Source: 		NTT DOCOMO, INC.

Title: 	6G RAN4 Study on demodulation

Document for:	Discussion

- Introduction

In RAN4#116, the plan and meeting arrangement for 6G RAN4 study were introduced through contribution R4-2511652[1]. Starting from this meeting, discussions on 6G begin within each RAN4 topic. In this contribution, we present our view on 6G in the context of RAN4 demodulation.

- Discussion

According to R4-2511652[1], the scope of demodulation as 6G topics was shared. We contribute the 6G concept from each RAN4 perspective.

In RAN#109, the following items related to demodulation were agreed in RAN1[2].

Waveform Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc. Frame structure Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP Channel coding Chairman Guidance For 6GR control channel coding, - Evaluations can be provided in form of BLER and FAR results. - Evaluations/analysis can be provided for complexity, decoding latency, - Other metrics are not precluded. - Proponent companies to provide evaluation assumptions and methodologies for respective evaluation. - Proponent companies to provide details of channel coding extension compared with NR channel coding - Proponent companies to provide justification for the channel coding extension, compared with control channel codes as defined in 5G NR. Chairman Guidance For 6GR data channel coding, - Evaluations can be provided in form of BLER results. - Evaluation/analysis on throughput, complexity, and decoding latency can be provided - Other metrics are not precluded. - Proponent companies to provide their target scenarios and requirements, evaluation assumptions and methodologies for respective evaluation/analysis, e.g., decoding algorithm and details, information sizes, code rates, HARQ scheme, channel type, modulation order, target BLER, etc. - Proponent companies to provide details of channel coding extension compared with NR channel coding. - Proponent companies to provide justification for the channel coding extension, and how to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off, compared with data channel codes as defined in 5G NR. Modulation, joint channel coding and modulation Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes Energy efficiency Agreement Study how to reuse and update reference configurations in TR 38.864 for 6G BS. Agreement Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. Agreement - Study metric(s) for UE energy efficiency. - Study metric(s) for BS energy efficiency. Agreement Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following: - TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations - TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models Agreement - Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s) - Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

First, regarding the waveform, it was agreed that the baseline will follow NR, where CP-OFDM is defined for DL and both CP-OFDM and DFT-S-OFDM are defined for UL. These waveforms were originally chosen in NR for their robustness and spectral efficiency, and similar benefits are expected for 6G. Therefore, RAN4 needs to establish the corresponding RF and demodulation specifications. If any additional waveforms are introduced in 6G (e.g., for new use cases or improved energy efficiency), new RAN4 specifications will also be required.

Observation 1: RAN1 agreed to adopt NR-based waveforms (CP-OFDM for DL, CP-OFDM and DFT-S-OFDM for UL) as the baseline for 6G. RAN4 needs to define corresponding RF and demodulation specifications and prepare for potential additional waveforms for new use cases or energy efficiency improvements.

Next, regarding the frame structure, it was agreed to basically reuse what was specified in NR to maintain backward compatibility and reduce complexity. For the newly considered FR3 (7.125–8.4 GHz, 14.8–15.35 GHz), demodulation requirements need to be discussed in parallel with numerology because higher frequencies may require different assumptions for phase noise and Doppler effects. For FR3, it has not yet been decided whether measurements will be conducted in a conducted setup or OTA, so RAN4 needs to define the test methodology accordingly.

Regarding multiple SCS per band introduced in NR for operational flexibility, this approach increased complexity in parameter configurations and UE implementation. For 6G, introducing only one SCS per band is under consideration to simplify design. However, regardless of the FR, RAN4 needs to define demodulation requirements for all SCSs defined by RAN1, as these directly impact RF performance and receiver design.

Observation 2: Frame structure will follow NR basically, but FR3 introduces new challenges such as phase noise and Doppler effects. RAN4 must define test methodologies for FR3, including whether measurements are conducted or OTA.

Observation 3: For 6G, only one SCS per band is under consideration, RAN4 must define demodulation requirements for all SCSs agreed in RAN1.

For TDD configuration, defining test requirements per operator would result in an excessive number of patterns; therefore, in NR, only practical patterns were specified. Similarly, for 6G, pattern reduction is necessary, and for FR1 and FR2, the TDD configurations used as a baseline in NR should be introduced to ensure consistency and interoperability.

Observation 4: To avoid excessive complexity, NR limited TDD configurations to practical patterns. A similar approach is needed for 6G, with FR1 and FR2 reusing NR baseline configurations.

Regarding channel coding, no concrete agreement has been reached in RAN1 yet, but based on the Chair guidance, RAN4 should ensure that evaluation metrics (e.g., BLER, throughput) can be tested according to what is defined by RAN1.

Observation 5: No final agreement on channel coding exists in RAN1 yet. RAN4 should ensure that evaluation metrics (e.g., BLER, throughput) can be tested according to RAN1 definitions.

From a modulation perspective, it was agreed that the baseline will follow NR for both DL and UL. However, for new 6G use cases, techniques such as QPSK rotation and QAM-CS are being discussed, particularly for UL, to achieve higher data rates and wider coverage. When a new MCS table for 6G is agreed in RAN1, RAN4 should develop corresponding test specifications to ensure proper performance validation.

Observation 6: Baseline modulation follows NR, but new techniques (e.g., QPSK rotation, QAM-CS) are under discussion for UL to support higher data rates and coverage. RAN4 must prepare test specifications once new MCS tables are agreed in RAN1.

Regarding energy efficiency, while network power consumption was discussed in later NR releases, for 6G, early introduction from day 1 is important to meet sustainability goals. Similarly, features such as (e)RedCap and LP-WUS are being discussed for day 1 introduction, and RAN4 should coordinate with RAN1 to make proposals that consider both performance and energy efficiency.

Observation 7: Energy efficiency and features like (e)RedCap and LP-WUS are targeted for day 1 introduction in 6G. RAN4 should coordinate with RAN1 to ensure testability and alignment with performance and energy-saving goals.

Furthermore, to ensure the specification's practical usability, we should consider lessons from 5G. For instance, performance requirements were sometimes not defined for practical BS configurations with a certain number of RX antennas, creating ambiguity in testing. For 6G, it is desirable to provide clear guidance for such cases, like how to test configurations with a non-specified number of RX antennas, to ensure consistent procedures.

Observation 8: Experience from 5G showed that unspecified parameters, such as the number of BS RX antennas for certain performance tests, can lead to ambiguity. For 6G, specifications need to provide guidance for such cases to ensure clear and practical test procedures.

Proposal 1: RAN4 needs to develop testable specifications for waveform, frame structure, channel coding, and modulation as defined by RAN1, ensuring that these can be practically implemented and validated in real products.

Next, regarding demodulation aspects not directly related to RAN1, our concept is as follows: In 5G day 1 (Release 15), HST requirements were not included. From a commercial perspective, it is desirable to introduce them from day 1 in 6G, and both BS and UE demodulation requirements should be completed within the same release to avoid interoperability issues. It is also necessary to ensure consistency of test parameters between BS and UE. In NR, 350 km/h and 500 km/h were discussed, and for 6G, these speeds are expected to be the baseline. While the 350 km/h requirement is stricter than 500 km/h for RAN4, if there is a significant SNR difference between the two, both should be specified to ensure robustness.

Similarly, requirements related to URLLC (referred to as HRLLC in 6G) were not included in the initial 5G specifications (Release 15). From a commercial perspective, it is also desirable to introduce HRLLC requirements from day 1 in 6G. At present, we assume that the functional features for 6G HRLLC will be the same as those defined for 5G URLLC, but it is necessary to discuss whether the demodulation performance requirements should follow the existing specifications defined for 5G URLLC or be updated to reflect 6G conditions.

Observation 9: Requirements of some feature enhancements were missing in 5G day 1 but should be included from day 1 in 6G. Both BS and UE demodulation requirements must be aligned within the same release.

For MIMO, CDL (Clustered Delay Line) models have been under discussion since Release 19. CDL is expected to be particularly useful for UL, for example in beam selection and spatial diversity. If CDL provides measurable gains compared to TDL, it should be actively introduced. Furthermore, consistency should be maintained regarding whether to use CDL or TDL for each function, to avoid mixing CDL-based and TDL-based requirements depending on the number of Rx antennas, which could complicate implementation and testing.

Observation 10: CDL models, introduced in Rel-19, are expected to provide gains for UL (e.g., beam selection). RAN4 should adopt CDL where beneficial and maintain consistency to avoid mixing CDL and TDL requirements across functions.

Proposal 2: 5GA Topics that were studied and proven useful should be introduced from day 1 in 6G, and RAN4 needs start discussions on corresponding test specifications to enable early adoption.

- Conclusion

In this contribution, we offer our 6GR view as follows.

Observation 1: RAN1 agreed to adopt NR-based waveforms (CP-OFDM for DL, CP-OFDM and DFT-S-OFDM for UL) as the baseline for 6G. RAN4 needs to define corresponding RF and demodulation specifications and prepare for potential additional waveforms for new use cases or energy efficiency improvements.

Observation 2: Frame structure will follow NR basically, but FR3 introduces new challenges such as phase noise and Doppler effects. RAN4 must define test methodologies for FR3, including whether measurements are conducted or OTA.

Observation 3: For 6G, only one SCS per band is under consideration, RAN4 must define demodulation requirements for all SCSs agreed in RAN1.

Observation 4: To avoid excessive complexity, NR limited TDD configurations to practical patterns. A similar approach is needed for 6G, with FR1 and FR2 reusing NR baseline configurations.

Observation 5: No final agreement on channel coding exists in RAN1 yet. RAN4 should ensure that evaluation metrics (e.g., BLER, throughput) can be tested according to RAN1 definitions.

Observation 6: Baseline modulation follows NR, but new techniques (e.g., QPSK rotation, QAM-CS) are under discussion for UL to support higher data rates and coverage. RAN4 must prepare test specifications once new MCS tables are agreed in RAN1.

Observation 7: Energy efficiency and features like (e)RedCap and LP-WUS are targeted for day 1 introduction in 6G. RAN4 should coordinate with RAN1 to ensure testability and alignment with performance and energy-saving goals.

Observation 8: Experience from 5G showed that unspecified parameters, such as the number of BS RX antennas for certain performance tests, can lead to ambiguity. For 6G, specifications need to provide guidance for such cases to ensure clear and practical test procedures.

Proposal 1: RAN4 needs to develop testable specifications for waveform, frame structure, channel coding, and modulation as defined by RAN1, ensuring that these can be practically implemented and validated in real products.

Observation 9: Requirements of some feature enhancements were missing in 5G day 1 but should be included from day 1 in 6G. Both BS and UE demodulation requirements must be aligned within the same release.

Observation 10: CDL models, introduced in Rel-19, are expected to provide gains for UL (e.g., beam selection). RAN4 should adopt CDL where beneficial and maintain consistency to avoid mixing CDL and TDL requirements across functions.

Proposal 2: 5GA Topics that were studied and proven useful should be introduced from day 1 in 6G, and RAN4 needs start discussions on corresponding test specifications to enable early adoption.

References

[1] 3GPP, R4-25116522, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple).

[2] 3GPP, RP-252475, “Status report for Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone.