3GPP TSG-RAN WG4 Meeting #116bis	R4-2513310

Prague, Czechia, October 13 – 17, 2025

Agenda Item:	8.9

Source:	Huawei, HiSilicon

Title:	Consideration on RAN4 AI study for 6G

Document for:	Approval

- Introduction

Guided by the framework of the RAN4 agenda in [1], RAN4 driven use cases are discussed in Q4'25 and Q1'26. RAN4 impacts of other WG driven use cases and RAN for AI will be discussed after Q1'26. In this paper, we provide our views on the RAN4 driven AI/ML uses cases.

- Principles for identifying RAN4-led AI/ML uses case

Prioritize use cases focusing on common issues or pain points identified within RAN4 that could be solved by AI.

For use cases that may potentially impact the specifications of other working groups, but where the issues can be identified, and solutions can be proposed and validated within RAN4, RAN4 need not wait for other WGs to initiate discussions on these issues - instead, it can proceed with preliminary studies first. This approach avoids delays in addressing the issues and ensures that RAN4’s proposed solutions can be incorporated into timely coordinated discussions with relevant design areas of other WGs.

Performance gains of AI-based solutions over non-AI counterparts can be verified with minimal reliance on fundamental design inputs from other 3GPP working groups.

Use cases that are highly dependent on design inputs from other working groups (WGs) should be assigned lower priority. For instance, the performance gains of AI receivers for advanced reference signal designs could await inputs from other WGs. This approach avoids potential duplicated workload. However, AI receivers - such as those leveraging AI-based CSI acquisition from baseline RS patterns for 6GR - can be studied within RAN4.

Strive for an appropriate trade-off between performance and complexity during use case identification.

AI has the potential to deliver either higher performance or lower overhead. However, AI models themselves introduce additional implementation complexity due to their inference and LCM procedures. This additional complexity is only justifiable if significant gains can be achieved. From an implementation complexity perspective, there exist scenarios where adopting AI yields negative gains. Therefore, even though AI may be introduced to enhance certain features, the scenarios in which AI models are enabled needs to be carefully considered.

Proposal 1: Principles for identifying RAN4-led AI/ML uses case are outlined below.

- Prioritize use cases focusing on common issues or pain points identified within RAN4 that could be solved by AI.

- Performance gains of AI-based solutions over non-AI counterparts can be verified with minimal reliance on fundamental design inputs from other 3GPP working groups.

- Strive for an appropriate trade-off between performance and complexity during use case identification.

- Potential RAN4-led AI/ML uses cases

AI for CSI acquisition enhancement

PMI codebook designs have evolved from NR Releases 15 through 20, aiming to support a range of MIMO scenarios including MU-MIMO, multi-TRP CJT, and others. For several types of PMI feedback, RAN4 has defined corresponding requirements and test cases, including Type I, Type II, eType II, typeII-doppler, TypeII-CJT.

While the Type II PMI family offers higher-precision precoders at the cost of increased computational complexity and feedback overhead, Type I PMI remains the only one deployed commercially.

For 6GR MIMO, greater numbers of antenna ports and larger channel bandwidths are anticipated. This poses new challenges for CSI-RS and CSI feedback that require low overhead while enabling high-resolution CSI acquisition. Both AI-based and non-AI solutions are envisioned to address these challenges. RAN4 is expected to define requirements for a series of new CSI feedback performance aspects.

By contrast with using CSI-RS to obtain DL CSI, directly acquiring raw channels via SRS in TDD systems - without relying on CSI/PMI feedback - is more appealing. Such CSI, with higher spatial and frequency resolution, is particularly essential for systems evolving toward a larger number of CSI-RS ports. This makes SRS an attractive complement or even a substitute for CSI-RS in 6GR.

However, CSI acquisition from SRS is facing several challenges as well, including SRS IL imbalance and channel aging issues.

SRS IL imbalance

SRS IL imbalance hinders the accurate acquisition of DL CSI, and this issue is exacerbated by the support for a higher number of antennas and upper mid-band spectrum in 6GR. In 5G NR, RAN4 identified the issue and proposed candidate solutions. Also, most companies in RAN1 observed that the existence of SRS IL imbalance will cause non-negligible performance degradation, especially under MU-MIMO and/or high-rank scenario. However, no consensus was reached on whether and how to support SRS IL imbalance reporting.

UE’s capability to pre-compensate the loss induced by ∆TRxSRS,p (p=0, 1, …) under different PCMAX_L, f, c scenarios varies, and different UEs may also employ distinct strategies regarding whether or how to perform pre-compensation. Dynamic reporting of exact value of SRS IL imbalance to ensure compatibility with different UE implementations is beneficial but will increase UE’s reporting overhead. Static or semi-persistent reporting is not sufficient which may lead to additional channel estimation error. Moreover, based on our simulation results, at medium to low SNRs, even with exact reporting, obvious compensation loss is still observed with compensation is performed at the NW side.

AI/ML has the potential to recover CSI even under low SNR, by modelling the relationship between CSI without and with IL imbalance. Specifically, the model takes as input the measured CSI derived from SRS with IL-induced power imbalance, while its output is the measured CSI free of such power imbalance. Labeled data for training/finetuning can be obtained through CSI-RS measurement and reporting procedures, or can be obtained through SRS measurements under higher enough SNRs, in real-world deployments. Further, NW-side data collection with lower overhead could also be achieved by introducing UE-side assistant information.

Simulation assumption and results

To model SRS IL imbalance, two cases are considered: 1) exact values of [0, -2.5, -5, -7.5] dB, and 2) a distribution where the exact values serve as the mean values, with uniform [-1,1] dB randomness added around each mean. Accordingly, two AI models are trained - one for scenario-specific use case and the other for generalized use case, respectively. Specifically, the training data for the first AI model consists of CSI generated from the exact values, while the training data for the second model consists of CSI generated from the distribution. For both cases, training datasets are collected covering a range of SNRs, which helps achieve model’s generalization across SNRs.

For the baseline scenario, we assume that non-AI-based compensation is performed at NW side, assuming exact values of SRS IL imbalance are reported by UE in a timely manner.

Given the gap between the received power of the UL SRS and DL CSI-RS, the performance gain achieved by AI may stem from both compensation for the SRS IL imbalance and enhanced channel estimation - provided that CSI-RS measurements are adopted as the ground truth during model training. The performance using SRS measurements without IL imbalance as the ground truth is also provided, to show the pure gain from compensating for SRS IL imbalance alone.

Figure 1 SGCS for AI and non-AI-based compensation under SRS power imbalance

Observation 1: AI-based compensation for SRS IL imbalance outperforms non-AI-based compensation across the entire SNR range, and the performance gains are more obvious at medium-to-low SNRs.

Compared to non-AI-based exact compensation, AI is capable of compensating without the need to acquire exact values – thanks to its generalization capability. Additionally, a minor performance loss is observed for the generalized AI model compared with the scenario-specific AI model.

SRS channel aging

Longer intervals required for CSI acquisition from SRS with frequency hopping exacerbate the channel aging issue, especially for larger bandwidth, more antennas at UE to be sounded, and a larger number of UEs with emerging services in a cell - all of which align with the evolving directions of 6GR.

By using AI to perform CSI prediction, this channel aging issue could be solved. Leveraging other RSs - for example, the UL DMRS - in a complementary fashion through configured or dynamically scheduled PUSCHs enables more flexible updates of CSI. Without additional overhead or with significantly reduced overhead compared to CSI-RS, channel aging can be greatly mitigated via CSI tracking. AI/ML demonstrates potential in addressing the occasional presence of UL DMRS and predicting CSI especially in the spatial domain, since physical Tx antenna mapping patterns for DMRS transmission could be a subset of patterns for antenna switching SRS transmission.

Taking historical SRS and DMRS measurements within the observation window as inputs to the NW-sided AI model, CSI for all SRS ports within the prediction window is obtained as the model output. The choice of training type can be left to the NW for implementation, with labeled data constructed from CSI-RS or SRS measurements.

Proposal 2: In 6GR, consider supporting enhancement to CSI acquisition from SRS using AI/ML.

- Conclusions

Proposal 1: Principles for identifying RAN4-led AI/ML uses case are outlined below.

- Prioritize use cases focusing on common issues or pain points identified within RAN4 that could be solved by AI.

- Performance gains of AI-based solutions over non-AI counterparts can be verified with minimal reliance on fundamental design inputs from other 3GPP working groups.

- Strive for an appropriate trade-off between performance and complexity during use case identification.

Observation 1: AI-based compensation for SRS IL imbalance outperforms non-AI-based compensation across the entire SNR range, and the performance gains are more obvious at medium-to-low SNRs.

Proposal 2: In 6GR, consider supporting enhancement to CSI acquisition from SRS using AI/ML.

- References

[1] RAN4 6G Plan and Meeting Arrangement, R4-2511652, RAN4 #116, Aug, 2025.