3GPP TSG-RAN4 Meeting #116bis	R4-2513339

Prague, Czech Republic, 13 October – 17 October 2025

Source:			Ericsson

Title:			BS RF and co-existence considerations for 6G

Agenda item:		8.5

Document for:		Discussion

# Introduction

At RAN4#116-bis the discussion related to 6G BS RF aspects and 6G co-existence evaluation aspects starts in RAN4.

In this contribution we provide an overview of some areas to discuss within the scope of RAN4 6G SI.

# Discussion

## General aspects

The 6G Radio Study item in RP-252912 outlines a number of objectives for the study, of which many have impact on BS RF. This includes, but is not limited to the following objectives:

- Single technology framework based on a stand-alone architecture, where adoption of multiple options is minimized for the same functionality and excessive configurations are avoided.

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments.

- Improved spectrum utilization and operations taking into account diverse spectrum allocations.

- Physical layer with 6GR waveforms (OFDM-based), where 5G waveforms are considered, and serves as benchmark.

- Avoiding multiple numerologies for the same band

- BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

- Study how to improve 6G BS core and conformance specifications, including structure and drafting principles

- Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested, considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR, including 5G-6G Multi-RAT Spectrum Sharing for migration.

The fundamental concepts of how earlier generations were specified should be kept and refined for BS RF in 6G. This includes the definition of supported frequency ranges through Operating bands, grouping the requirement for different operating bands in Frequency Ranges (FR1, FR2, …), differentiating BS for different deployments in Base Station classes and providing frameworks for both conducted and radiated (OTA) requirements. Support of multiple RATs in the same BS should be made through Multi-Standard Radio (MSR) BS and AAS BS specifications. Details of the fundamental concepts should be studied for possible improvement and simplification.

Proposal 1: The fundamental concepts of Operating bands, Frequency Ranges, Base Station classes, separate conducted and OTA requirements, MSR BS and AAS BS specifications to be applied also for 6G, with appropriate improvements and simplifications.

The overall specification framework should thus remain, but complexity must be minimized in terms of number of options, variations and combinations of features. While many requirements can be re-used from NR, the requirements should also be re-evaluated and if possible simplified and improved in terms of the definitions. The ongoing effort to re-evaluate some NR BS requirements should thus continue into 6G.

Proposal 2: The complexity of BS specifications must be minimized in terms of number of options, variations and combinations of features. Requirements can be re-used from NR but need to be re-evaluated and if possibly simplified and improved in terms of definitions.

In the first release, 6G should be defined for a limited set of bands in FR1 and FR2. For 6G in all existing NR bands, the NR numerology should be re-used. New 6G bands should be added when made available, including 7-15 GHz cm-wave bands. With BS requirements organized through the Frequency Range concept, frequency ranges in addition to FR1 and FR2 should only be introduced for operating bands that need new requirements, procedures and/or requirements concepts. This is also discussed in our other contribution ([4]).

With MRSS defined from the start, there will be a need for MSR BS specification from the start, covering at least 6G+5G NR including NB-IoT. This will also be facilitated by using 5G NR waveforms for 6G and the re-use of numerology for NR bands. Migration with Standalone operation is also defined from the start. This is further discussed in the following subclause 2.2.

## Specification framework

### Impact on BS core specifications

The straightforward approach would be to create a new TS xx.104 for single-RAT 6G including TDD, FDD and support for IoT and SBFD operation. The MSR specifications in Rel-21 must be drafted including the new 6G RAT, besides NR and E-UTRA. The MSR BS core specification TS 37.104 should include the 6G RAT (BS Type 1-C). We assume that the interest in GSM and UTRA will be close to zero at the time when Rel-21 is stable and ready for being utilized in products.

Proposal 3: When introducing 6G RAT in MSR specifications, RAN4 should further evaluate the need of specifying new category sets supporting 6G with GSM and UTRA.

The AAS BS core specification TS 37.105 will also be enhanced with the introduction of the new 6G RAT (Hybrid AAS BS and OTA AAS BS), and similar to TS 37.104, there will be support only for E-UTRA, NR and 6G. Also, FR2-1 needs to be introduced.

Proposal 4: In the 6G scope, FR2-1 support should be added to MSR AAS specifications.

A second alternative approach would be to not create a new single RAT specification for 6G, instead extend the scope of MSR BS specification 37.104 to include full support for single RAT requirements as well. Such an approach can lead to the withdrawal of single RAT specifications 36.104 and 38.104 from Rel-21 and have only two multi-standard specifications enabled instead: 37.104 (for BS with up to 8 antenna ports) and 37.105 (for hybrid and OTA BS).

Proposal 5: When introducing 6G RAT, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.104 and TS 37.105) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.104 and TS 37.105).

### Impact on BS conformance specifications

Conformance specifications can follow the same structure as we have described for the core specifications. If we go for the straightforward approach, two new specifications TS xx.141-1 and TS xx.141-2 are expected, that are dedicated to the 6G new RAT conformance testing.

New test signals, Test Models (x-TM) for DL and Fixed Reference Channels (FRC) for UL and test configurations (TC) should be needed.

The MSR BS specification TS 37.141 will need introduction of the new 6G new RAT (BS Type 1-C). There will be a need for review of the capability sets, being possible to reduce their number.

In the case of AAS BS conformance testing specifications TS 37.145-1 and -2 we see a similar impact as for the core specifications when the new 6G RAT is introduced.

The alternative of dropping completely the single RAT conformance specifications and rely totally on TS 37.141 (for BS with conducted requirements) and 37.145-1/-2 (for hybrid and OTA AAS BS) is appealing and proposed for further discussion.

Proposal 6: Similar and aligned with the core specifications decision, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2).

A general comment would be to increase the quality of the BS specifications in general. We need to consider more the drafting rules, take care of the nature of manufacturer declarations, possible choices of the test requirements and methods, also make visible how requirements apply in different regions. The OTA test methods descriptions need to be improved, for more clarity, but content wise will be similar to NR.

## Requirements evolution

The current BS RF requirements framework supports efficiently standalone 5G and MSR operations. 6G will operate in new spectrum, but also in existing 3GPP bands, sharing spectrum efficiently with 5G. To support MRSS (Multi-RAT Spectrum Sharing) feature, the 5G BS RF requirements in FR1 should remain stable, considering limited modifications when required. In the following we area listing the different areas for improvement and for further study above 7 GHz.

Proposal 7: 6G requirements need to be designed to enable compatibility with 5G operation and MRSS.

For bands above 6 GHz (including n104), channel bandwidth wider than 100 MHz should be considered and the corresponding requirements should be specified. Based on RAN1 discussion and future agreements, at least 200 MHz channel bandwidth should be introduced while other larger channel bandwidths if necessary, should be further studied (for spectrum above 7 GHz for example), see our other contribution ([5]) for more details.

Proposal 8: Current assumption is 200MHz channel bandwidth for bands in or above 6GHz and further study the need for other wider channel bandwidths, depending on the conclusions on system parameters.

When AAS BS concept was introduced in 3GPP, RAN4 initially focused on BS architecture supporting digital beamforming (e.g. for FR1) or analog beamforming (e.g. for FR2). When considering new spectrum above 6 GHz, there is an increasing need to consider larger antenna arrays to maintain coverage comparing to mid bands deployment, increasing significantly the number of antenna elements.

To address the limited number of layer 1 logical ports, different types of port expansion techniques are required to map signals from logical ports to physical antenna elements. This could be done by combining elements as vertical sub-arrays. This combination is done using passive combination networks in the Radio Distribution Network (RDN) and often in conjunction with digital port expansion between baseband and radio. For large arrays, the vertical sub-array tends to be very large which would restrict the vertical steering range. Therefore, hybrid beamforming schemes are often considered to add steering capabilities in the RDN by adding analogue RF components, such as phase shifters, switched delay lines, etc. With the introduction of hybrid beam forming schemes, beam forming is applied at multiple stages (passive sub-arrays, analog RF phase-shifters and digitally before baseband processing) in combination. To model spatial characteristics properly an update to the array antenna model used for co-existence evaluation may be required.

Consequently, hybrid beamforming type of architecture should be considered and RAN4 specification should clarify this concept for BS type 1-H, most likely introducing a new BS type that would better correspond to the hybrid beamforming type of architecture and facilitate the transition from 1-C to 1-O. The requirements (conducted or OTA) for this new BS type would need further discussion.

Proposal 9: Further study BS hybrid beamforming type of architecture, possibly introducing a new BS type.

Some RF requirements (e.g. additional spurious/coexistence or in-band blocking requirements) have been specified based on simulation models and assumptions made in 1999, considering UTRA type of deployment, with fixed antenna and same gain in the 2 GHz frequency range. The 6G networks, especially in the upper FR1 and above, will be built differently, considering denser network grid, AAS BS with dynamic beamforming. For AAS BS requirement derivation it would then make sense to consider the dynamic behaviors and the statistical points on a CDF, and not the worst-case scenarios like it was done before, when UTRA RAT was specified.

It is also reasonable to assume that AAS will take a more dominating role in 6G for both higher frequencies and lower frequencies. Considering maintain coverage as at 2 GHz, the number of elements will grow rapidly in the upper region of FR1, while for lower region of FR1 RAN4 needs to establish relevant antenna parameters for AAS BS.

In the 6G context, RAN4 should then re-evaluate those requirements based on more relevant assumptions considering dynamic AAS behaviour, outside carrier gain characteristics, antenna size and more recent deployment parameters. A tentative requirement re-evaluation study has been initiated for the BS coexistence requirements in the scope of Rel-19. As the time of this document was written, it was not sure if RAN4 would be able to conclude on the coexistence requirements, but this attempt could be used as a starting point to reevaluate other relevant requirements (e.g. the in-band blocking requirement), considering as well that bands in the upper FR1 are much wider.

Proposal 10: If not finalized in Rel-19, further study BS coexistence requirement (additional spurious).

Also, other requirements were derived for UTRA have been re-used for 4G and 5G. For 6G it be worth to re-evaluate in-band blocking requirement based on for 6G relevant deployment assumptions. For bands in the upper region of FR1 it is expected that AAS BS with large antenna arrays will dominate. For BS with large array, a single receiver branch will see a fraction of the interferer signal seen by a non-AAS with a single column passive antenna. Therefore, RAN4 need to re-evaluate receiver in-band blocking requirement levels for BS operating in the frequency region where large array antennas are expected to be used.

For BS-type 1-O, out-of-band blocking requirement is defined as a field strength level applicable for all bands. Considering AAS BS with large antenna arrays, depending on frequency and array geometry the relation between in-band blocking and out-of-band blocking requirement levels may need further considerations.

Proposal 11: Re-evaluate some requirements (e.g. in-band blocking, out-of-band blocking) assuming today’s actual deployments instead of assumptions from 1999 and UTRA deployments.

In Rel-16 (2020), RAN4 studied the 7-24 GHz frequency range. With 6G, new spectrum in that frequency range will most likely be allocated to IMT, e.g. the WRC-27 agenda includes consideration of 4.4-4.8 GHz, 7.125-8.4GHz and 14.8-15.35 GHz for IMT allocation. RAN4 did additional studies in those frequency ranges ([3]), answering the ITU LS which was requesting characteristics of IMT systems deployed in those frequency ranges.  The outcomes of the 7-24GHz study ([2]) and those additional studies (captured in [3]) would be used as inputs when specifying any new band in this frequency range. Nevertheless, RAN4 should re-assess some of the 7-24GHz SI outcomes, consolidating them with the additional studies done to reply to ITU LS. For 6G it is essential to identify vital RF characteristics, such as filter suppression, PA characteristics (including efficiency and ACLR), phase noise characteristics, noise figure characteristics needed to derive relevant requirement levels. RAN4 should then agree on the architecture, possibly on PA model but more important on the phase noise model. Those inputs are key to correctly specify BS RF requirements in this new frequency range.

Proposal 12: Re-assess 7-24GHz study outcomes and its considered models (i.e. PA, phase noise).

In the Rel-19 IMT work, coexistence was evaluated in terms of ACLR and ACS for BS and UE. In addition, also additional transmitter spurious emission for BS needs to be considered. In the similar manner as for Rel-19 re-evaluation of transmitter additional spurious emission for selected bands in the upper region of FR1.

Proposal 13: Re-use coexistence study outcomes for 7.125-8.4 GHz and 14.8-15.35 GHz.

Moreover, further discussion would be needed if a new frequency range should be introduced and/or if the existing ones (FR1 and FR2) should be extended. For the RF specification BS types (1-C, 1-H, 1-O and 2-O) have been introduced per FR. The background for that is that the requirement applicability, requirement definitions and requirement levels are different for different FRs. This aspect is further discussed in our other contribution related to 6G spectrum ([4]).

With 5G and AAS BS, RAN4 introduced many instantiations of the BS maximum output power (15 symbols have been defined in total, i.e. maximum carrier output power measured per antenna connector, maximum carrier output power per TAB connector TX min cell group, maximum carrier output power per TAB connector, Maximum carrier TRP output power measured at the RIB, ...). While each definition has its own rationale and justified usage when specifying requirements, it would be worth, in the scope of this 6G study, to further study if it would be possible to simplify how power is handled in RAN4 specification, eventually considering PSD based requirements instead when relevant.

Proposal 14: Re-visit how BS output power is handled in relevant TS 38.104 requirements and consider introducing PSD based requirements instead.

6G will introduce new features like ISAC. When relevant, coexistence studies might be needed and new or additional RF requirements would have to be specified. RAN4 would need to decide how to introduce those new features support, including the corresponding RF requirements in the 6G core specifications or creating new specifications like it was done for NR IAB or NR NCR.

Observation 1: Coexistence studies might be needed with the introduction of new 6G features (e.g. ISAC).

Observation 2: RAN4 should discuss how to specify RF requirements for the new 6G features (e.g. ISAC), creating or not a new Technical Specification.

With the introduction of new bands above 7 GHz, RAN4 will have to evaluate the expected measurement uncertainties associated to RF requirements testing. In the work in Rel-19 related to band n104, some potential improvements related to bands in the upper region of FR1 was identified.

The evolution of the OTA test methods towards 6G introduction is mainly affected by the trend towards increasing number of elements in the antenna array (e.g. over 1024 elements and higher) intended for operation within new bands above FR1 and higher total output power. These two parameters have direct impact on the BS physical properties as size, weight and power consumption, which directly affect the measurement distance, size of the OTA chamber, output power of the interfering signals and environmental control of OTA test range.

Among the challenges we see when 6G BS will end up on the testing, we see the need for measurements performed at distances which are smaller than far-field, especially for BS with very large antenna arrays (e.g. 1024 antenna elements or more) and also very large output power. The new frequency bands for 6G might also introduce new challenges (e.g. need for new absorbers in the OTA chamber, etc.), but these aspects are expected to be manageable and not impact the 3GPP specifications.

Description of OTA measurements for total radiated power (TRP) as presented now in the Annex I in TS 38.141-2 is already under discussion in Rel-19 and we hope a number of improvements will be adopted before copy-pasting this annex into the new 6G specifications. One of the important aspects are the definition of a reference measurement method for TRP, revision of some of current methods and eventually removal of the obsolete ones. Handling the reverberation chamber, as an alternative to the anechoic chamber, is also a subject for improvement.

A few enhancements can be also added when measuring TRP. One is to use a concept that we already introduced, but without giving many details, the beam sweeping.  There is significant potential to reduce the density of the TRP measurement grid if the beams can be swept during the measurement time. A careful choice of the sweeping period, together with proper setting of the measurement device can lead to very fast measurements compared to the ones currently described. Beam sweeping would also better mimic normal operation.

Pre-scan procedure, used to further reduce the test time in case of spurious emissions, can be enhanced so it can deliver EIRP or even TRP values that can be utilized for deciding compliance, without the need for a very large margin (e.g. 30 dB as considered today). The use of reverberation chamber during pre-scan can also help, as measurements can be made faster with a smaller number of independent samples. More precise measurements can be later performed in the same setup.

Proposal 15: Study OTA test methods with the intention to improve description and measurement uncertainties for bands in the upper region of FR1 and above.

## Timing and synchronization

### Methodology of 6G time & sync requirement design

Introducing new features and functions demands a balanced approach that considers cost, complexity, and benefits. To ensure value and effectiveness, it is essential to conduct a comprehensive background analysis for each proposed feature or function. This analysis should determine whether synchronization is necessary, explain the reasons behind this need, outline the specific conditions or assumptions under which it is required, and define the appropriate level of synchronization needed. This is shown in Figure 1.

During each BS requirement discussion, the first objective should be whether the feature or function could be designed to operate effectively without synchronization or strict synchronization. Additionally, synchronization or strict synchronization can be optionally leveraged to enhance the feature or function when available. The strict conditions can be optionally leveraged to enhance the feature or function when available.

Figure 1: General approach regarding the need for synchronization

This strategy can be tailored to specific user equipment (UE) capabilities. For instance, a UE with advanced capabilities might not need synchronization or strict synchronization, whereas other UE implementations might conditionally support the complete feature/function, or provide enhancements to the feature/function, if certain synchronization performance levels are met.

Observation 3: Its essential to conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Robustness and resilience will be crucial for many 6G features and functions. Designing features or functions that rely on strict requirements could compromise these qualities. For example, when we discuss a timing synchronization requirement, we can deduce that a less strict synchronization requirement will be more tolerant to temporary loss of synchronization and help to enable a more robust network, feature or function. When RAN4 discuss the 6G requirement, RAN4 should consider the trade off between a strict requirement and robustness of the total system.

On the one hand, a more relaxed requirement leads to a feature or a function that can work in more deployment configurations which enables the operator to deploy in a flexible way. On the other hand, a stricter level of synchronization could in some cases enhance the feature or function and/or allow a larger set of UE types to be supported. On the other hand, a stricter level of synchronization could in some cases enhance the feature or function and/or allow a larger set of UE types to be supported. Therefore, RAN4 should not always aim to design a strict requirement but also consider the flexibility of the real field deployment.

Its essential to conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

For requirement clarity link each requirement to specific features or functions (which may be optionally supported), when needed clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints, like robustness—and, where applicable, to specific UE capabilities.

Proposal 16: When RAN4 defines time and sync requirements, RAN4 should conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Proposal 17: When RAN4 defines time and synchronization requirements, it should consider system robustness and deployment flexibility. Features and functions should be designed to operate effectively with a base level of synchronization, ensuring reliable performance in common and typical scenarios, even without strict synchronization. Where possible, the design may optionally utilize strict synchronization to enhance performance or extend support to less common use cases in situations where such synchronization can be achieved.

Proposal 18: When RAN4 defines time and synchronization requirements, for requirement clarity link each requirement to specific features or functions, when needed clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints—and, where applicable, to specific UE capabilities.

#### What and if to specify

Consider whether the synchronization requirements of a feature or function really need to be specified in the standard. Often, a sync requirement cannot be adequately captured by a single figure, as there generally are complex and varying input parameters associated with it. In the worst-case scenario, specifying a value based on the most challenging dimensioning can lead to increased costs and complexity, which will not be necessary in most scenarios. If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Let us discuss an important example. In Figure 2, we know that:

RTD= TAE+ ∆RF_Propagation+Channel_dispersion

If the RAN4 requirement is specified as Maximum Receive Time difference, MRTD then it is possible to trade TAE and ∆RF_Propagation, within a total budget.

Figure 2: Receive time difference example

Proposal 19: If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system., like defining MRTD in RRM and avoid stating TAE between ARP.

### Transmitter transient period

Due to strong inter-dependencies, TX transient times shall not be set or discussed isolated from (RRM), TX off level (RF), TDD cell phase sync (RRM), and TX ⇔ RX  switching times (RRM, RF).

The RRM requirementin RRM specification is related to Cell Phase Synchronization and TDD transient times in RF specifications through the formula:

NTA offset≥Cell_Phase_Synchronization+Transmitter transient period

It also needs to accommodate sufficient time for TX<=>RX switching. This means that, Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together.

Observation 4: NTA offset (RRM) , Cell Phase Synchronization (RRM) and TDD transient times (RF) are inter dependent and has to be discussed together, since:

NTA offset≥Cell_Phase_Synchronization+Transmitter transient period.

## EMC

The development of 6G standards presents a suitable opportunity to review the structure of Electromagnetic Compatibility (EMC) specifications. The current framework has evolved with each new generation of technology:

- TS 36.113, Evolved Universal Terrestrial Radio Access (E-UTRA); BS (BS) and repeater Electromagnetic Compatibility (EMC)

- TS 37.113, NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) BS (BS) Electromagnetic Compatibility (EMC)

- TS 37.114, Active Antenna System (AAS), BS (BS) Electromagnetic Compatibility (EMC)

- TS 38.113, NR; BS (BS) ElectroMagnetic Compatibility (EMC)

- TS 38.114, NR; Repeaters ElectroMagnetic Compatibility (EMC)

- TS 38.175, NR; Integrated access and backhaul ElectroMagnetic Compatibility (EMC)

A rationalized structure for 6G could improve clarity, ease of use, and long-term maintainability. The following text outlines three potential options for structuring the 6G EMC specifications for network nodes. Each option offers a different approach with advantages and disadvantages.

One option is to consolidate all EMC requirements into one comprehensive specification, i.e., a single specification would contain EMC requirements for all RATs for BS, Repeater, and IAB. Equipment-specific requirements/test setup/configurations would be in separate clauses.

Another option is to merge all BS-related EMC specifications while maintaining separate specifications for other type of equipment, i.e., one specification for BS including single RAT, AAS and multi-RAT operation (covering TS 36.113, TS 37.113, TS 37.114, TS 38.113 and 6G BS), and separate specifications for Repeaters and IAB nodes.

The third option is to continue the existing methodology by creating new, separate specifications for each 6G equipment type, i.e., new individual EMC specification for 6G BS/Repeater/IAB, and existing specifications for previous technologies would remain unchanged.

Table 1: Comparison of the pros and cons of the three options

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: A single unified specification | - Provides a single reference point for EMC requirements for all RATs for all types of network nodes. Make maintenance work simple. A single change request can update requirements for all equipment types. | - May result in a large document, potentially making it difficult to navigate. - Careful construction is needed to prevent the requirements/test setup/configurations of one equipment type from being incorrectly applied to another. |
| Option 2: A consolidated BS specification with separate specification for Repeaters and IAB | - Reduce the overlap that currently exists between the various BS EMC specifications. - Maintains a more focused scope for each individual specification. | - While reduced, the effort to maintain multiple specifications remains. |
| Option 3: Maintain the current structure | - Follows a well-established and understood process. - Allows each document to be focused on a single equipment type. | - Continue the current maintenance overhead with updating repeated text in multiple documents. - Lead to an increase in the number of specifications over time. |

Observation 5: The following three options could be considered for 6G EMC specification structure regarding network nodes:

- Option 1: A single unified specification

- Option 2: A consolidated BS specification with separate specifications for Repeaters and IAB

- Option 3: Maintain the current structure

Our preference is Option 2, as it maintains a more focused scope for each type of network node and ensures a reasonable workload on the EMC specifications within RAN4.

Proposal 20: Adopt Option 2 for the 6G EMC specification structure: a consolidated BS EMC specification with separate EMC specifications for Repeaters and IAB.

# Conclusion

In this contribution, we discussed several key items to be further studied in the scope of the 6G SI. We made the following observations and proposals:

Proposal 1: The fundamental concepts of Operating bands, Frequency Ranges, Base Station classes, separate conducted and OTA requirements, MSR BS and AAS BS specifications to be applied also for 6G, with appropriate improvements and simplifications.

Proposal 2: The complexity of BS specifications must be minimized in terms of number of options, variations and combinations of features. Requirements can be re-used from NR but need to be re-evaluated and if possibly simplified and improved in terms of definitions.

Proposal 3: When introducing 6G RAT in MSR specifications, RAN4 should further evaluate the need of specifying new category sets supporting 6G with GSM and UTRA.

Proposal 4: In the 6G scope, FR2-1 support should be added to MSR AAS specifications.

Proposal 5: When introducing 6G RAT, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.104 and TS 37.105) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.104 and TS 37.105).

Proposal 6: Similar and aligned with the core specifications decision, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2).

Proposal 7: 6G requirements need to be designed to enable compatibility with 5G operation and MRSS.

Proposal 8: Current assumption is 200MHz channel bandwidth for bands in or above 6GHz and further study the need for other wider channel bandwidths, depending on the conclusions on system parameters.

Proposal 9: Further study BS hybrid beamforming type of architecture, possibly introducing a new BS type.

Proposal 10: If not finalized in Rel-19, further study BS coexistence requirement (additional spurious).

Proposal 11: Re-evaluate some requirements (e.g. in-band blocking, out-of-band blocking) assuming today’s actual deployments instead of assumptions from 1999 and UTRA deployments.

Proposal 12: Re-assess 7-24GHz study outcomes and its considered models (i.e. PA, phase noise).

Proposal 13: Re-use coexistence study outcomes for 7.125-8.4 GHz and 14.8-15.35 GHz.

Proposal 14: Re-visit how BS output power is handled in relevant TS 38.104 requirements and consider introducing PSD based requirements instead.

Observation 1: Coexistence studies might be needed with the introduction of new 6G features (e.g. ISAC).

Observation 2: RAN4 should discuss how to specify RF requirements for the new 6G features (e.g. ISAC), creating or not a new Technical Specification.

Proposal 15: Study OTA test methods with the intention to improve description and measurement uncertainties for bands in the upper region of FR1 and above.

Observation 3: Its essential to conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Proposal 16: When RAN4 defines time and sync requirements, RAN4 should conduct a comprehensive background analysis for each feature and function, where such analysis should address if and why synchronization is necessary, the conditions or assumptions under when it is required, and what level of synchronization needed.

Proposal 17: When RAN4 defines time and synchronization requirements, it should consider system robustness and deployment flexibility. Features and functions should be designed to operate effectively with a base level of synchronization, ensuring reliable performance in common and typical scenarios, even without strict synchronization. Where possible, the design may optionally utilize strict synchronization to enhance performance or extend support to less common use cases in situations where such synchronization can be achieved.

Proposal 18: When RAN4 defines time and synchronization requirements, for requirement clarity link each requirement to specific features or functions, when needed clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints—and, where applicable, to specific UE capabilities.

Proposal 19: If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system., like defining MRTD in RRM and avoid stating TAE between ARP.

Observation 4: NTA offset (RRM) , Cell Phase Synchronization (RRM) and TDD transient times (RF) are inter dependent and has to be discussed together, since:

NTA offset≥Cell_Phase_Synchronization+Transmitter transient period.

Observation 5: The following three options could be considered for 6G EMC specification structure regarding network nodes:

- Option 1: A single unified specification

- Option 2: A consolidated BS specification with separate specifications for Repeaters and IAB

- Option 3: Maintain the current structure

Proposal 20: Adopt Option 2 for the 6G EMC specification structure: a consolidated BS EMC specification with separate EMC specifications for Repeaters and IAB.

# References

- RP-251881, Study on 6G radio

- TR 38.820, Study on the 7 to 24 GHz frequency range for NR

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz

- R4-2513273, Considerations on 6G Spectrum and 6G Band Combinations, Ericsson

- R4-2513116, System parameter considerations for 6G, Ericsson