3GPP TSG-RAN WG4 Meeting #116bis	R4-2513345

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.1.2

Source:	MediaTek Inc.

Title:	Simulation results on 6Rx UE demodulation performance requirements

Document for:	Information

# Introduction

In this contribution, we provide simulation results for 6Rx PDSCH and SDR requirements.

- Simulation results for PDSCH demodulation performance

The simulation assumptions for PDSCH requirements presented in Table 1 are derived from the WF [1]. Notably, the results for 6 layers are based on Rel-15 DMRS, utilizing antenna ports {1000-1003, 1004, 1006}.

Table 1. Simulation Assumptions for PDSCH perforamnce requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: Refer to Issue 2-2-1 and 2-2-2 - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers - ULA Low: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: DMRS ports {1000-1003, 1004, 1006} |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  |  | SNR@70% max TP |

4 layers results

The simulation results for 4 layers case are provided in Figure 1 and 2 and SNR values achieving 70% of maximum throughput are summarized in Table 2.

| Figure 1. Simulation results for 4T6R4L with MCS13 (Table 1) and non-zero MIMO correlation, α = 0 and β = 0.07179. | Figure 2. Simulation results for 4T6R4L with MCS17 (Table 1) and zero MIMO correlation, α = 0 and β = 0. |
| --- | --- |

Table 2. Summary for the required SNR achieving 70% maximum throughput (4T6R4L)

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 4 | 40 / 30 | MCS13 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0 and β = 0.07179 | 70 | 16.32 |
| Rank 4 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 4x6 ULA with α = 0 and β = 0 | 70 | 12.47 |

6 layers results

The simulation results for 6 layers case are provided in Figure 3 and SNR values achieving 70% of maximum throughput are summarized in Table 3.

Figure 3. Simulation results for 8T6R6L with MCS17 (Table 1) and zero MIMO correlation, α = 0 and β = 0.

Table 3. Summary for the required SNR achieving 70% maximum throughput (8T6R6L)

| Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| Rank 6 | 40 / 30 | MCS17 (MCS Table 1) | TDLA30-10 | 8x6 ULA with α = 0 and β = 0 | 70 | 18.83 |

# Simulation results for SDR requirements

We provide simulation results based on Table 5.5A-1 in TS38.101-4. Also, we use the static channel used for 8T6R (fixed precoder with i1,1=i1,2=i2=0) and 2T6R (random precoder) agreed in [1]. It is noted that there is no user data scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz and no user data scheduled on special slots according to Clause 5.5A.1 in TS38.101-4. The SNR values achieving 85% of maximum throughput are summarized in Table 4.

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5A-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz, 30 kHz / 10 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 15 kHz / 15 MHz, 30 kHz / 20 MHz, 30 kHz / 25 MHz and 30 kHz / 30 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For Number of Tx = 2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder index 0 and 2 For Number of Tx= 4 and 8: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | wideband |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | Rank 2/4: 1 Rank 8: 2 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000-1003, 1004, 1006} for 6 Layers CCs {1000-1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers, 6 Layers and 8 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PUCCH HARQ ACK spaitial bundling |  |  |  | Not configured |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x1 or 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 or 2x6 or 2x8 |
|  | 4 layers CCs |  |  | 4x4 or 4x8 |
|  | 6 layers CCs |  |  | 8x6 |
|  | 8 layers CCs |  |  | 8x8 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Es |  |  | dBm/Hz | -112 for MCS indexes in Table 5.5A-5: -110.5 For 1024QAM MCS indexes in Table 5.5A-6: |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |  |  |  |  |

Table 4. Summary for the required SNR achieving 85% maximum throughput

| MCS | SNR(dB)@85% of maxTP |  |  |
| --- | --- | --- | --- |
|  | MCS Table 1 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 2 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 4 2T6R2L Random precoder |
| 28 | 21.2 |  |  |
| 27 | 19.2 | 26.2 |  |
| 26 | 18.1 | 25.1 |  |
| 25 | 17.1 | 24.1 | 31.6 |
| 24 | 16.1 | 23.1 | 27.1 |
| 23 | 15.0 | 22.0 | 24.8 |
| 22 |  | 20.3 |  |

# Reference

- R4-2512617, Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon