3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513353

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Inputs on RAN4 6G Spectrum

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.6 Spectrum [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#109, the RAN led 6G Study objectives were updated in [1] adding the RAN4 related topics. In this contribution, we share our views on spectrum for 6GR.

# Discussion

## 2.1 6G SI objectives relevant to UE RF

The portion of the 6G SI objective most relevant to the UE RF requirements is reproduced below in italics and highlighted in yellow.

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

The detailed objectives of the study are:

Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

- Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

- Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

6GR core and performance requirements

- General RF aspects [RAN4]

- Intra-3GPP co-existence studies

- Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

- BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

- Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

- UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

- Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

- RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

- Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

- Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

- Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

- Handling irregular channel bandwidths including the definition [RAN4, RAN1]

- Definition of ‘frequency range(s)’[RAN4]

Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

- PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

- Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

- Aspects of integration with communication services [RAN1]

- higher layer procedures and protocol aspects [RAN2]

- RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In this contribution, we cover spectrum considerations for 6GR.

## 2.2 Singe TS for all 6GR Frequency Ranges

5G requirements cover two frequency ranges (FR):

- FR1: up to 7.125GHz

- FR2:

  - Non-NTN UEs: two sub-ranges:

      - FR2-1: 24.25 – 52.6GHz

      - FR2-2: 52.6 – 71GHz

  - NTN UEs: 17.3 – 30GHz, aka. FR2-NTN

RF requirements are verified in conducted test conditions for FR1 and radiated or “over-the-air” (OTA) for FR2.

The SI for 6G identifies at least three frequency ranges:

- FR1: up to 7.125GHz

- FR2-1: 24.25GHz – 52.6GHz. Frequencies beyond 52.6GHz are not in scope.

- A “in between FR1 and FR2-1” FR, where inputs from TR 38.921 and TR 38.922 will be taken into account, e.g., 7125-8400MHz, 10.0-10.5GHz, 14.8-15.35GHz. For simplicity, we name throughout this document this 7-24GHz range as “FR3”.

In FR3, it may no longer be possible to draw a clear-line between frequency bands for which RF requirements should be specified only under conducted and bands which should be specified only under OTA test conditions. RAN4 should not preclude any UE implementation. Both conducted and radiated RF requirements may be needed for certain bands within FR3 so that UEs supporting beam-forming using an FR2-like antenna implementation and UEs supporting an FR1-like implementation are both supported. Practical considerations on the performance of the RF test connectors per UE type may be a deciding factor to set a higher frequency limit to verifying the RF requirements under conducted test conditions. For example, surface mounted test connectors in FR1 smartphones may be deliver sufficiently good performance up to ~11-12GHz [2].

Observation: Depending on the types of UE, a given frequency band within FR3 could be supported both by an FR2-like antenna implementation (e.g. arrays with dual polarization), hereby supporting some level of beam forming, while for some other UEs the same band could be supported using FR1-like antennas (non-beam forming other than MIMO)

Proposal on TS handling:

Consider developing a single technical specification document for 6G where RF requirements are no longer split per frequency ranges but by type of RF requirements like done for NTN in 38.101-5, i.e.

        - Conducted transmitter/receiver characteristics for UEs not supporting beamforming via antenna arrays, e.g. FR1-like

        - Radiated transmitter/receiver characteristics for UEs supporting beam-forming via antenna arrays, e.g. FR2-like.

To support different types of UE implementations, RF requirements for a given frequency band within FR3 could be specified both conducted and radiated.

## 2.2 Handling of self-interference between FR1-FR3

For 5G, RAN4 assumed that no interference occurs between FR1 and FR2, hence no inter-band FR1-FR2 MSD are specified.

For a 6G UE supporting a given frequency band within FR3, we may consider two options:

        - UE not supporting beam-forming (e.g. FR1-like): FR1-FR3 interference may be considered for MSD analysis. However, it is unlikely that all FR1-FR3 scenarios would need to considered, for example is MSD analysis needed for say inter-band operation between a band below 2.5GHz and a band above 7GHz?

        - Fpr UE supporting beam-forming in FR3, ignore FR1-FR3 interference.

Proposal on FR1-FR3 interference:

  - Do not consider FR1-FR3 interference for UEs supporting beam-forming in an FR3 frequency band (FR2-like antenna implementation)

  - For UEs not supporting beam-forming in an FR3 band, further discuss criteria for which self-interference may need to be considered. For example, ignore self-interference for inter-band configurations of FR1 <2.5GHz + FR3, etc..

## 2.3 Handling Band Combination Requirements

5G has specified thousands of inter-band CA band combinations (BC) within FR1, and thousands of inter “range” FR1-FR2 band combinations. And hundred of new band combinations are introduced at every RAN4 meeting. This leads to a very complex TS and in some cases some tedious RF requirements. As a results, RAN4 has specified hundreds of inter-band CA MSD requirements. Eventhough inter-band CA MSD simplification using Look-Up Tables will help reduce some of this work-load, both TS 38.101-1 and 38.101-3 will remain very complex due to the large number of band combinations.

At these early stages of the 6G discussions, deployment scenarios remain unclear. Depending on operator’s views, there may be opportunities to significantly reduce the 5G BC’s complexity:

    - If 6G is to be deployed on stand-alone core-network,

    - can we assume that inter-band 5G-6G dual-connectivity is not a supported scenario, i.e. no longer needed? This would at least considerably simplify MSD requirements.

    - If inter-band 5G-6G DC is to be supported, can we simplify the RF requirements by no longer supporting dynamic power sharing?

    - can we assume that intra-band 5G-6G CA is not a valid deployment scenario?

    - Regarding inter-band CA, if in the long term 6G were to re-farm 5G bands, is there a way to simplify RF requirements by considering the concept of band groups rather than individual discrete band combinations ? For example, could RAN4 develop same RF requirements that would apply equally to any LB-MB CA combination (Low-Band / Mid-Band) no matter which band from the LB group is aggregated with which band from the MB group?

    - If the concept of band group specific requirements were to be adopted, how many band groups shall RAN4 consider and which criteria shall be considered for say MSD analysis?

    - For example, the concept of band groups agreed to detect if triple-beat MSD requirements are needed could could be extended here to cover FR1+FR2+FR3. One may consider the following band group split:

    - For FR1-like antenna implementations 0.3-0.6GHz, 0.6-1.25GHz, 1.25-2.5GHz, 2.5-5GHz, 5-10GHz?

    - For FR2-like antenna array implementations: 10-20GHz, 20-40GHz, 40-52.6GHz?

Regarding UL CA, 5G currently supports:

    - For CA within FR1, a maximum of 2UL bands and up to 3CCs within a maximum of 2 UL clusters, meaning that UL CA_nX-nY(2A) is not supported, but CA_nX-nYB/C is supported.

    - For NR DC between FR1 and FR2, a maximum of 2UL bands, with FR2 band supporting >2 UL CCs eg. DC_n41A_n260M,

Settings boundaries for 6G may also help reduce TS complexity while still enabling an improvement over 5G. For example one may consider restricting UL CA aggregating two bands with conducted RF requirements to a maximum of 2UL bands with up to 4CCs and a maximum of 2UL clusters.

Proposal on handling inter-band combinations

- Confirm if 5G+6G dual connectivity is out of scope from a network deployment perspective. If considered valid, can RAN4 no longer consider dynamic power sharing?

- Consider simplifying inter-band CA RF requirements by specifying band-group requirements rather than band combination specific requirements, eg. Develop LB-MB RF requirements that would apply equally to any LB aggregated to any MB ?

- Consider different band group split depending on type of UE/ type of RF requirement:

  - For conducted requirements / UE type with FR1-like antenna implementation, one example of band grouping could be 0.3-0.6GHz, 0.6-1.25GHz, 1.25-2.5GHz, 2.5-5GHz, 5-10GHz

  - For radiated requirements / UE type with FR2-like antenna array/beam forming support, one example of band grouping could be 10-20GHz, 20-40GHz, 40-52.6GHz.

- For inter-band UL CA aggregating two bands supported with conducted RF requirements, restrict to a maximum of 2UL bands with up to 4CCs and a maximum of 2UL clusters, eg. CA_nXB/C-nyB/C for band “x” and “y”.

## 2.4 Simplification of Intra-Band Contiguous CA Requirements

RF requirements for intra-band contiguous CA for 5G are rather complex. In addition 5G achieves poor uplink performance for non-contiguous RB allocations due to very high MPR. Considering that the 6G is looking at supporting single carrier operation with greater CBW than 5G, e.g. 200MHz single carrier, it may be worth considering the following simplifications for 6G.

Proposal on simplifying intra-band CA requirements:

- Do not introduce intra-band contiguous CA RF requirements when the CA aggregated bandwidth can be supported by single carrier operation. For example, in bands where 6G single carrier operation can be supported up to 200MHz CBW, do not introduce RF requirements for say intra-band CA aggregated CBW<= 200MHz.

- To optimize uplink coverage, restrict intra-band contiguous CA operation only to contiguous RB allocations,

- To minimize RAN4 workload and TS complexity, do not specify RF requirements for CC1/CC2 CBW that cannot be practically deployed in a given region, for example to account for spectrum ownership considerations.

# Conclusion

In this contribution, we provide our views on the Release 20 RAN4 6G SI spectrum related topics.

Proposal on TS handling:

Consider developing a single technical specification document for 6G where RF requirements are no longer split per frequency ranges but by type of RF requirements like done for NTN in 38.101-5, i.e.

- Conducted transmitter/receiver characteristics for UEs not supporting beamforming via antenna arrays, e.g. FR1-like

- Radiated transmitter/receiver characteristics for UEs supporting beam-forming via antenna arrays, e.g. FR2-like.

To support different types of UE implementations, RF requirements for a given frequency band within FR3 could be specified both conducted and radiated.

Proposal on FR1-FR3 interference:

  - Do not consider FR1-FR3 interference for UEs supporting beam-forming in an FR3 frequency band (FR2-like antenna implementation)

  - For UEs not supporting beam-forming in an FR3 band, further discuss criteria for which self-interference may need to be considered. For example, ignore self-interference for inter-band configurations of FR1 <2.5GHz + FR3, etc..

Proposal on handling inter-band combinations

- Confirm if 5G+6G dual connectivity is out of scope from a network deployment perspective. If considered valid, can RAN4 no longer consider dynamic power sharing?

- Consider simplifying inter-band CA RF requirements by specifying band-group requirements rather than band combination specific requirements, eg. Develop LB-MB RF requirements that would apply equally to any LB aggregated to any MB ?

- Consider different band group split depending on type of UE/ type of RF requirement:

  - For conducted requirements / UE type with FR1-like antenna implementation, one example of band grouping could be 0.3-0.6GHz, 0.6-1.25GHz, 1.25-2.5GHz, 2.5-5GHz, 5-10GHz

  - For radiated requirements / UE type with FR2-like antenna array/beam forming support, one example of band grouping could be 10-20GHz, 20-40GHz, 40-52.6GHz.

- For inter-band UL CA aggregating two bands supported with conducted RF requirements, restrict to a maximum of 2UL bands with up to 4CCs and a maximum of 2UL clusters, eg. CA_nXB/C-nyB/C for band “x” and “y”.

Proposal on simplifying intra-band CA requirements:

- Do not introduce intra-band contiguous CA RF requirements when the CA aggregated bandwidth can be supported by single carrier operation. For example, in bands where 6G single carrier operation can be supported up to 200MHz CBW, do not introduce RF requirements for say intra-band CA aggregated CBW<= 200MHz.

- To optimize uplink coverage, restrict intra-band contiguous CA operation only to contiguous RB allocations,

- To minimize RAN4 workload and TS complexity, do not specify RF requirements for CC1/CC2 CBW that cannot be practically deployed in a given region, for example to account for spectrum ownership considerations.

# References

[1] RP-252912 Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN4#109

[2] R4-1903994, On performance of existing RF connector and probe technology, LG Electronics Finland, RAN4#90b