# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513365

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.10.3

Source:	China Telecom

Title:	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN4 #116 meeting, RRM performance requirements of fast SCell activation for UE supporting Rel-18 EMR were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1] and summary [2]:

Issue 2-2-1: Test case list Further discuss this issue taking the options in the summary R4-2509055 into consideration. - Proposals - Direct SCell activation at SCell addition for FR2 - Consider this scenario: CATT, ZTE, vivo (Configuration 1 and 2), Huawei, Ericsson, Nokia - Do not consider this scenario: OPPO - Configuration 3 - Consider this configuration: CATT, OPPO, CMCC, ZTE, vivo, Huawei, Ericsson, Nokia - Do not consider this configuration: Apple - Recommended WF - Based on the majority view, define test cases for - Direct SCell activation at SCell addition for FR2 - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode

In our views, FR2 SCell activation delay reduction is important and can achieve significant gain with EMR report. Therefore, the scenario on Direct SCell activation at SCell addition for FR2 needs to be considered.

Proposal 1: Consider the scenario on Direct SCell activation at SCell addition for FR2.

The three configurations are included in the core part, in our views, different test cases covering different UE capability and NW configurations need to be considered. Therefore, the scenario with Configuration 3 needs to be considered. If UE support multiple configurations, UE is allowed to pass one of the test cases.

Proposal 2: Consider the scenario with Configuration 3.

Status in the WF [1] and summary [2]:

Issue 2-2-2: Applicability rules of test case Further discuss this issue taking the options in the summary R4-2509055 into consideration. - Proposals - Option 1: (CATT) - No applicability rules are required if the following test case list is agreed. Otherwise, option 1 (i.e., option 2 in this issue) can be a good starting point. TC No.; Scenario; Comment. 1; Normal SCell activation + FR1 + Configuration 1; Non-DRX. 2; Normal SCell activation + FR2 + Configuration 1; N/A. 3; Normal SCell activation + FR1 + Configuration 2; N/A. 4; Normal SCell activation + FR2 + Configuration 2; DRX. 5; Normal SCell activation + FR1 + Configuration 3; Non-DRX. 6; Normal SCell activation + FR2 + Configuration 3; N/A. 7; Direct SCell activation at SCell addition + FR1 + Configuration 1; N/A. 8; Direct SCell activation at SCell addition + FR2 + Configuration 1; DRX. 9; Direct SCell activation at SCell addition + FR1 + Configuration 2; Non-DRX. 10; Direct SCell activation at SCell addition + FR2 + Configuration 2; N/A. 11; Direct SCell activation at SCell addition + FR1 + Configuration 3; N/A. 12; Direct SCell activation at SCell addition + FR2 + Configuration 3; DRX. 13; PUCCH SCell activation + FR1 + Configuration 1; Non-DRX. 14; PUCCH SCell activation + FR2 + Configuration 1; N/A. 15; PUCCH SCell activation + FR1 + Configuration 2; N/A. 16; PUCCH SCell activation + FR2 + Configuration 2; DRX. 17; PUCCH SCell activation + FR1 + Configuration 3; N/A. 18; PUCCH SCell activation + FR2 + Configuration 3; Non-DRX. - Option 2: (Apple) - Introduce some applicability rule to reduce testing effort. - Agree the following testing applicability rules: - Rule 1: for UE supporting Direct SCell activation at SCell addition or PUCCH SCell activation, eEMR-based fast SCell activation for normal SCell activation can be skipped. - Rule 2: for UE supporting both Direct SCell activation at SCell addition and PUCCH SCell activation, eEMR-based fast SCell activation for Direct SCell activation at SCell addition can be skipped. - Rule 3: for UE which can pass R19 test case for fast SCell activation based on R18 early measurement report, corresponding R18 test case with valid report can be skipped. - Option 2a: (Huawei) - UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC. - Option 2b: (Qualcomm) - If a UE supports Direct SCell Activation or PUCCH SCell Activation, the Normal SCell Activation test case can be skipped. - Option 3: (OPPO) - For normal SCell activation and PUCCH SCell activation, choose only one set of legacy configurations (e.g., non-DRX, 640ms SCell measurement cycle) to define the test case for fast SCell activation. - Recommended WF Discuss the option 1 and 2 as a starting point.

In our views, if UE support multiple configurations or scenarios, in order to limit the numbers of test cases that UE is required to pass, UE can be allowed to pass one of the test cases. Therefore, it’s proposed to define applicability rules for the designed test cases.

Proposal 3: Applicability rules for the test cases can be considered.

Status in the WF [1] and summary [2]:

Issue 2-2-3: Test case design Further discuss this issue taking the options in the summary R4-2509055 into consideration. - Proposals for principle of defining test cases - Option 1: (CATT, CT) - RAN4 to reuse the legacy test cases for Rel-18 eEMR measurement with valid result reporting, i.e., A.6.6.9/A.7.6.23, and SCell activation, i.e., A.6.5.3/A.7.5.3. - Option 2: (Apple) - Basic principle is to reuse the legacy test cases as much as possible. For instance, RAN4 can add SCell configuration and activation procedure on top of R18 eEMR test cases - Option 3: (ZTE, Ericsson) - The performance of Rel-19 fast Scell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing Scell activation test case with the Idle/Inactive mode fast CA/DC reporting test case. - Option 4: (Nokia) - Discuss which combinations of EMR configurations are tested with each SCell scenario. - Proposals for details of test cases - Option 5: (CATT) - Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions: - For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell, 5*DRX cycles); - For target SCell in FR2, time gap needs to be larger than 4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4. - Option 6: (OPPO) - The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively. - T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation. - For direct SCell activation, T3 and T4 can be merged into one. - The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline. - The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline. - Option 7: (Qualcomm) - RAN4 to not define test cases for DRX - RAN4 to consolidate test cases for EMR capability-related configurations into unified test cases based on SCell activation types (Normal SCell activation, Direct SCell activation, and PUCCH SCell activation) - Within each consolidated test case, adapt the test flow to accommodate the specific UE capabilities. - Use conditional logic or branching within the test case to handle capability-specific behaviors. - Recommended WF - Discuss and polish the test procedure for normal SCell activation with Configuration 1 as a starting point: - Discuss whether the X1 and X2 in the test procedure are needed. - Discuss DRX configuration: - Alt 1: DRX is not configured in all test cases. - Alt 2: DRX is configured in some test cases. - Discuss whether to define a unified test case for all SCell activation types (Option 7). - Note: After the test procedure is concluded, it can be used as a template when drafting other test cases, while the corresponding legacy test cases can also be referred to. Test procedure for normal SCell activation with Configuration 1: The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively as the figure below illustrates: - T1: During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2 (SCell to be activated). UE is configured with measurement reporting for Cell 2. The time point when UE receives RRC_Release message from the TE defines the starting point of T2. - T2: At the beginning of T2, Cell 2 becomes detectable. Signal level of Cell 2 and the duration of T2 are set to fixed values. - T3: At the beginning of T3, the signal level of Cell 2 is set to another value. The duration of T3 equals to measIdleValidityDuration-r18. - T4: The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report within the duration of T4. - T5: At the beginning of T5, the TE sends a MAC message for activation of the SCell (Cell 2). The point in time at which the MAC message is received at the UE antenna connector is denoted as slot #n. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3.2A..

The requirements on Fast SCell activation for UE supporting Rel-18 EMR are based on the requirements on Rel-18 EMR and legacy SCell activation, thus the test cases for Rel-18 EMR measurement with valid result reporting and SCell activation can be the baseline to design the test cases for Fast SCell activation for UE supporting Rel-18 EMR.

Proposal 4: The test cases for Rel-18 EMR measurement with valid result reporting and SCell activation can be the baseline to design the test cases for Fast SCell activation for UE supporting Rel-18 EMR.

- Conclusion

The contribution presents our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR, with the following proposals:

Proposal 1: Consider the scenario on Direct SCell activation at SCell addition for FR2.

Proposal 2: Consider the scenario with Configuration 3.

Proposal 3: Applicability rules for the test cases can be considered.

Proposal 4: The test cases for Rel-18 EMR measurement with valid result reporting and SCell activation can be the baseline to design the test cases for Fast SCell activation for UE supporting Rel-18 EMR.

- References

- R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116, 25 – 29 August, 2025.

- R4-2509055, Topic summary for [116][212] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #116, 25 – 29 August, 2025.