# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513366

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.13.3.1

Source:	China Telecom

Title:	Discussion on RRM performance requirements for UE-to-UE CLI handling

Document for:	Discussion

- Introduction

In RAN4 #116 meeting, RRM performance requirements for UE-to-UE CLI handling were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for UE-to-UE CLI handling captured in the WF [1] and summary [2].

- Discussion

Accuracy requirements

Status in the summary [2]:

Issue 3-1-4: L1-SRS-RSRP – accuracy requirements - Proposals - Proposal 1 (CTC, HW, E///): - L1-SRS-RSRP measurements accuracy requirements are defined based on simulation results for the agreed side conditions. - Proposal 2 (HW): - Accuracy requirements apply for SRS resources from both intra- and inter-cell aggressor, as long as the side conditions are met. - Recommended WF - Suggest to agree on both P1 and P2.

For L1-SRS-RSRP accuracy requirements, in our views, L1-SRS-RSRP accuracy requirements should be defined based on simulation results for the agreed side conditions.

Proposal 1: RAN4 to define accuracy requirements for L1-SRS-RSRP measurements, and the accuracy requirements are defined based on simulation results for the agreed side conditions.

Status in the summary [2]:

Issue 3-1-5: L1-CLI-RSSI – accuracy requirements - Proposals - Proposal 1a (CATT, LGE, Samsung, CTC, HW, E///, MTK, QC): - Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Proposal 1b (Nokia): - Same accuracy requirements are defined for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Recommended WF - Suggest to agree on P1a which also includes with P1b.

For L1-CLI-RSSI accuracy requirements, R16 accuracy requirements for CLI-RSSI can be reused for L1-CLI-RSSI accuracy requirements since there is no signal power estimation, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Proposal 2: RAN4 to define accuracy requirements for L1-CLI-RSSI measurements, and RAN4 to re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Report mapping

Status in the summary [2]:

Issue 3-2-1: Absolute report mapping - Proposals - Proposal 1 (CATT, LGE, HW, E///): - Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively. - Recommended WF - Suggest to agree on P1 based on RAN4#114-bis agreements.

For absolute report mapping, it’s proposed to re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 3: Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Status in the summary [2]:

Issue 3-2-2: Differential report mapping - Proposals - Proposal 1 (CATT, LGE, HW, E///): - Use Table 1 and Table 2 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively Table 1: Reporting mapping of differential L1-SRS-RSRP Reported value; Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP); Unit. DIFF_SRS-RSRP_0; 0≥ΔSRS-RSRP>-2; dB. DIFF_SRS-RSRP_1; -2≥ΔSRS-RSRP>-4; dB. DIFF_SRS-RSRP_2; -4≥ΔSRS-RSRP>-6; dB. DIFF_SRS-RSRP_3; -6≥ΔSRS-RSRP>-8; dB. DIFF_SRS-RSRP_4; -8≥ΔSRS-RSRP>-10; dB. DIFF_SRS-RSRP_5; -10≥ΔSRS-RSRP>-12; dB. DIFF_SRS-RSRP_6; -12≥ΔSRS-RSRP>-14; dB. DIFF_SRS-RSRP_7; -14≥ΔSRS-RSRP>-16; dB. DIFF_SRS-RSRP_8; -16≥ΔSRS-RSRP>-18; dB. DIFF_SRS-RSRP_9; -18≥ΔSRS-RSRP>-20; dB. DIFF_SRS-RSRP_10; -20≥ΔSRS-RSRP>-22; dB. DIFF_SRS-RSRP_11; -22≥ΔSRS-RSRP>-24; dB. DIFF_SRS-RSRP_12; -24≥ΔSRS-RSRP>-26; dB. DIFF_SRS-RSRP_13; -26≥ΔSRS-RSRP>-28; dB. DIFF_SRS-RSRP_14; -28≥ΔSRS-RSRP>-30; dB. DIFF_SRS-RSRP_15; -30≥ΔSRS-RSRP; dB. Table 2: Reporting mapping of differential L1-CLI-RSSI Reported value; Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI); Unit. DIFF_CLI-RSSI_0; 0≥ΔCLI-RSSI>-2; dB. DIFF_CLI-RSSI_1; -2≥ΔCLI-RSSI>-4; dB. DIFF_CLI-RSSI_2; -4≥ΔCLI-RSSI>-6; dB. DIFF_CLI-RSSI_3; -6≥ΔCLI-RSSI>-8; dB. DIFF_CLI-RSSI_4; -8≥ΔCLI-RSSI>-10; dB. DIFF_CLI-RSSI_5; -10≥ΔCLI-RSSI>-12; dB. DIFF_CLI-RSSI_6; -12≥ΔCLI-RSSI>-14; dB. DIFF_CLI-RSSI_7; -14≥ΔCLI-RSSI>-16; dB. DIFF_CLI-RSSI_8; -16≥ΔCLI-RSSI>-18; dB. DIFF_CLI-RSSI_9; -18≥ΔCLI-RSSI>-20; dB. DIFF_CLI-RSSI_10; -20≥ΔCLI-RSSI>-22; dB. DIFF_CLI-RSSI_11; -22≥ΔCLI-RSSI>-24; dB. DIFF_CLI-RSSI_12; -24≥ΔCLI-RSSI>-26; dB. DIFF_CLI-RSSI_13; -26≥ΔCLI-RSSI>-28; dB. DIFF_CLI-RSSI_14; -28≥ΔCLI-RSSI>-30; dB. DIFF_CLI-RSSI_15; -30≥ΔCLI-RSSI; dB. - Recommended WF - Suggest to agree on P1 based on RAN4#114-bis agreements.

For differential report mapping, support Proposal 1 to use Table 1 and Table 2 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 4: Support Proposal 1 to use Table 1 and Table 2 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Test cases

Status in the summary [2]:

Issue 3-3-2: Test case list - Proposals Samsung; Nokia; CTC; CMCC; E///; HW; ZTE; MTK. L1-SRS-RSRP measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y; Y. L1-CLI-RSSI measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y; Y. SSB L1 measurement; Y; Y; Y. SSB L3 measurement; Y; Y. CSI-RS RLM; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS BFR; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-RSRP measurement; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-SINR measurement; Y; Y; Y; Y; Y. CSI-RS L3 measurement; Y; Y; Y. RACH; Y. - Recommended WF - Suggest to agree to define at least following test cases - L1-SRS-RSRP measurement (delay and accuracy) - L1-CLI-RSSI measurement (delay and accuracy) - CSI-RS RLM - CSI-RS BFR - CSI-RS L1-RSRP measurement (delay and accuracy) - Discuss whether to define following test cases - CSI-RS L1-SINR measurement (delay and accuracy) - SSB L1 measurement - SSB L3 measurement - CSI-RS L3 measurement - RACH

Based on the core part discussion in RAN4 previous meetings, there are two objectives been discussed and defined, i.e., RRM requirements for UE-to-UE CLI handling and RRM impacts for SBFD operation.

For RRM requirements for UE-to-UE CLI handling, L1-SRS-RSRP measurements and L1-CLI-RSSI measurements are considered for UE-to-UE CLI handling, it’s proposed to design UE-to-UE CLI handling test cases based on L1-SRS-RSRP measurements and L1-CLI-RSSI measurements.

Proposal 5: It’s proposed to design the following test cases

- L1-SRS-RSRP measurement (delay and accuracy)

- L1-CLI-RSSI measurement (delay and accuracy)

Status in the summary [2]:

Issue 3-3-4: Test case scenarios - Proposal 1a (Nokia): - RAN4 to define the test case list for SBFD in SA operation and wait for decision in RAN2 on the support of SBFD with DC. - Proposal 1b (E///): - Define test cases for single cell operation. - Proposal 2 (Nokia, CTC, HW, E///): - Define test cases for FR1 and FR2. - Recommended WF - Suggest to agree on P2, i.e. to define SBFD RRM test cases for both FR1 and FR2. - Wait for RAN2 conclusion, and further discuss whether to define test cases for EN-DC.

In our views, test cases for FR1 and FR2 need to be considered.

Proposal 6: RAN4 to define RRM test cases for both FR1 and FR2.

- Conclusion

The contribution presents our views on RRM performance requirements for UE-to-UE CLI handling, with the following proposals:

Proposal 1: RAN4 to define accuracy requirements for L1-SRS-RSRP measurements, and the accuracy requirements are defined based on simulation results for the agreed side conditions.

Proposal 2: RAN4 to define accuracy requirements for L1-CLI-RSSI measurements, and RAN4 to re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Proposal 3: Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 4: Support Proposal 1 to use Table 1 and Table 2 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 5: It’s proposed to design the following test cases

- L1-SRS-RSRP measurement (delay and accuracy)

- L1-CLI-RSSI measurement (delay and accuracy)

Proposal 6: RAN4 to define RRM test cases for both FR1 and FR2.

- References

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #116, 25 – 29 August, 2025.

- R4-2509061, Topic summary for [116][218] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #116, 25 – 29 August, 2025.