3GPP TSG-RAN WG4 Meeting #116bis           	R4-2513406

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.22.3

Source:	Samsung

Title:	View on demodulation requirement for IoT NTN TDD

Document for:	Discussion

- Introduction

In the RAN#106 meeting, the revised WID on introduction of IoT-NTN-TDD mode was approved with the following objectives [1]

- The work objectives assume the following - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band - The work item includes the following objectives - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition configuration (if needed ) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands - Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements) - Objective of Performance part WI - Specify the following - RRM performance requirements, as needed [RAN4] - Necessary SAN and UE demodulation requirements [RAN4] - SAN conformance testing requirements [RAN4]

According to the meeting agenda and work plan for this WI,  this is the 1st meeting to discuss demodulation requirements for SAN and UE to support IoT NTN TDD operation

In this contribution, the view on the test scope on the SAN and UE demodualtion requirement to suppport TDD operation was provided.

# 2	General Scope

In this section, the view on general scope of demodulation requirement for SAN and UE is provided.

Based on the WID, the new NB-IoT NTN TDD mode allows configuration the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with at least N=9 as baseline. Based on core requirement, the target band N=9 and minimum of D=8 and U=8 to be supported. And the new band 249 was introduced to support NB-IoT TDD operation.

Observation 1:  New TDD pattern with N=9 and minimum of D=8 and U=8 introduced for band 249 to support NB-IoT TDD operation.

For NB-IoT NTN in Rel-18, RAN4 has only introduced SAN and UE demodulation requirement for FDD operation. In LTE NB-IoT, the requirements with UE supporting both Half-duplex FDD mode and TDD mode has been specified. As for this TDD pattern, RAN4 could consider to define the requirement for both SAN and UE side to support IoT NTN TDD operation.

UE requirement

For UE demodulation, only NPDSCH requirement was introduced for NTN scenario, and no requirement introduced for NPDCH and NPBCH. Follow the similar process, we think only NPDSCH requirement could be considered for defining requirement for IoT TDD operation.

SAN requirement

For SAN side, both NPUSCH 1/2 with 15KHz and 3.75KHz SCS and NPRACH requirements were specified in Rel-17.

For 3.75KHz, the tx duration is large than 8ms, which can not meet the 8ms TDD Tx time, corresponding to D/U=8 TDD pattern.  Therefore, our preference is only to consider 15KHz SCS with NPUSCH1 and NPUSCH2. In order to align the TDD pattern, the related test parameters for FDD should be revised.

As for NPRACH, in current TN spec, the following requirements for TDD was introduced.

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD Number of TX antennas; Number of RX antennas; Repetition number; Propagation conditions and correlation matrix (Annex B); Frequency offset; SNR[dB]. Preamble format 0; Preamble format 1; Preamble format 0-a; Preamble format 1-a. 1; 2; 8; AWGN; 0; 5.6; 2.5; 4.0; 1.0. EPA1 Low; 200 Hz; 14.7; 10.1; 13.7; 9.0. 32; AWGN; 0; 0.9; -2.2; -0.7; -3.6. EPA1 Low; 200 Hz; 9.8; 4.3; 10.0; 2.5.

For NPRCH requirement to support TDD mode, we think the existing table can be considered as staring point. For NPRACH formats, similar as FDD, only the basic formats with F0 and F1 will be considered.

Proposal 1:  RAN4 could consider to define SAN and UE demodulation requirement to support NB-IoT TDD operation. FFS on the related test parameters to align with the TDD pattern (D/U=8ms)

- UE demodulation requirement:  NPDSCH

- SAN demodulation requirement:  NPUSCH 1 and NPUSCH 2 with 15KHz, NPRACH with format 0 and format 1

# 3	Test Setup

In this section, the view on the test setup for SAN and UE demodulation requirement to support TDD pattern

NPDSCH

The following is the existing test configuration for NPDSCH requirement for TDD defining in current TN spec, it can be considered as starting point for defining UE demodulation requirement to support NTN TDD operation

Table 8.12.1.2-1: Common Test Parameters Parameter; Unit; Value. Uplink downlink configuration (Note 1); 4. Special subframe configuration (Note 2); 4. Number of HARQ processes per component carrier (Note 4); Processes; 1 or 2. Maximum number of HARQ transmission; 4. Cyclic Prefix; Normal. eutraControlRegionSize-r13; 2 for In-band, N/A for Standalone/Guard-band unless otherwise stated. downlinkBitmap-r13 and dl-Gap-r13; Not configured. dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13; Not configured. Unused REs or RB (Note 3); OCNG. OCNG pattern; NB.OP.1. Note 1: As specified in Table 4.2-2 in TS 36.211 [4] Note 2: As specified in Table 4.2-1 in TS 36.211 [4] Note 3: For in-band mode, the REs for transmission of LTE signals including PDCCH, CRS should be filled by OCNG. Note 4: 1 is applicable for both UE Category NB1 and NB2; 2 is only applicable for UE Category NB2 if UE supports it.. Table 8.12.1.2-2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations Parameter; Unit; Value. DCI format; DCI format N1. scheduling delay field (); 1 for the first HARQ process; 2 for the second HARQ process if configured. (ack-NACK-NumRepetitions-r13); 1. ACK/NACK resource field; 0. Reference channel for NPDCCH; R.NB.3 TDD for one NRS antenna port; R.NB.4 TDD for two NRS antenna ports. (npdcch-Offset-USS-r13); 0.

Proposal 2: RAN4 could take the existing test configuration defining in current TN spec as starting point to define the UE demodulation requirement to support NTN TDD operation.

NPUSCH1 and NPUSCH 2

As our proposed, only 15KHz SCS for NPUSCH 1 and NPUSCH 2 TDD requirement will be considered. In current IoT NTN spec as 36.108, RAN4 has specified the following test cases for both NPUSCH format 1 and NPUSCH format 2

In current test, the length of Tx duration is 16ms and 32ms for NPUSCH1 and NPUSCH2, with configuring the number of repetitions is 16. Given the number of limitations for TDD operation, we would like to configure the number of repetitions as 8 for NPUSCH1, and the related frequency offset should be 4Hz with reusing the current FRC table. And the number of repetitions as 4 for NPUSCH 1, the related frequency offset should be 4Hz, respectively.

In summary, the following test case could be considered for defining the NPUSCH1 and NPUSCH2 requirement for TDD operation

Table 1: Minimum requirements for NPUSCH format 1, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex X) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |

Table 2: Minimum requirements for NPUSCH format 2, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN TDLA100-1 | 1 | 15KHz | 4Hz | 8 | 4 | TBD |
| 1 | 2 | NTN TDLA100-1 | 1 | 15KHz | 4 Hz | 8 | 4 | TBD |

Proposal 3: RAN4 could take the following test cases to define the NPUSCH format1 and format 2 requirement for TDD operation

- NPUSCH format 1

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex X) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |

- NPUSCH format 2

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN TDLA100-1 | 1 | 15KHz | 4Hz | 8 | 4 | TBD |
| 1 | 2 | NTN TDLA100-1 | 1 | 15KHz | 4 Hz | 8 | 4 | TBD |

NPRACH

TDD configuration

In current TN spec 36.104 for NPRACH requirement with related TDD scenario, as for TDD configuration, the following configuration was considered when defining the requirement

Table 8.5.3.2.1-2: NPRACH missed detection requirements for TDD Parameter; Value. Narrowband physical layer cell identity; 0. Initial subcarrier index; 0. Uplink-downlink configuration for TDD; 1. Special subframe configuration for TDD; 7. Where the LTE TDD configuration can be followed Uplink-downlink configuration; Downlink-to-Uplink Switch-point periodicity; Subframe number. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9. 0; 5 ms; D; S; U; U; U; D; S; U; U; U. 1; 5 ms; D; S; U; U; D; D; S; U; U; D. 2; 5 ms; D; S; U; D; D; D; S; U; D; D. 3; 10 ms; D; S; U; U; U; D; D; D; D; D. 4; 10 ms; D; S; U; U; D; D; D; D; D; D. 5; 10 ms; D; S; U; D; D; D; D; D; D; D. 6; 5 ms; D; S; U; U; U; D; S; U; U; D.

Since new TDD pattern with N, D/U=8 was introduced, the related TDD configuration can be updated.

Proposal 4: RAN4 can apply the TDD pattern with N, D/U=8 for related TDD configuration for NPRACH

Channel model

For channel model, since it is not applicable for AWGN channel in NTN scenario. Similar as FDD, we only consider fading channel for specifying NPRACH requirements with TDD operation, NTN-TDLA100-1 can be considered with including the frequency offset.

Proposal 5: RAN4 could apply the channel model with NTN-TDLA100-1 with 200Hz frequency offset for specifying NPRACH requirement with supporting TDD operation

Number of Repetition

The length of NPRACH and number of repetitions will determine the length of Tx duration. As our preference, the format 0 and format 1 are considered. The following are the structure of NPRACH formats in TDD scenario.

Table 3:  Structure of NPRACH formats in TDD scneario

| Format | Description | G | P | N | CP length | Nominal cell size |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Two symbol groups followed by a guard time fit into 1 UL subframe | 2 | 4 | 1 | 4778 Ts (~155.5us) | ~23.3km |
| 1 | Two symbol groups followed by a guard time fit into 2 UL subframes | 2 | 4 | 2 | 8192 Ts (~266.7us) | ~40.0km |

Figure 1: Structure of NPRACH format 0

Figure 2: Structure of NPRACH format 0

Observation 2:  The Tx duration of NPRACH format 0 is 2ms per each repetition. The Tx duration of NPRACH format 1 is 4ms per each repetition.

To align with TDD with D/U=8, we think the number of repetitions for NPRACH format0 can be considered as 4, and number of repetitions for NPRACH format 1 can be considered as 2, according to the number of repetitions supported for NPRAH TDD operation.

Proposal 6: RAN4 could consider the number of repetitions for NPRACH format 0 as 4, and number of repetitions for NPRACH format 1 as 2 when defining NPRACH requirement for TDD operation

Antenna configuration

Similar as Rel-18 IoT NTN requirement, we think both 1Rx and 2Rx configuration can be considered for requirement definition based on the manufacture declaration. To reduce the test effort, the test applicability rule for different antenna configuration can be reused.

Proposal 7: RAN4 could specify the requirement with both 1 Rx and 2Rx, and reuse the test applicability rule for different antenna configuration to test

Applicability rule

Similar as TN specification, we think the requirements for TDD are optional and only valid for SAN supporting this new TDD patten.

Proposal 8: The requirements for TDD are optional and only valid for SAN supporting TDD pattern.

# 4	Conclusion

In this contribution, the overview on test scope and test setup for SAN and UE demodulation requirement to support TDD operation was provided

Proposal 1:  RAN4 could consider to define SAN and UE demodulation requirement to support NB-IoT TDD operation. FFS on the related test parameters to align with the TDD pattern (D/U=8ms)

- UE demodulation requirement:  NPDSCH

- SAN demodulation requirement:  NPUSCH 1 and NPUSCH 2 with 15KHz, NPRACH with format 0 and format 1

NPDSCH

Proposal 2: RAN4 could take the existing test configuration defining in current TN spec as starting point to define the UE demodulation requirement to support NTN TDD operation.

NPUSCH1 and NPUSCH 2

Proposal 3: RAN4 could take the following test cases to define the NPUSCH format1 and format 2 requirement for TDD operation

- NPUSCH format 1

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex X) | FRC (Annex A) | Frequency offset | Tx-Duration | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 1 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLA100-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |
| 1 | 2 | 15KHz | 12 | NTN TDLC5-1 | A7-2 | 4 Hz | 8 | 8 | 70% | TBD |

- NPUSCH format 2

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Frequency offset | TxDuration | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN TDLA100-1 | 1 | 15KHz | 4Hz | 8 | 4 | TBD |
| 1 | 2 | NTN TDLA100-1 | 1 | 15KHz | 4 Hz | 8 | 4 | TBD |

NPRACH

TDD configuration

Proposal 4: RAN4 can apply the TDD pattern with N, D/U=8 for related TDD configuration for NPRACH

Channel model

Proposal 5: RAN4 could apply the channel model with NTN-TDLA100-1 with 200Hz frequency offset for specifying NPRACH requirement with supporting TDD operation

Number of Repetition

Observation 2:  The Tx duration of NPRACH format 0 is 2ms per each repetition. The Tx duration of NPRACH format 1 is 4ms per each repetition.

Proposal 6: RAN4 could consider the number of repetitions for NPRACH format 0 as 4, and number of repetitions for NPRACH format 1 as 2 when defining NPRACH requirement for TDD operation

Antenna configuration

Proposal 7: RAN4 could specify the requirement with both 1 Rx and 2Rx, and reuse the test applicability rule for different antenna configuration to test

Applicability rule

Proposal 8: The requirements for TDD are optional and only valid for SAN supporting TDD pattern.

# References

RP-243293 Revised WID on introduction of IoT-NTN TDD