3GPP TSG RAN WG4 Meeting#116bis                                 R4-2513417

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.9.3

Source:	vivo

Title:	Discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

# Introduction

In R18 SI and R19 SI [1][2], RAN4 has studied interoperability and testability aspects of two-sided AI/ML models. A follow-up Rel-20 WI [3] was agreed in the RAN#108 meeting. According to the WID, RAN4 is the leading group to work on test/reference models to guarantee inter-vendor training collaboration.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we provide our views on inter-vendor training collaboration and interoperability for two-sided AI/ML models.

# Discussion

In last meeting, the following aspects are suggested to be discussed in R20 WI. The discussion will focus on the following aspects.

The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on the dataset assumptions and dataset - Discuss and align on assumptions for the test decoder and reference encoder structures - Also may need to consider how many test decoders and encoders - Discuss and align on the scalability alternatives - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model - Develop a means to capture model structures and model parameters in 3GPP - Discuss and agree the test metric for the performance requirements. - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

## General consideration on inter-vendor training collaboration and interoperability

In this section, general consideration on inter-vendor training collaboration and interoperability is provided.

Model specification for R20 WID

In Direction C in inter-vendor training collaboration of RAN1, the reference model is fully defined/specified. Note that, reference model may not be the actual implemented model. It may just be used for NW/UE to develop real decoder/encoder. For Interoperability and RRM requirement of RAN4, model specification at least includes the fully specified test decoder.

Observation 1: Model specification for R20 WID includes:

- Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C.

- Fully specified test decoder.

RAN4 job for check-point

For inter-vendor training collaboration Direction C and Direction A sub-option 3a-1, there is check-point in RAN#110 on fully defined/specified reference model. There are only 2 meetings left before the check-point. RAN4 does not need to rush to agree detailed model structure and scalability solutions. The main goal would be that whether RAN4 can work on R19 RAN1/4 agreements and fulfill the requirements from WID on reference model/structure. From these perspectives, RAN4 is suggested to agree on steps to align the model structure and scalability, based on R19 RAN1/4 agreements, to show that RAN4 can fulfill the requirements from WID on reference model/structure.

Proposal 1: RAN4 job for check-point of inter-vendor training collaboration Direction C and Direction A, sub-option 3a-1:

- Agree on steps to align the model structure and scalability, based on R19 RAN1/4 agreements, to show that RAN4 can fulfill the requirements from WID on reference model/structure.

Relation between RAN4 test/reference model and RAN1 inter-vendor collaboration direction C

In current RAN1 scope, there is no further discussion on the design of reference model for RAN1 inter-vendor collaboration direction C, which means there would be no more requirements from RAN1 in R20 WI for the test/reference model. RAN1 will directly use the test/reference model of RAN4, for the inter-vendor collaboration direction C. To collaborate with RAN1, other than the consideration on the RAN4 test, RAN4 would only need to consider the R19 RAN1/RAN4 agreements.

Proposal 2: RAN4 test/reference model is used by RAN1 for inter-vendor collaboration direction C.

- There will be no more further requirements from RAN1 in R20 WI.

- RAN4 test/reference model should be designed based on R19 RAN1/RAN4 agreements.

## Consideration on model structure and scalability

In this section, model structure and scalability will be discussed.

Backbones of reference model

As seen in Appendix C, RAN1 agreed Transformer as the backbone for reference model during the study. All RAN1 scalability results, observation and conclusions are based on Transformer. The observations agreed in RAN1 shows that the scalability issue can be solved by the listed solutions.

In current RAN4 study on test decoder, CNN, Transformer and MLP are evaluated. As seen in below table, for either encoder or decoder, Transformer has better performance compared to CNN and MLP, with similar complexity.

Table 2.2-1. SGCS results for different encoder structure with 1 to 1 joint training.

| Different encoder backbones | SGCS | Flops |
| --- | --- | --- |
| Encoder-1 (CNN) | 0.696 | 47M |
| Encoder-2 (Transformer) | 0.709 | 3.63M |
| Encoder-3 (CNN) | 0.689 | 4.14M |
| Encoder-4 (MLP) | 0.671 | 0.22M |

Table 2.2-2. SGCS results for different decoder structure with 1 to 1 joint training.

| Different decoder backbones | SGCS | Flops |
| --- | --- | --- |
| Aligned CNN decoder | 0.709 | 740M |
| Proposed transformer decoder: dmodel = 192, Nhead = 8, dhead = 24, NTF = 4 | 0.729 | 31.3M |

Thus, it is suggested that at least Transformer is considered as the backbone for reference encoder for 3a-1 and direction C. More backbones can be considered.

Proposal 3: At least Transformer is considered as the backbone for reference encoder for 3a-1 and direction C. More backbones can be considered.

- RAN1 agreed Transformer as the backbone for reference model during the study.

- All RAN1 scalability results, observation and conclusions are based on Transformer.

- CNN, Transformer and MLP are evaluated in RAN4.

- Transformer has better performance compared to CNN and MLP, with similar complexity.

Model structure scalability

As seen in Appendix C, the following alternatives for scalability were agreed in RAN1 during the study, where the number of sources and performance loss of common structure compared to dedicated structure are added in each alternative. RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- For the choice of token dimension and feature dimension,

  - Alt 1: Use subband as the token dimension and Tx port as a feature dimension (8 sources, median -1.10%). For transformer, there are some input dimensions, where one is token dimension and another is feature dimension. Transformer has one special characteristic that for different token size, the main transformer block can be natural scalable, and only the embedding layer and output linear layer need to be designed. Since the subband size would be very flexible in field and the Tx port number is fixed, most companies support to use subband as the token dimension and Tx port as a feature dimension.

  - Alt 2: Use Tx port as the token dimension and subband as a feature dimension. As discussed in Alt 1, it is not suitable to use the flexible subband as the feature dimension, while use the fixed Tx port as token dimension.

  - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. (1 source, median -0.3%). This alternative is kind of combination of Alt 1 and Alt 2. The feature dimension is fixed n_Tx_ports*m_Subbands. It could achieve good performance, but is not so straightforward as Alt 1.

- For scalability over the feature dimension,

  - Alt1: specific embedding layer for each feature size (4 sources, median -2.08%). In this alternative, different feature size would go through different embedding layer, and then processed to the same transformer block. There would be the burden of the storage of different embedding layer.

  - Alt2: a common embedding layer with padding (7 sources, median -0.85%). In this alternative, different feature size would be padding to the same size, and then go through the same embedding layer and the same transformer block. Most companies support this alternative, due to its simple design and good performance.

- For scalability over the token dimension,

  - Alt1: positional embedding specific to each token index (2 sources, median -1.5%). In this alternative, different subband would have different positional embedding. Some methods could be used for this alternative. Some extra complexity is introduced and some performance loss is achieved.

  - Alt2: Padding at the input (3 sources, median -0.75%). In this alternative, different token size would be padding to the same size, and then the model inference would be the same. Most companies support this alternative, due to its simple design and good performance.

- For scalability over payload configurations,

  - Alt1: specific output linear layer for each payload configuration (4 sources, median -2.08%). In this alternative, different payload configuration would go through different output linear layer. There would be the burden of the storage of different output linear layer.

  - Alt2: truncation/masking of the output linear layer output (3 sources, median -0.85%). In this alternative, the output linear layer would provide the maximum size of output. And then, due to different payload configuration, part of the output linear layer output will be selected as CSI feedback, while other part of the output linear layer output will be truncated/masked. This alternative is simple and achieve good performance.

  - Alt3: by varying quantization parameters. In this alternative, for different payload configuration, the output linear layer output is the same, while the quantization parameters would be different.

Below is the illustration for some potential scalability solutions.

Figure 2.2-1. Illustration for some potential scalability solutions.

The observations agreed in RAN1 shows that the scalability issue can be solved by the listed solutions. RAN4 does not need repeat RAN1’s work again in R20. Thus, for scalability down-selection, RAN4 to directly consider RAN1 scalability observations.

Below is one suggested scalability solution, which is Alt 1 for choice of token and feature, Alt 2 for feature dimension, Alt 2 for token dimension, and Alt 2 for payload configuration. This scalability solution is straight forward and future-proof. Use subband as the token dimension and Tx port as a feature dimension. A common embedding layer with padding is used for scalability over the feature dimension. Padding before quantization is used for scalability over the token dimension. Truncation/masking of the output linear layer output is used for scalability over the payload configurations. Below table is the simulation results for 16/32 Tx, 9/13 subbands and PC1/3/8, which shows that this proposed scalability solution can provide good performance for different configurations of antenna ports, subbands and payload configurations.

Figure 2.2-2. Illustration for the proposed scalability solution, which is Alt 1 for choice of token and feature, Alt 2 for feature dimension, Alt 2 for token dimension, and Alt 2 for payload configuration.

Table 2.2-3. Simulation results for proposed scalability solution, for scalability over 16/32 Tx, 9/13 subbands and PC1/3/8.

| Dedicated structure with dedicated parameters | Common structure with dedicated parameters | Common structure with common parameters |
| --- | --- | --- |
| Baseline | Up to 0.1% SGCS loss | Average 0.8~1.0% SGCS loss |

Thus, we have the following proposal.

Proposal 4: For model structure scalability,

- RAN1 confirmed the scalability of model structure and parameters for direction A 3a-1 and direction C.

- RAN4 to consider RAN1 scalability observations, and these simulations would not need to be repeated in RAN4.

  - For the choice of token dimension and feature dimension,

    - Alt 1: Use subband as the token dimension and Tx port as a feature dimension (8 sources, median -1.10%).

    - Alt 2: Use Tx port as the token dimension and subband as a feature dimension.

    - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. (1 source, median -0.3%).

  - For scalability over the feature dimension,

    - Alt1: specific embedding layer for each feature size (4 sources, median -2.08%).

    - Alt2: a common embedding layer with padding (7 sources, median -0.85%).

  - For scalability over the token dimension,

    - Alt1: positional embedding specific to each token index (2 sources, median -1.5%).

    - Alt2: Padding at the input (3 sources, median -0.75%).

  - For scalability over payload configurations,

    - Alt1: specific output linear layer for each payload configuration (4 sources, median -2.08%).

    - Alt2: truncation/masking of the output linear layer output (3 sources, median -0.85%).

    - Alt3: by varying quantization parameters.

- RAN4 to pick the most straight forward and future-proof one among the alternatives, e.g., padding for Tx port and subband, truncation/masking for payload configuration.

Steps to align the model structure and scalability

During Rel-19 study, RAN4 concludes the feasibility of all testing model options. Similar methodology of RAN4 R19 study for test model options, can be carried on to Rel-20 WI to align on the reference model structure and scalability.

- Step 0: Evaluation assumptions and mixed dataset based on R19 can be reused. In R19, RAN4 reused the RAN1 simulation assumptions. For the requirement definition, extra dataset may be needed for scalability solutions.

- Step 1: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 1-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

  - Step 1-2: Check on performance alignment -> see simulation results from contributing companies.

  - Step 1-3: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed. If companies can achieve good performance alignment in above step, this step may be not needed.

  - Step 1-4: Select one or more model structures, which includes both model backbone and model hyperparameters.

  - Step 1-5: Select one or more scalability solutions. This step can be done in parallel with Step 1-4 (selection of model structures).

- Step 2: Specify the model for the following work in WID. Where and how to specify may need further discussion.

  - Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C, and

  - Fully specified test decoder.

Thus, we have the following proposal.

Proposal 5: Similar methodology of RAN4 R19 study for test model options, can be carried on to Rel-20 WI to align on the reference model structure and scalability.

- Step 0: Evaluation assumptions and mixed dataset based on Rel-19 can be reused.

- Step 1: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 1-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

  - Step 1-2: Check on performance alignment -> see simulation results from contributing companies.

  - Step 1-3: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed.

  - Step 1-4: Select one or more model structures.

  - Step 1-5: Select one or more scalability solutions. This step can be done in parallel with Step 1-4.

- Step 2: Specify the model for the following work in WID. Where and how to specify may need further discussion.

  - Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C, and

  - Fully specified test decoder.

How to capture model structures and model parameters in 3GPP

There would be several methods to capture model structures and model parameters in 3GPP.

- One public format for model description that has existed in the AI community, such as ONNX. In R19 SI, ONNX is successfully used for companies to share the models. ONNX can be compatible with any AI framework including TensorFlow, PyTorch and so on.

- New format for model description defined by 3GPP. The model structure can be easily drawn, which has been done in R19 SI. The main challenge would be the description of model parameters. For popular model backbone, the model parameters would be two/three/four-dimension matrix, which would be easy to describe in paper or txt file. For MLP, the model parameters would be two-dimension matrix for the multiplicative coefficient of one layer, and one-dimension vector for the additive coefficient of one layer. For CNN, the model parameters would be four-dimension matrix for the multiplicative coefficient of one layer, and three-dimension matrix for the additive coefficient of one layer. For Transformer and MLP-mixer, the model parameters that need to be transferred are similar to MLP.

Thus, we have the following proposal.

Proposal 6: RAN4 consider the following methods to capture model structures and model parameters in 3GPP.

- One public format for model description that has existed in the AI community, such as ONNX.

- New format for model description defined by 3GPP. The model structure can be easily captured as R19 SI. For popular model backbone (MLP, CNN, Transformer, MLP-mixer), the model parameters would be two/three/four-dimension matrix, which would be easy to describe in paper or txt file.

  - For MLP, the model parameters would be two-dimension matrix for the multiplicative coefficient of one layer, and one-dimension vector for the additive coefficient of one layer.

  - For CNN, the model parameters would be four-dimension matrix for the multiplicative coefficient of one layer, and three-dimension matrix for the additive coefficient of one layer.

  - For Transformer and MLP-mixer, the model parameters that need to be transferred are similar to MLP.

## Consideration on dataset assumption and dataset

In this section, dataset assumption and dataset will be discussed.

Channel model for training and testing

In RAN4 test, the channel model is LLS, which is TDL or CDL. However, SLS is used in RAN1 and RAN4 evaluations in R19 for reference/test model. Also, as shown in field results in our previous contribution [4], UMa trained model can achieve good performance in field, while CDL trained model works bad in field.

Below table presents the baseline link level simulation assumptions of CDL channel for reference model derivation, where some unsuitable information is removed.

Table 2.3-1. Baseline link level simulation assumptions of CDL channel for reference model derivation.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD OFDM |
| Carrier frequency | 2GHz |
| Bandwidth | 20MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C |
| UE speed | 3kmhr |
| Delay spread | 30ns |
| Channel estimation | Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |
| Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. |  |

Based on our proposed LLS assumptions for CDL-C channel model and CNN model structures, below table shows the SGCS results for different model structures under the different tests and training datasets. Note that the UMa dataset is generated based on the SLS assumption from RAN1 and related parameters are same as our proposed LLS assumptions for CDL-C. The mixed dataset contains the partial CDL-C dataset and UMa dataset, which are mixed together in one dataset randomly for model training.

Table 2.3-2. SGCS under different channel model for the proposed CNN and Transformer.

| Model structure |  | CNN |  | Transformer |  |
| --- | --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | Test dataset |  |
|  |  | CDL-C | UMa | CDL-C | UMa |
| 116 bits | CDL-C | 0.994 | 0.336 | 0.996 | 0.253 |
|  | UMa | 0.791 | 0.787 | 0.831 | 0.821 |
|  | Mixed | 0.982 | 0.773 | 0.988 | 0.801 |
| CNN encoder: FLOPS 126M, size 0.1M. CNN decoder: FLOPS 1974M, size 2.4M. Transformer encoder: FLOPS139M, size 5.6M. Transformer decoder: FLOPS 139M, size 5.6M Note: Mixed training dataset includes CDL-C and UMa channel model. |  |  |  |  |  |

It can be observed that, the models trained under CDL-C dataset could provide a very high SGCS results under the CDL-C test datasets, while the performance degrades severely under the UMa test datasets. However, the models trained under UMa could provide a medium good performance of SGCS for UMa test dataset, and provide a higher SGCS under the CDL-C test dataset compared with the model training under CDL-C but tested in UMa. For the model trained under mixed dataset, it could provide good SGCS results under CDL-C and UMa test datasets and only degrade a little compared with the results of <CDL-C trained, CDL-C tested> and <UMa trained, UMa tested>. Thus, the best option is to use the mixed dataset for training and one of types of the dataset for testing. Since the TDL channel is a simpler channel compare with CDL and UMa, it can also be used for mixing and RAN4 test case. Similar to the previous test case, RAN4 tests could only use TDL channel. Based on the following analysis, we could have the following proposal.

Proposal 7: Using the mixed dataset for reference model training, including the mixing of SLS and LLS, while using the LLS dataset for RAN4 tests. Other mixing rules are not precluded.

How to capture reference dataset

For the dataset, the channel information is necessary, which is encoder input and the label of decoder output. The latent information, which is encoder output and decoder input, could be obtained by putting channel information into reference encoder. The method has been successfully used in R19 SI.

Proposal 8: RAN4 to capture at least the channel information. Latent information could be obtained by putting channel information into reference encoder.

Considering eigenvectors and raw channel, eigenvectors would be much smaller than raw channel, while raw channel would provide more information than eigenvector. Thus, it is suggested to capture at least the eigenvectors for reference dataset.

Proposal 9: RAN4 to capture at least the eigenvectors for reference dataset, for the channel information.

# Summary

Based on above analysis, following proposals and observations are present.

Observation 1: Model specification for R20 WID includes:

- Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C.

- Fully specified test decoder.

Proposal 1: RAN4 job for check-point of inter-vendor training collaboration Direction C and Direction A, sub-option 3a-1:

- Agree on steps to align the model structure and scalability, based on R19 RAN1/4 agreements, to show that RAN4 can fulfill the requirements from WID on reference model/structure.

Proposal 2: RAN4 test/reference model is used by RAN1 for inter-vendor collaboration direction C.

- There will be no more further requirements from RAN1 in R20 WI.

- RAN4 test/reference model should be designed based on R19 RAN1/RAN4 agreements.

Proposal 3: At least Transformer is considered as the backbone for reference encoder for 3a-1 and direction C. More backbones can be considered.

- RAN1 agreed Transformer as the backbone for reference model during the study.

- All RAN1 scalability results, observation and conclusions are based on Transformer.

- CNN, Transformer and MLP are evaluated in RAN4.

- Transformer has better performance compared to CNN and MLP, with similar complexity.

Proposal 4: For model structure scalability,

- RAN1 confirms the scalability of model structure and parameters for direction A 3a-1 and direction C.

- RAN4 to consider RAN1 scalability observations, and these simulations would not need to be repeated in RAN4.

  - For the choice of token dimension and feature dimension,

    - Alt 1: Use subband as the token dimension and Tx port as a feature dimension (8 sources, median -1.10%).

    - Alt 2: Use Tx port as the token dimension and subband as a feature dimension.

    - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. (1 source, median -0.3%).

  - For scalability over the feature dimension,

    - Alt1: specific embedding layer for each feature size (4 sources, median -2.08%).

    - Alt2: a common embedding layer with padding (7 sources, median -0.85%).

  - For scalability over the token dimension,

    - Alt1: positional embedding specific to each token index (2 sources, median -1.5%).

    - Alt2: Padding at the input (3 sources, median -0.75%).

  - For scalability over payload configurations,

    - Alt1: specific output linear layer for each payload configuration (4 sources, median -2.08%).

    - Alt2: truncation/masking of the output linear layer output (3 sources, median -0.85%).

    - Alt3: by varying quantization parameters.

- RAN4 to pick the most straight forward and future-proof one among the alternatives, e.g., padding for Tx port and subband, truncation/masking for payload configuration.

Proposal 5: Similar methodology of RAN4 R19 study for test model options, can be carried on to Rel-20 WI to align on the reference model structure and scalability.

- Step 0: Evaluation assumptions and mixed dataset based on Rel-19 can be reused.

- Step 1: Align on the model backbone, model hyperparameters and the scalability solution.

  - Step 1-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution.

  - Step 1-2: Check on performance alignment -> see simulation results from contributing companies.

  - Step 1-3: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed.

  - Step 1-4: Select one or more model structures.

  - Step 1-5: Select one or more scalability solutions. This step can be done in parallel with Step 1-4.

- Step 2: Specify the model for the following work in WID. Where and how to specify may need further discussion.

  - Fully defined/specified reference encoder for testing decoder, for both 3a-1 and direction C, and

  - Fully specified test decoder.

Proposal 6: RAN4 consider the following methods to capture model structures and model parameters in 3GPP.

- One public format for model description that has existed in the AI community, such as ONNX.

- New format for model description defined by 3GPP. The model structure can be easily captured as R19 SI. For popular model backbone (MLP, CNN, Transformer, MLP-mixer), the model parameters would be two/three/four-dimension matrix, which would be easy to describe in paper or txt file.

  - For MLP, the model parameters would be two-dimension matrix for the multiplicative coefficient of one layer, and one-dimension vector for the additive coefficient of one layer.

  - For CNN, the model parameters would be four-dimension matrix for the multiplicative coefficient of one layer, and three-dimension matrix for the additive coefficient of one layer.

  - For Transformer and MLP-mixer, the model parameters that need to be transferred are similar to MLP.

Proposal 7: Using the mixed dataset for reference model training, including the mixing of SLS and LLS, while using the LLS dataset for RAN4 tests. Other mixing rules are not precluded.

Proposal 8: RAN4 to capture at least the channel information. Latent information could be obtained by putting channel information into reference encoder.

Proposal 9: RAN4 to capture at least the eigenvectors for reference dataset, for the channel information.

# References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, RAN#108

- R4-2510342, vivo, “Discussion on testing issues for CSI compression two-sided models”, RAN4#116, Aug 25 – 29, 2025.

Appendix A: Agreements of standardization of model structure in RAN1

RAN1#120 agreed the full-pictured model structure for studying the standardized model structure as following

Agreement For studying the standardized model structure, for temporal domain Case 0, in case of spatial-frequency domain input, adopt the following model structure as one example structure for study purpose, Encoder description: Embedding layer (Note: linear embedding in above figure to be revised to Embedding layer); Number of tokens Feature dimension of each token Output dimension dmodel per token. Positional encoding; Number of tokens positions Dimension of positional encoding for each token position: dmodel. Transformer blocks; Number of transformer blocks NTF Dimension of transformer block dmodel Number of self-attention heads Nhead Dimension of attention head dhead Dimension of latent space inside feedforward module dFF Activation function choice. Output linear layer; Reshape the matrix to a vector Output dimension zdim. Quantization; Scalar quantization: - Number of bits per latent dimension: Nbit - Total payload size for Scalar quantization = Nbit * Zdim Vector quantization - segment size = Nseg - Number of bits per latent dimension: Nbit - bits per segment = Nbit * Nseg - Number of segments = Zdim / Nseg - Total payload size for Vector quantization = Number of segments * bits per segment.

And RAN1 #120 agreed some options on the scalable model structure specification.

A Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

RAN1 #120-bis agreed some observations on the scalable model structure solutions.

Observation For choice of token/feature dimension in the scalable model design, companies observe that there is small loss in SGCS for Case1 over Case2 under tokenization (Alt1, Alt3) For layer1, under tokenization (Alt1), - 8 sources [Qualcomm, CATT, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe that average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.33% with median value of -1.10% - 2 sources [CATT, Xiaomi] observe average gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 2.04% to 7.84% with median value of 6.61% - 3 sources [CATT, Xiaomi, Ericsson] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under tokenization (Alt3), - 1 source [vivo] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.7% to 0.4% with median value of -0.3% - 1 source [Ericsson] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.4% to 6% For scalability over the feature dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2, Alt1+Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Fujitsu ] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.34% to -0.4% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 7 sources [Qualcomm, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.1% with median value of -0.85% - 1 source [Xiaomi] observes gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For layer1, under (Alt1+Alt2), - 1 source [Qualcomm] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.3% to -2.9% with median value of -3.1%. For scalability over the token dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 2 sources [Qualcomm, vivo] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.9% with median value of -1.5%. For layer1, under (Alt2), - 3 sources [CATT, vivo, ZTE ] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.4% with median value of -0.75%. - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51% to 7.84% with median value of 6.87% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12% to 10.2% with median value of 10.1% For scalability over the payload configurations, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Huawei] observe average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.73% to -3.34% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 3 sources [Xiaomi, vivo, ZTE] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.4% with median value of -0.85% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For scalability over feature dimension, token dimension and payload configuration jointly, For tokenization Alt1, feature scalability Alt1, token scalability Alt2 and payload scalability Alt1, - 1 source [CATT] observes average gain in SGCS of Case 1 over case 2 is in the range of -3.34%~2.03% with median value of -2.93% - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51%~7.84% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12%~10.23% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, payload configuration Alt2, - 2 sources [ZTE, vivo] observe average gain in SGCS of Case 1 over case 2 is in the range of -2.8%~1.2% with median value of -1.2%, under same parameter set. - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.1% ~ 0.8% with median value of 0.1%, under different parameter set For tokenization Alt1, feature scalability Alt2, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -1.7% ~ 5.4% with median value of -2.6%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.6% ~ 0% with median value of -0.9%, under different parameter set For tokenization Alt1, feature scalability Alt1, payload configuration Alt1, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -4.76%~-0.82% with median value of -2.08%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 3% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, payload configuration Alt2, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -5.52%~-1.59% with median value of -2.96%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, - 1 source [ZTE] observes average gain in SGCS of Case 1 over case 2 is in the range of -0.4%~0.9% with median value of -0.23%. For tokenization Alt3, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.1% ~ 1.6% with median value of -0.5%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.8% ~ 0% with median value of -0.4%, under different parameter set