# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513448

# Prague, CZ, 13-17 Oct, 2025

Agenda Item:	7.8.2

Source: 	Xiaomi

Title: 	UE RF requirements on LB CA with switching

Document for:	Discussion

- Introduction

In Rel-19, a new WI was agreed to enable LB-LB CA with swicthing in [1]. The motivication of this WI was to efficiently use the SDL spectrum together with low FDD band via CA by swicthing to eliminate UE implemenation channllenge  of supporting co-current transmission/reception between these two carriers due to antenna performance with wide frequnecy separation (fractional bandwith), and or duplexer design w/o sufficient frequency gap between DL and UL.

In Rel-19, a continued enhanced WI was approved in RAN#109 which target to enable LB CA switching between 2 bands with dual DL.

Detailed WID is reproduced here for reference:

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this contribution, we analyze the impact UE RF requirements for LB-LB CA via switching with Dual DL.

- Discussion

According to the WID description, on the target LB CA switching with following cases:

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

  - Example band combinations: CA_n12A-n29A and CA_n28A-n67A

  - No in gap interference between 2 carriers is expected

Switching period and UE capabilities

The required transient period from UE transceiver chain switching between carrier 1 and carrier is depending on UE architecture which can be categorized as following:

- Partially shared receiver chain: UE share RF front-end including antenna and LNA with separate LO/Mixer, LBF between two carriers

  - UE needs to switch duplex/RF filter in front-end between transient period with necessary RF retuning as well

- Fully shared receiver chain: UE share whole receiver chain from antenna to ADC

  - UE needs to switch duplex/RF filter in front-end between transient period with necessary RF retuning for mixer/LO and other analogy components as well

Compared to LB CA via switching with single carrier between case 1 and case 2, introduced in Rel-19, additional RF front-end duplex/RF filter is required to support Dual DL under case 3.

Observation 1: From UE RF architecture perspective, similar architecture as CA via switching between {case 1 and case 2} with additional RF front-end filter/duplex is required for Dual DL case.

Similar as Rel-19 LB CA via switching between {case 1, case 2}, new UE capability with multiple values can be introduced to accommodate different UE implementation and the corresponding UE capability shall be per band combination pair basis.  As mentioned above, additional RF component/effort is required from UE perspective to support LB CA via switching with dual DL compared to Rel-19 LB CA via switching with single carrier case. 
A new separate UE capability shall be introduced for CA switching between {case 1 and case 3},.

Proposal 1: Introduce new separate UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementation.

- Switching period capability values {35us, 70us, 140us}

- The capability shall be per band pair per band combination basis

Time mask requirements

Regarding time mask requirements, similar as Rel-19 case, UL Tx on/off time mask requirements shall be applied instead of UL carrier switching requirements.

Regarding switching scheme and location of switching gap, according to the WID, we shall reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching to minimize RAN1 impact if any.

According to RAN1 agreements in Rel-19, switching gap always located in switching from carrier.

Observation 2: Existing switching gap can be extended to cover Rel-20 LB CA via switching case

- For Dual DL CA switching to FDD carrier, the switching period/switching gap located out of FDD carrier slot.

  - UE expects that the switching gap configured by the NW for Dual DL carriers to FDD carrier switching covers at least the switching period and UL TA

- For FDD carrier switching to Dual DL carriers, the switching period/switching gap located within FDD carrier slot

Proposal 2: Tx on/off time mask requirements which is introduced in Rel-19 for LB CA switching can be applied to LB CA via switching with dual DL.

- Some editorial update expected on the description below  on/off mask figure

- Conclusion

In this contribution, we analyze the impact UE RF requirements for LB-LB CA via switching with Dual DL.

Observation 1: From UE RF architecture perspective, similar architecture as CA via switching between {case 1 and case 2} with additional RF front-end filter/duplex is required for Dual DL case.

Proposal 1: Introduce new separate UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementation.

- Switching period capability values {35us, 70us, 140us}

- The capability shall be per band pair per band combination basis

Observation 2: Existing switching gap can be extended to cover Rel-20 LB CA via switching case

- For Dual DL CA switching to FDD carrier, the switching period/switching gap located out of FDD carrier slot.

  - UE expects that the switching gap configured by the NW for Dual DL carriers to FDD carrier switching covers at least the switching period and UL TA

- For FDD carrier switching to Dual DL carriers, the switching period/switching gap located within FDD carrier slot

Proposal 2: Tx on/off time mask requirements which is introduced in Rel-19 for LB CA switching can be applied to LB CA via switching with dual DL.

- Some editorial update expected on the description below  on/off mask figure

Reference

[1] RP-243317, New WID on low band carrier aggregation via switching, AT&T TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE

[2] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei)