3GPP TSG-RAN WG4 Meeting #116bis	R4-2513470

Prague, Czech Republic, 13-17 October 2025

Agenda item:	6.10.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance for Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

The enhancement of fast SCell activation for UE supporting EMR was completed in previous meetings. In the last RAN4 meeting, RAN4 initially discussed the performance part for this feature with agreements and open issues captured in [1]. In this paper, we further provide our views on the remaining issues.

- Discussion

In the last RAN4 meeting, the performance part for fast SCell activation for UE supporting Rel-18 EMR was discussed with the status as follows:

Issue 2-2-1: Test case list - Recommended WF - Based on the majority view, define test cases for - Direct SCell activation at SCell addition for FR2 - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 has not expired at the moment of initiation of RRC state transition to CONNECTED mode Agreement in RAN4#115: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Regarding the WF from the last meeting, the test case of Direct SCell activation at SCell addition for FR2 is able to be defined. From our understanding, for the core part discussion, it is identified that direct SCell activation is a promising scenario for this feature, where gNB can activate the SCell upon UE entering connected mode within the same RRC message. Thus, we think RAN4 shall introduce TC for direct SCell activation at SCell addition for FR2.

Proposal 1: RAN4 to define TC for direct SCell activation at SCell addition for FR2.

For the applicable configurations, three cases are considered:

- Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

From our understanding, UE may only support one of these cases. And, RAN4 shall define test cases for all configurations. But to limit the number of TCs that UE is required to pass, applicability rules can be discussed when UE supports multiple configurations.

Proposal 2: RAN4 to define test case(s) for all the following configurations, and RAN4 to discuss the applicability rules when UE supports multiple configurations.

- Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

One may have concerns about too many TC cases, considering normal SCell activation/PUCCH SCell activation/direct SCell activation. The applicability rules can be considered as the previous Release. Particularly, for PUCCH SCell activation, it also verifies the UL procedures compared with normal SCell activations. Thus, it is proposed to define similar applicability rules as:

- For UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

Proposal 3: UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

The following is the design of the test case that was captured in the WF. To kick off the discussion on test design the Configuration 1 is able to be the starting point. The details are shown as follows:

Issue 2-2-3: Test case design - Recommended WF - Discuss and polish the test procedure for normal SCell activation with Configuration 1 as a starting point: - Discuss whether the X1 and X2 in the test procedure are needed. - Discuss DRX configuration: - Alt 1: DRX is not configured in all test cases. - Alt 2: DRX is configured in some test cases. - Discuss whether to define a unified test case for all SCell activation types (Option 7). Note: After the test procedure is concluded, it can be used as a template when drafting other test cases, while the corresponding legacy test cases can also be referred to.

In terms of this feature, the EMR report determines the SCell activation. Therefore, the key focus of this test design should emphasize the UE behavior after the Rel-18 EMR result becomes valid. The general functional test for SCell activation is unnecessary; for example, there is no need to include DRX mode in the test configuration.

Proposal 4: The non-DRX mode is able to be considered in the test case for normal SCell activation with Configuration 1.

- Conclusions

Proposal 1: RAN4 to define TC for direct SCell activation at SCell addition for FR2.

Proposal 2: RAN4 to define test case(s) for all the following configurations, and RAN4 to discuss the applicability rules when UE supports multiple configurations.

- Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured

- Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Proposal 3: UE supporting two PUCCH groups, UE is required to pass PUCCH SCell activation TC and skip normal SCell activation TC.

Proposal 4: The non-DRX mode is able to be considered in the test case for normal SCell activation with Configuration 1.

4. References

[1] R4-2512125 WF on RRM requirements for NR_RRM_Ph5_Part2