3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513536

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	General views on NR_LBCA_Sw_enh

Source: 	Huawei, HiSilicon

Agenda item:	7.8.2

Document for:	Discussion

# Introduction

TSG RAN#109 approved a new WI of “Enhancement of low NR band carrier aggregation via switching”.  “NR_LBCA_Sw_enh” [1]. This paper provides general views to be associated with our future work.

# Discussion

## Principle for the future work

An excerpt of the objective of the WI from [1] is as follows.

Figure 1: An excerpt of the objective of the WI [1]

If we take a look at the texts in yellow, we see several “reuse” in the objective. In our understanding, these “reuse” come from the fact that the amount of available RAN4 TU is very limited and RAN1/2 chairmen declared that there is no available TU in TSG RAN#109.

Observation 1: The WI was approved under the situation that RAN4 TU is limited and no RAN1/2 TU is available.

With the above Observation 1 in mind, RAN4 should stick around reusing Rel-19 LB-CA requirements associated with not only RAN4, but also RAN1/2 to minimize the amount of workload. Any unnecessary enhancements such as shorter switching period, etc., other than the objective in Rel-20 LB-CA should be avoided.

Observation 2: Any unnecessary enhancements such as shorter switching period, etc., other than the objective in Rel-20 LB-CA produce unnecessary workload.

Proposal 1: RAN4 should focus on competing only the objective in the WID and avoid discussing unnecessary enhancements such as shorter switching period, etc. unless the necessity is agreed.

Regarding texts in green, our interpretation is that R20 Rel-19 LB-CA will introduce band combinations with only two bands as Rel-19 LB-CA didn’t introduce higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA). Based on the principle of Proposal 1, RAN4 should consider t is out of scope of R20 WI.

Proposal 2: RAN4 shouldn’t consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19. It is noted that for CA_n28-n67, only the difference from CA_n12-n29 is captured.

## Possible co-existence issues

This sub-section discusses possible co-existence issues per scenario. The below Figure 2 is an excerpt of spectrum arrangement for two example band combinations from TR 38.768.

Figure 2: An excerpt of frequency arrangements for example BCs from TR38.768

Observation 4: Following observations are obtained on co-existence issues per scenario.

| Scenario | Type of interference | CA_n12-n29 | CA_n28-n67 |
| --- | --- | --- | --- |
| UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation) | Co-channel | UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, the situation can be mitigated thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)) |
|  | Blocking | Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out |
| BS to UE / UE to BS | Given that co-location is assumed, all those wouldn’t have an issue. |  |  |
| BS to BS | Co-channel interference and blocking | Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out. |

Observation 5: Some issues captured in Observation 4 are not R20 specific, but rather also apply to R19.

Proposal 3: For UE to UE co-existence (FDD UL Transmission to SDL reception), RAN4 should discuss whether or not special care is needed or not.

    - For CA_n12-n29, e.g., necessity of special care in terms of ASE and/or blocking

    - For CA_n28-n67, e.g., clarification of frequency restriction is needed.

Proposal 4: For BS to BS co-existence, RAN4 should discuss how practically this issue is resolved and if there is any restriction, e.g., guard band.

# Conclusions

In this paper we provided our views on R20 LB-LB SW.

Observation 1: TWI was approved under the situation that RAN4 TU is limited and no RAN1/2 TU is available.

Observation 2: Any unnecessary enhancements such as shorter switching period, etc., other than the objective in Rel-20 LB-CA produce unnecessary workload.

Proposal 1: RAN4 should focus on competing only the objective in the WID and avoid discussing unnecessary enhancements such as shorter switching period, etc. unless the necessity is agreed.

Proposal 2: RAN4 shouldn’t consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19. It is noted that for CA_n28-n67, only the difference from CA_n12-n29 is captured.

Observation 4: Following observations are obtained on co-existence issues per scenario.

| Scenario | Type of interference | CA_n12-n29 | CA_n28-n67 |
| --- | --- | --- | --- |
| UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation) | Co-channel | UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, the situation can be mitigated thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)) |
|  | Blocking | Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out |
| BS to UE / UE to BS | Given that co-location is assumed, all those wouldn’t have an issue. |  |  |
| BS to BS | Co-channel interference and blocking | Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz. | The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out. |

Observation 5: Some issues captured in Observation 4 are not R20 specific, but rather also apply to R19.

Proposal 3: For UE to UE co-existence (FDD UL Transmission to SDL reception), RAN4 should discuss whether or not special care is needed or not.

    - For CA_n12-n29, e.g., necessity of special care in terms of ASE and/or blocking

    - For CA_n28-n67, e.g., clarification of frequency restriction is needed.

Proposal 4: For BS to BS co-existence, RAN4 should discuss how practically this issue is resolved and if there is any restriction, e.g., guard band.

# Reference

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei)