3GPP TSG-RAN WG4 #116bis	R4-2513549

Prague, Czech Republic, October 13-17, 2025

Agenda item:	6.15.2.2		Adaptation of common signal/channel transmission

Source:	Qualcomm Incorporated

Title:	On performance requirements for SSB adaptation

Document for:	Discussion

# Introduction

In RAN4 #115 the work on RRM performance requirements for Rel-19 NES started. In this paper we provide our view on RRM performance requirements for SSB adaptation as identified in [1] in RAN4 #116.

# Discussion

Issue 3-3-1: Test case list

Agreement

RAN4 to at least define the following SSB adaptation test case.

  - L3 intra-frequency measurement

RAN4 to discuss whether the following SSB adaptation test case list can be defined.

- L1-RSRP

- L1-SINR

- CBD

The way forward in [1] lists the following list of test cases:

In our understanding, deactivated SCell measurements and SCell activation are not supported for SSB adaptation. Hence, no test can be introduced in RAN4 for these scenarios.

Proposal 1: Introduce test cases for SSB adaptation for L1-RSRP, L1-SINR, CBD and L3 measurements.

Issue 3-3-2: General principle

RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed.

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.

The way forward in [1] lists the following general principle how to define test cases for SSB adaptation:

We agree to the general principle how to define test cases for SSB adaptation.

Proposal 2: RAN4 should adopt the general principle in Issue 3-3-2 from RAN4 #116 how to define test cases for SSB adaptation.

The way forward in [2] lists the following SMTC and SSB configurations:

Issue 3-3-3: SMTC and SSB configuration

- Proposals

  - Option 1:

    - TSSB and TSMTC periodicity use large deltas in the periodicity before and after SSB adaptation.

  - Option 2:

    - SSB periodicity: 40 ms to 20 ms

    - SMTC periodicity: 40ms

  - Option 3:

    - SSB periodicity from 80ms to 20ms

    - SMTC periodicity from 80ms to 20ms

Since for L1-RSRP and L1-SINR reporting, the requirement when the measurement report is sent is an upper bound, differences may only be visible for large deltas in the periodicity before and after SSB adaptation. Hence, the delta of SSB and SMTC periodicity before and after adaptation should be large. Option 3 fulfills this pre-requisite in our view.

Proposal 3: RAN4 should adopt Option 3 in Issue 3-3-3 from RAN4 #115: SSB and SMTC periodicity changes from 80 ms to 20 ms and 20 ms to 80 ms.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #116. Our observations and proposals are summarized as follows:

Proposal 1: Introduce test cases for SSB adaptation for L1-RSRP, L1-SINR, CBD and L3 measurements.

Proposal 2: RAN4 should adopt the general principle in Issue 3-3-2 from RAN4 #116 how to define test cases for SSB adaptation.

Proposal 3: RAN4 should adopt Option 3 in Issue 3-3-3 from RAN4 #116: SSB and SMTC periodicity changes from 80 ms to 20 ms and 20 ms to 80 ms.

# References

- R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1, RAN4 #116, August 2025

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, RAN4 #115, May 2025